| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E156 WG11 Number: m19673 |
| --- | --- |

| Title: | Simplification of transform coefficient coding in LCEC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yukinobu Yasugi Tomoyuki Yamamoto 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | Tel: +81-43-299-8526 yasugi.yukinobu@sharp.co.jp, yamamoto.tomoyuki@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

In this proposal, a method is presented to simplify the process of transform coefficient coding in LCEC (Low Complexity Entropy Coding). The run-mode loop of the coefficient coding process for a block is bypassed depending on the level of the last non-zero coefficient and type of the block. Experimental results show that parts of codes and tables for the run-mode can be removed with negligible impact on coding efficiency and complexity. The proposal has been cross-checked.

# Introduction

According to HM 2.0 software [1] there are two types of coding modes for transform coefficients in LELC: the run-mode and the level-mode. The encoder/decoder processes the last non-zero coefficient first and goes on the run-mode loop and subsequently the level-mode loop.

The run-mode is used for coding/decoding pairs of a length of zero-run and a level of nzCoeff (a “run-level” pair). After the run-mode, the level-mode is used for coding/decoding coefficients one-by-one. These steps work properly in most situations.

# Technical description

Generally, when a non-zero coefficient (hereinafter called “nzCoeff”) has a level greater than one, the next nzCoeff is more likely to have a level greater than one, too. Because the level-mode coding is simpler and enough efficient for higher levels, a greater-than-one level is used as a trigger for exiting from the run-mode loop. In particular, intra-coded blocks have relatively high level coefficients compared with inter-coded blocks, thus having higher chance that the levels of both the last two nzCoeffs are greater than one.

However, the HM-2.0 software executes the run-mode loop at least once whether or not the last two levels are high. Therefore it sometimes executes the run-mode loop unnecessarily. In addition, the run-mode coding involves some derivation process for mapping between code numbers and run-level pairs for intra-coded luma blocks, which was proposed in JCTVC-D374 [2].

## Run-mode bypassing

This proposal is to bypass the loop depending on the last nzCoeff and block types. Figure 1 shows a brief coding process with Run-mode bypassing.

Figure 1: Coding process of coefficients with Run-mode bypassing

This simplified run-mode bypassing is used only when the level of the last nzCoeff is greater than one and the current block is intra-coded.

## Reduction of codes and table

The proposed method can makes the run mode codes simpler.

Figure 2 shows the simplification of the mapping function xRunLevelInd(). We can simplify the function because a condition using lrg1Pos>0 will be false only when the last nzCoeff has the level greater than one. Additionally, we can also remove two tables regarding positive lrg1Pos value, which is used in the mapping function. The similar simplification applies to the inverse mapping function xRunLevelIndInv().

__inline UInt xRunLevelInd(Int lev, Int run, Int maxrun, UInt lrg1Pos)

{

UInt cn;

if ( lrg1Pos > 0 )

{

if ( lev == 0 )

{

if ( run < lrg1Pos )

cn = run;

else

cn = (run << 1) - lrg1Pos + 1;

}

else

{

if ( run > (maxrun - (Int)lrg1Pos + 1) )

cn = maxrun + run + 2 ;

else

cn = lrg1Pos + (run << 1);

}

}

else

{

cn = (run << 1);

if ( lev == 0 && run <= maxrun )

{

cn++;

}

}

return(cn);

}

__inline UInt xRunLevelInd(Int lev, Int run, Int maxrun, UInt lrg1Pos)

{

UInt cn;

if ( lev == 0 )

{

if ( run < lrg1Pos )

cn = run;

else

cn = (run << 1) - lrg1Pos + 1;

}

else

{

if ( run > (maxrun - (Int)lrg1Pos + 1) )

cn = maxrun + run + 2 ;

else

cn = lrg1Pos + (run << 1);

}

return(cn);

}

(a) Original					(b) Simplified

Figure 2: Simplification of mapping function

# Experiments

The experimental results are summarized in Table 2. Detailed results are included in the attachment (JCTVC-E156.xls). It is confirmed that the results match those generated by Sony [3].

Table 2: Experimental results

|  | Intra LoCo |  |  | Random access LoCo |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 |  |  |  |
| Class B | 0.0 | 0.0 | 0.1 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.2 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | -0.2 | 0.1 | -0.1 | -0.1 | -0.2 |
| Class D | -0.1 | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 | 0.1 | 0.1 | 0.1 |
| Class E | 0.0 | 0.0 | 0.1 |  |  |  | 0.1 | -0.6 | 0.3 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | 0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  | 101% |  |  |

The evaluation was carried out with LELC settings. The computing platforms are summarized in Table 1. In the experiments, encoding and decoding were done using the grid system composed of variety of PCs, but it was configured so that the same-performance PC is used to encode/decode both anchor and the proposed method for each test point.

Table 1: Computing platforms

| OS | Linux (64bit) |
| --- | --- |
| CPU | Intel Core2 Quad 2.66GHz Intel Core2 Quad Q9550 2.83GHz Intel Xeon X5667 3.07GHz |
| Compiler | gcc 4.4.3 |

# Conclusion

In this contribution, a simplification method of coding transform coefficients for LCEC was proposed. The experimental results show that the method can simplify the coding process and the impact on average performance and computation time is negligible. We recommended adapting this simplification of the coefficient coding into the HEVC.

# Reference

- HM 2.0 software:  http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-2.0

- Marta Karczewicz, et al., “CE5: Improved coefficient coding with LCEC”, JCTVC-D374, 4th JCT-VC Meeting, Daegu, January 2011

- Kazushi Sato, “X-Check of E-156: Simplification of transform coefficient coding in LCEC”, JCTVC-E169, 5th JCT-VC Meeting, Geneva, March 2011

# Patent rights declaration(s)

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).