| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E158 |
| --- | --- |

| Title: | CE9 : Cross-verification of experiment M11, M12, M19, M20, M21, M29, M30 and M31 (JCTVC-E048) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Joonyoung Park Seungwook Park Byeongmoon Jeon #221 Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea | Tel: Email: | 82-10-6272-9179 jy.park@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This document reports the verification results of JCTVC-E048 [1]. Among the experiments included in JCTVC-E048, experiment M11, M12, M19, M20, M21, M29, M30 and M31 are crosschecked in this document.

# Observation

MediaTek provided their source codes. We have investigated them and make sure that there is no mismatch between the source code and algorithm descriptions from the document.

# Cross-verification results

The simulation succeeded and produced as same BD BR rate as proponent provided. The encoding and decoding times also show very similar aspect. The spreadsheets which contain the detailed data of the simulation are accompanied.

table 2.1. Result of experiment M11

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | -0.1 | -0.3 | -0.2 | -0.2 | -0.2 |
| Class B | -0.1 | 0.0 | -0.1 | -0.1 | 0.0 | 0.0 |
| Class C | -0.1 | -0.1 | -0.2 | -0.1 | 0.0 | -0.1 |
| Class D | -0.2 | -0.1 | -0.1 | -0.2 | -0.2 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | -0.1 | -0.1 | -0.2 | -0.1 | -0.1 | -0.1 |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.2 | -0.1 | -0.2 | -0.6 | -0.2 | 0.0 |
| Class C | -0.2 | 0.1 | 0.0 | -0.4 | -0.1 | -0.2 |
| Class D | -0.2 | 0.1 | -0.3 | -0.2 | 0.3 | 0.4 |
| Class E | -0.3 | -0.6 | -0.3 | -0.3 | -0.3 | -0.1 |
| All | -0.2 | -0.1 | -0.2 | -0.4 | -0.1 | 0.0 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

table 2.2. Result of experiment M12

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class B | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 |
| Class D | 0.0 | 0.1 | 0.0 | 0.0 | -0.1 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.3 | -0.2 | -0.2 | -0.9 | -0.4 | 0.0 |
| Class C | -0.3 | -0.1 | -0.1 | -0.5 | -0.1 | -0.3 |
| Class D | -0.4 | -0.1 | -0.6 | -0.4 | 0.5 | 0.7 |
| Class E | -0.5 | -1.0 | -0.8 | -0.6 | -0.6 | -0.3 |
| All | -0.3 | -0.3 | -0.4 | -0.6 | -0.2 | 0.1 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

table 2.3. Result of experiment M19

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.6 | -1.8 | -1.8 | -2.2 | -2.4 | -2.4 |
| Class B | -0.9 | -0.8 | -0.8 | -1.7 | -1.4 | -1.3 |
| Class C | -1.5 | -1.5 | -1.6 | -1.8 | -1.7 | -1.8 |
| Class D | -1.6 | -1.5 | -1.6 | -1.9 | -1.7 | -1.6 |
| Class E |  |  |  |  |  |  |
| All | -1.3 | -1.4 | -1.4 | -1.9 | -1.8 | -1.8 |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.9 | -0.6 | -0.7 | -2.0 | -1.3 | -0.6 |
| Class C | -1.3 | -1.1 | -1.3 | -2.0 | -1.5 | -1.7 |
| Class D | -1.2 | -1.1 | -0.7 | -1.6 | -0.7 | -0.4 |
| Class E | -0.9 | -0.7 | -1.0 | -1.7 | -0.7 | -1.2 |
| All | -1.1 | -0.9 | -0.9 | -1.8 | -1.1 | -0.9 |
| Enc Time[%] | 105% |  |  | 106% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |

table 2.4. Result of experiment M20

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.9 | -0.8 | -1.0 | -1.1 | -1.0 | -1.0 |
| Class B | -0.8 | -0.5 | -0.6 | -0.8 | -0.6 | -0.6 |
| Class C | -0.9 | -0.8 | -0.9 | -0.9 | -0.7 | -0.8 |
| Class D | -1.0 | -0.9 | -0.9 | -1.0 | -0.9 | -0.9 |
| Class E |  |  |  |  |  |  |
| All | -0.9 | -0.8 | -0.8 | -0.9 | -0.8 | -0.8 |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.1 | -0.7 | -0.9 | -1.7 | -1.1 | -0.6 |
| Class C | -1.2 | -1.0 | -1.2 | -1.5 | -1.1 | -1.0 |
| Class D | -1.1 | -1.2 | -0.7 | -1.2 | -0.2 | -0.1 |
| Class E | -1.5 | -1.3 | -0.9 | -1.9 | -1.0 | -1.4 |
| All | -1.2 | -1.0 | -0.9 | -1.6 | -0.8 | -0.7 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

table 2.5. Result of experiment M21

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.5 | -1.7 | -2.0 | -2.1 | -2.4 | -2.3 |
| Class B | -0.8 | -0.8 | -0.8 | -1.6 | -1.2 | -1.2 |
| Class C | -1.4 | -1.4 | -1.5 | -1.7 | -1.5 | -1.7 |
| Class D | -1.5 | -1.6 | -1.6 | -1.8 | -1.6 | -1.6 |
| Class E |  |  |  |  |  |  |
| All | -1.3 | -1.3 | -1.4 | -1.8 | -1.7 | -1.7 |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.1 | -0.8 | -1.1 | -2.6 | -1.4 | -0.9 |
| Class C | -1.4 | -1.3 | -1.4 | -2.2 | -1.6 | -1.6 |
| Class D | -1.4 | -0.9 | -1.4 | -1.9 | -0.4 | -0.3 |
| Class E | -1.0 | -0.8 | -0.9 | -1.8 | -1.0 | -1.2 |
| All | -1.2 | -1.0 | -1.2 | -2.2 | -1.1 | -1.0 |
| Enc Time[%] | 104% |  |  | 105% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

table 2.6. Result of experiment M29

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.5 | -2.0 | -1.9 | -2.3 | -2.5 | -2.3 |
| Class B | -0.9 | -0.8 | -0.9 | -1.5 | -1.1 | -1.1 |
| Class C | -1.4 | -1.5 | -1.4 | -1.7 | -1.6 | -1.7 |
| Class D | -1.4 | -1.5 | -1.5 | -1.7 | -1.5 | -1.5 |
| Class E |  |  |  |  |  |  |
| All | -1.3 | -1.4 | -1.4 | -1.8 | -1.6 | -1.6 |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.2 | -1.1 | -1.1 | -2.7 | -1.9 | -1.4 |
| Class C | -1.6 | -1.5 | -1.5 | -2.6 | -2.1 | -2.1 |
| Class D | -1.8 | -1.6 | -2.0 | -2.3 | -0.9 | -0.9 |
| Class E | -1.3 | -1.1 | -0.6 | -1.9 | -1.4 | -1.3 |
| All | -1.5 | -1.3 | -1.3 | -2.5 | -1.6 | -1.5 |
| Enc Time[%] | 105% |  |  | 106% |  |  |
| Dec Time[%] | 100% |  |  | 103% |  |  |

table 2.7. Result of experiment M30

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.5 | -1.8 | -1.8 | -2.1 | -2.4 | -2.3 |
| Class B | -0.9 | -0.7 | -0.7 | -1.4 | -1.1 | -1.1 |
| Class C | -1.4 | -1.5 | -1.5 | -1.7 | -1.6 | -1.7 |
| Class D | -1.5 | -1.4 | -1.4 | -1.7 | -1.5 | -1.5 |
| Class E |  |  |  |  |  |  |
| All | -1.3 | -1.3 | -1.3 | -1.7 | -1.6 | -1.6 |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.0 | -0.9 | -0.6 | -2.4 | -1.2 | -0.7 |
| Class C | -1.4 | -1.3 | -1.3 | -2.2 | -1.5 | -1.7 |
| Class D | -1.3 | -1.1 | -1.2 | -1.7 | -0.4 | -0.3 |
| Class E | -0.9 | -0.9 | -1.3 | -1.6 | -0.3 | -0.8 |
| All | -1.2 | -1.1 | -1.1 | -2.0 | -0.9 | -0.9 |
| Enc Time[%] | 104% |  |  | 105% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

table 2.8. Result of experiment M31

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.5 | -2.0 | -1.9 | -2.3 | -2.5 | -2.3 |
| Class B | -0.9 | -0.8 | -0.9 | -1.5 | -1.1 | -1.1 |
| Class C | -1.4 | -1.5 | -1.4 | -1.7 | -1.6 | -1.7 |
| Class D | -1.4 | -1.5 | -1.5 | -1.7 | -1.5 | -1.5 |
| Class E |  |  |  |  |  |  |
| All | -1.3 | -1.4 | -1.4 | -1.8 | -1.6 | -1.6 |
| Enc Time[%] | 104% |  |  | 104% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.2 | -1.1 | -1.1 | -2.7 | -1.9 | -1.4 |
| Class C | -1.6 | -1.5 | -1.5 | -2.6 | -2.1 | -2.1 |
| Class D | -1.8 | -1.6 | -2.0 | -2.3 | -0.9 | -0.9 |
| Class E | -1.3 | -1.1 | -0.6 | -1.9 | -1.4 | -1.3 |
| All | -1.5 | -1.3 | -1.3 | -2.5 | -1.6 | -1.4 |
| Enc Time[%] | 104% |  |  | 105% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  |