| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E160 |
| --- | --- |

| Title: | Modified Joint Coding on Prediction Mode Signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jaehyun Lim Byeongmoon Jeon 221, Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea | Tel: Email: | jaehyun.lim@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This document presents a modified joint coding scheme for signaling of prediction modes in inter slices. In HM 2.0, prediction mode and partitioning type for inter slices are jointly coded. And after coding of each CU, adaptive mapping between symbol and codeword index is exploited. In this contribution, the range of joint coding is limited to prediction mode and partitioning type is signaled by another indicator. Additionally, initial mapping tables between symbol and codeword index are rearranged according to the portion of each prediction mode. The simulation result reportedly shows that the proposed method achieves 0.1%, 0.1% bit rate reduction for RALC and LDLC configurations respectively without any complexity increase.

# Introduction

At January JCT-VC meeting, the HM 2.0 was released. In LCEC, prediction mode and partitioning type for inter slices are signaled by the syntax element Inter_Partition which indicates the jointly coded information on prediction mode and partitioning type as shown in Table.1. After the decision of symbol 6 in minimum CU case, additional indicator is signaled for determining prediction mode and partitioning type.

Initially, the symbol which has the lower index is represented by the shorter codeword. In addition, adaptive mapping of codeword table which is dependent on each depth is used. Therefore, the most common symbol uses the shortest codes.

Table.1 Symbols for representing prediction mode and partitioning type in HM 2.0

| prediction mode | partitioning type | symbol |  |
| --- | --- | --- | --- |
|  |  | CU size > min | CU size = min |
| SPLIT |  | 0 | - |
| SKIP |  | 1 | 1 |
| CU MRG |  | 2 | 2 |
| INTER | 2Nx2N | 3 | 3 |
|  | 2NxN | 4 | 4 |
|  | Nx2N | 5 | 5 |
|  | NxN | - | 6 |
| INTRA | 2Nx2N | 6 |  |
|  | NxN | - |  |

In the adaptation method in HM 2.0, shorter codeword is assigned for the commonly selected prediction mode as shown in Figure.1. But, in this process, longer codeword is assigned for signaling some case inevitably. The effect of the mapping table adaptation is occurred in the trade off of positive case and negative case.

Figure.1 The effect of mapping table adaptation

The purpose of the proposed method is to reduce the negative case. Therefore, a range of the joint coding is limited to prediction mode to prevent the commonly selected prediction modes (e.g. split, skip, mrg) have the higher index (and longer codeword) in mapping table.

# Proposed Methods

In the proposed method, the range of joint coding is limited to prediction mode as shown in Table.2. After the decision of INTER or INTRA mode, additional indicator is signaled for determining partitioning type.

Table.2 Symbols for representing prediction mode and partitioning type in the proposed method

| prediction mode | partitioning type | symbol |  |
| --- | --- | --- | --- |
|  |  | CU size > min | CU size = min |
| SPLIT |  | 0 | - |
| SKIP |  | 1 | 0 |
| CU MRG |  | 2 | 1 |
| INTER | 2Nx2N | 3 | 2 |
|  | 2NxN |  |  |
|  | Nx2N |  |  |
|  | NxN | - | 4 |
| INTRA | 2Nx2N | 4 | 3 |
|  | NxN | - |  |

In addition to this modification, initial mapping table for adaptation is reordered reflecting the distribution of each prediction modes. This rearrangement has applied independently by depth.

# Experimental Results

The proposed method was implemented on the HM 2.0 software and the performance was measured under both of low complexity configuration (Random Access and Low Delay). Common test condition is used for measuring the performance of the proposed method.

Table.3 shows the experimental results in terms of BD rate reduction. The average gains for RALC and LDLC configurations are 0.1%, 0.1% respectively. There are no noticeable encoder and decoder complexity increases.

Table.3 BD rate (%) reduction of the proposed method

|  | Random access LoCo |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.1 | -0.1 |  |  |  |
| Class B | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
| Class C | -0.1 | -0.1 | -0.1 | -0.2 | -0.2 | -0.6 |
| Class D | -0.2 | -0.2 | -0.2 | -0.2 | -0.5 | -0.3 |
| Class E |  |  |  | 0.2 | 0.0 | -0.2 |
| All | -0.1 | -0.1 | -0.1 | -0.1 | -0.2 | -0.3 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusion

In this document, modified joint coding of prediction mode signaling for inter slice was proposed. The proposed scheme shows bit-rate reduction without causing any encoder or decoder complexity change.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).