| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E163 WG11 Number: m 19681 |
| --- | --- |

| Title: | Cross verification of Microsoft proposal JCTVC-E147 on motion information compression |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA | Tel: Email: | +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

The document reports cross-check results of Microsoft proposal JCTVC-E147 on motion information compression. It has been verified that the results provided by Microsoft for the combinations of following three tools are accurate: 1) set the ref_idx of 16x16 region to the ref_idx of the chosen 4x4 block  (Ref_idxCompression) 2) instead of picking the motion vector upper_left corner 4x4 block as the motion vector for entire 16x16 region, use the motion vector of bottom-right corner 4x4 block as motion vector for 16x16 region after compression (MVcompressionBR) , and 3) disable temporal MVP in merge mode (noTMVPinMerge). Compared to HM2.0 anchor, a total gain of 0.5% (HE_RA), 0.7% (LC-RA), 0.7% (HE_LD) and 1.1% (LC-LD) has been verified. The source code is also checked to verify the accuracy of proposal description from Microsoft.

# Introduction

The document reports cross-check results of Microsoft proposal JCTVC-E147 on motion information compression [2].

# Test Settings and Conditions

JCTVC-E147 basically proposes the following three tools:

- set the ref_idx of 16x16 region to the ref_idx of the chosen 4x4 block  (Ref_idxCompression)

- instead of picking the motion vector upper-left corner 4x4 block as the motion vector for entire 16x16 region, use the motion vector of bottom-right corner 4x4 block as motion vector for 16x16 region after compression (MVcompressionBR)

- disable temporal MVP in merge mode (noTMVPinMerge)

The following three combinations are tested:

| Settings | Ref_idxCompression | MVcompressionBR | noTMVPinMerge |
| --- | --- | --- | --- |
| Compress10 | x |  |  |
| Compress11 | x | x |  |
| noMergeTMVP | x | x | x |

The source code is checked to verify that the above settings match the source code.

The simulations of this document have used HM2.0 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [1] are followed.

# Experimental Results

A summary of test results is provided in Table 1.

| Settings | Random access (%) | Random access LoCo (%) | Low delay (%) | Low delay LoCo (%) |
| --- | --- | --- | --- | --- |
| Compress10 | 0.0 | 0.0 | -0.1 | -0.1 |
| Compress11 | -0.5 | -0.6 | -0.7 | -0.7 |
| noMergeTMVP | -0.5 | -0.7 | -0.7 | -1.1 |

Table 1: Summary of testing results (Y BD-rate vs. HM2.0 anchor)

The detailed results of three settings are provided in Table 2 to Table 4. It has been verified the results presented in this section perfectly match the results provided by Microsoft.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.2 | -0.1 | 0.1 | -0.1 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 |
| Class D | -0.1 | -0.1 | -0.1 | -0.1 | -0.1 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.1 | 0.0 | 0.0 | -0.1 | 0.0 |
| Enc Time[%] | 102% |  |  | 96% |  |  |
| Dec Time[%] | 104% |  |  | 90% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | -0.1 | -0.1 | 0.0 | 0.1 |
| Class C | -0.1 | 0.2 | -0.1 | -0.1 | 0.0 | -0.1 |
| Class D | -0.2 | 0.2 | -0.4 | -0.1 | -0.3 | -0.3 |
| Class E | -0.1 | 0.4 | -0.1 | -0.1 | 0.1 | 0.1 |
| All | -0.1 | 0.2 | -0.2 | -0.1 | -0.1 | -0.1 |
| Enc Time[%] | 101% |  |  | 97% |  |  |
| Dec Time[%] | 104% |  |  | 94% |  |  |

Table 2: Coding gain by ref_idx compression (Compress10)

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.5 | -0.4 | -0.5 | -0.5 | -0.6 | -0.4 |
| Class B | -0.3 | -0.2 | -0.2 | -0.5 | -0.4 | -0.4 |
| Class C | -0.5 | -0.5 | -0.6 | -0.6 | -0.5 | -0.5 |
| Class D | -0.9 | -0.8 | -0.9 | -0.9 | -0.8 | -0.8 |
| Class E |  |  |  |  |  |  |
| All | -0.5 | -0.5 | -0.5 | -0.6 | -0.6 | -0.5 |
| Enc Time[%] | 99% |  |  | 95% |  |  |
| Dec Time[%] | 100% |  |  | 91% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.4 | -0.3 | -0.5 | -0.7 | -0.7 | -0.2 |
| Class C | -0.6 | -0.4 | -0.5 | -0.8 | -0.8 | -1.0 |
| Class D | -0.8 | -0.7 | -0.9 | -0.8 | -0.8 | -0.5 |
| Class E | -0.8 | -1.2 | 0.1 | -0.5 | -0.8 | -0.6 |
| All | -0.7 | -0.6 | -0.5 | -0.7 | -0.8 | -0.5 |
| Enc Time[%] | 100% |  |  | 97% |  |  |
| Dec Time[%] | 103% |  |  | 94% |  |  |

Table 3: Coding gain by ref_idx compression plus picking motion vector of bottom right corner 4x4 block as compressed vector (Compress11)

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.4 | -0.4 | -0.5 | -0.6 | -0.6 | -0.5 |
| Class B | -0.3 | -0.2 | -0.3 | -0.7 | -0.6 | -0.5 |
| Class C | -0.6 | -0.6 | -0.6 | -0.7 | -0.6 | -0.6 |
| Class D | -0.9 | -0.9 | -0.9 | -1.0 | -0.9 | -0.8 |
| Class E |  |  |  |  |  |  |
| All | -0.5 | -0.5 | -0.5 | -0.7 | -0.6 | -0.6 |
| Enc Time[%] | 98% |  |  | 97% |  |  |
| Dec Time[%] | 104% |  |  | 97% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.4 | -0.3 | -0.6 | -1.1 | -1.0 | -0.8 |
| Class C | -0.7 | -0.7 | -0.7 | -1.1 | -0.9 | -1.1 |
| Class D | -1.0 | -1.0 | -1.0 | -1.2 | -1.3 | -1.4 |
| Class E | -0.9 | -1.4 | -0.5 | -0.9 | -1.3 | -0.6 |
| All | -0.7 | -0.8 | -0.7 | -1.1 | -1.1 | -1.0 |
| Enc Time[%] | 97% |  |  | 96% |  |  |
| Dec Time[%] | 104% |  |  | 99% |  |  |

Table 4: Coding gain by additionally disabling TMVP in merge mode (noMergeTMVP)

# Conclusions

It has been cross-verified that the results provided by Microsoft in [2] are accurate. It is recommended to adopt ref_idx compression as bug fix for motion information compression, and  the removal of TMVP from the merge mode. Further study on the proposed motion vector compression algorithm (i.e. picking motion vector of bottom right corner 4x4 block as compressed motion vector) should be conducted together with other proposals in the same category in core experiment. It is recommended to carry out the further study in conjunction with the MVP list construction algorithms for merge mode, skip mode and motion vector coding.

# References

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-C600, 4th JCT-VC Meeting, Daegu, KR, 20-28 January, 2011

- JCT-VC, “Motion information compression” JCTVC-E147, 5th JCT-VC Meeting, 5th Meeting: Geneva, CH, 16-23 March, 2011