| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E165_r1 |
| --- | --- |

| Title: | CE6.e: Cross verification of SCU’s proposal (JCTVC-E110) on Planar Intra Prediction (1.d) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yongjoon Jeon Byeongmoon Jeon #221 Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea | Tel: Email: | yongjoon.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

The purpose of this document is to report the cross-check results of JCTVC-E110 on Planar Intra Prediction by Santa Clara University. The verification task has been done successfully and the results exactly match the results provided by the proponent.

- Verification

The proponent provided the software for CE.6.e.1.d. The code was compiled using Microsoft® Visual Studio® 2008. The simulation follows the common test condition of [1]. Encoding is done on the PCs with the following configurations: Microsoft® Windows® XP 64bits, Intel® Xeon® CPU E5420 @2.50GHz, 16GB of RAM. Decoding is done on the PCs with the following configurations: Microsoft® Windows® XP 64bits, Intel® Xeon® CPU E5355 @2.66GHz, 16GB of RAM. In the simulation, -o encoder option (YUV writing) was used to confirm that a reconstructed sequence matches its decoded sequence, and -o decoder option (YUV writing) was not used.

- Results

The simulation results are shown Table 1. All of them match exactly provided by proponents.

Table 1: Summary of BD-rate and Encoding/Decoding Time

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | -0.1 | 0.0 | -0.1 | -0.1 |
| Class B | 0.0 | -0.1 | -0.2 | 0.0 | -0.2 | -0.2 |
| Class C | 0.0 | -0.1 | 0.0 | 0.0 | -0.2 | -0.2 |
| Class D | 0.1 | -0.1 | -0.1 | 0.0 | -0.1 | 0.0 |
| Class E | -0.1 | 0.1 | 0.1 | -0.1 | -0.3 | -0.5 |
| All | 0.0 | 0.0 | -0.1 | 0.0 | -0.2 | -0.2 |
| Enc Time[%] | 105% |  |  | 106% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

- References

| [1] | F. Bossen, " Common test conditions and software reference configurations" Doc. JCTVC-D600, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Daegu, KR, January 2011 |
| --- | --- |