| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E166_r1 |
| --- | --- |

| Title: | CE6.e: Report on Planar Intra Prediction by LG |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjoon Jeon Seungwook Park Joonyoung Park Byeongmoon Jeon #221 Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea | Tel: Email: | yongjoon.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This document reports results for the planar prediction schemes which are tested within the CE 6.e. This CE measures the performances of the tools for planar mode which include “bilinear interpolation as introduced in JCTVC-D326”, “averaging two linear interpolations introduced in JCTVC-D235”, “refinement process introduced in JCTVC-D083”, and “a simplified version of the refinement process”. Those tools are tested together with the DST and the signaling method introduced in JCTVC-D235. This document presents the results for the combination of the tools and figures out the best combination of the tools in terms of coding gain. The best combination reportedly results in 1.20% and 1.43% BD BR reduction on average for Intra high efficiency and Intra low complexity configuration respectively.

# Planar prediction

At the JCT-VC meeting in Daegu, in January 2011, Core Experiment 6.e [1] was established to investigate the performance of different configurations and combinations of planar intra prediction methods. This contribution focuses on finding the best combination of them.

Schemes are categorized into prediction method, prediction error coding, and indication method for planar mode. Each category has different approaches.

Prediction

- 1.a : JCTVC-D326 method

- 1.b : JCTVC-D083 method (JCTVC-D326 method + refinement process of JCTVC-D083)

- 1.c : JCTVC-D235 method

- 1.e : multiplication-free implementation of 1.a

- 1.f : JCTVC-D235 method + simplified refinement process of JCTVC-D083

Prediction error coding

- DCT

- DST introduced in JCTVC-D235

Indication method

- Replace DC mode

- Additional mode as defined in JCTVC-D235

Brief description of the four prediction methods are presented below.

1.a (JCTVC-D326 method)

Fig. 1 shows JCTVC-D326 [2] method. In this method, bottom-right sample is reconstructed and border samples are linearly interpolated using the bottom-right sample. Then inner samples are obtained by bilinear interpolation.

| (a) | (b) |
| --- | --- |

Figure1. Prediction process of JCTVC-D326

1.b (JCTVC-D083 method : JCTVC-D326 method + refinement process of JCTVC-D083)

Fig.2 shows the refinement process of JCTVC-D083 [3]. This process is applied after prediction process of 1.a. Refinement process is a recursive filtering of samples using [1 2 1] filter with above and left sample as shown in Fig.2.

Figure2. Refinement process of JCTVC-D083

1.c (JCTVC-D235)

Fig. 3 shows JCTVC-D235 [4] method which replaces bilinear interpolation with the average of two linear interpolations. This implementation removes multiplications and divisions.

Figure3. Prediction process of JCTVC-D235

1.e

1.e is a multiplication-free implementation of 1.a. It is based on average of two linear interpolations similar to 1.c

1.f (CE6.e.1.c prediction + simplified refinement process of CE6.e.1.b)

Fig.4 shows the simplified refinement process introduced in 1.b. The original scheme is simplified by restricting the number of samples to which the refinement is applied. It is designed to be harmonized with 1.c prediction design. In this process, top-left (T) and left-bottom (L) samples are filtered using [1 2 1] filter prior to 1.c prediction.

Figure4. Simplified refinement process of CE6.e.1.f

Next section presents simulation results for the schemes introduced above.

# Simulation results

Each prediction method is tested with the two prediction error coding method (DCT/DST) and the two different indication methods (Substituting DC mode/Additional mode as defined in JCTVC-D235). The results for 1.c and 1.e are copied from [5] and [6] to compare those schemes with 1.b and 1.f. In this contribution, the results for 1.a and 1.d are not presented because the results of 1.a are identical to those of 1.e and to focus on most promising tools. In the same context, 1.b is not tested with DST and the indication method of JCTVC-D235.

Simulations are configured with the combination of the following schemes.

1. Prediction method

1.b, 1.c, 1.e, and 1.f

2. Prediction error coding

2.a – DCT

2.b – DST

3. Indication method for planar mode

3.a – Substituting DC mode

3.b – Indicating as an additional mode

For the simulation, the test condition of [7] is used and HM2.0 is used for the anchor. Encoding is done on the PCs with the following configurations: Microsoft® Windows® XP 64bits, Intel® Core™ i7 CPU 950 @3.07GHz, 12GB of RAM. Decoding is done on the PCs with the following configurations: Microsoft® Windows® XP 64bits, Intel® Xeon® CPU E5355 @2.66GHz, 16GB of RAM. In the simulation, -o encoder option (YUV writing) was used to confirm that a reconstructed sequence matches its decoded sequence, and -o decoder option (YUV writing) was not used.

Table 1 to 4 show results for the combination of the tools introduced above.

Substituting DC mode (3.a)

- With DCT (2.a+3.a)

Table 1 “1.b+2.a+3.a”, “1.c+2.a+3.a”, “1.e+2.a+3.a”, and “1.f+2.a+3.a”

| Intra |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.b + 2.a + 3.a |  |  | 1.c + 2.a + 3.a |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.39 | 1.04 | 1.05 | -0.21 | 1.27 | 1.37 |
| Class B | -0.61 | 0.62 | 0.75 | -0.57 | 0.55 | 0.74 |
| Class C | -0.31 | 1.03 | 1.40 | -0.36 | 0.77 | 1.15 |
| Class D | -0.37 | 1.07 | 1.35 | -0.45 | 0.77 | 1.00 |
| Class E | -0.68 | 0.09 | 0.60 | -0.88 | 0.54 | 0.88 |
| All | -0.47 | 0.80 | 1.04 | -0.48 | 0.78 | 1.02 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |
|  | 1.e + 2.a + 3.a |  |  | 1.f(1.c+simplified refinement) + 2.a + 3.a |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.18 | 1.21 | 1.28 | -0.31 | 1.12 | 1.23 |
| Class B | -0.53 | 0.62 | 0.80 | -0.70 | 0.43 | 0.63 |
| Class C | -0.32 | 0.83 | 1.18 | -0.47 | 0.64 | 1.00 |
| Class D | -0.40 | 0.81 | 0.99 | -0.55 | 0.67 | 0.92 |
| Class E | -0.69 | 0.53 | 0.83 | -0.95 | 0.51 | 0.79 |
| All | -0.42 | 0.80 | 1.02 | -0.58 | 0.67 | 0.91 |
| Enc Time[%] | 103% |  |  | 100% |  |  |
| Dec Time[%] | 105% |  |  | 101% |  |  |
| Intra LoCo |  |  |  |  |  |  |
|  | 1.b + 2.a + 3.a |  |  | 1.c + 2.a + 3.a |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.80 | 0.13 | 0.45 | -0.54 | 0.44 | 0.74 |
| Class B | -0.54 | -0.24 | 0.13 | -0.62 | -0.21 | 0.17 |
| Class C | -0.40 | 0.08 | 0.23 | -0.48 | -0.08 | 0.15 |
| Class D | -0.49 | 0.14 | 0.28 | -0.57 | -0.04 | 0.08 |
| Class E | -0.73 | -0.54 | -0.63 | -0.91 | -0.06 | -0.12 |
| All | -0.58 | -0.07 | 0.13 | -0.61 | 0.00 | 0.22 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 104% |  |  | 102% |  |  |
|  | 1.e + 2.a + 3.a |  |  | 1.f(1.c+simplified refinement) + 2.a + 3.a |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.50 | 0.28 | 0.64 | -0.65 | 0.34 | 0.66 |
| Class B | -0.50 | -0.14 | 0.24 | -0.71 | -0.31 | 0.10 |
| Class C | -0.40 | -0.01 | 0.20 | -0.57 | -0.20 | 0.00 |
| Class D | -0.50 | -0.13 | 0.05 | -0.65 | -0.22 | -0.12 |
| Class E | -0.67 | 0.14 | -0.02 | -0.97 | -0.25 | -0.28 |
| All | -0.51 | 0.01 | 0.23 | -0.70 | -0.13 | 0.09 |
| Enc Time[%] | 103% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |

- With DST (2.b+3.a)

Table 2 “1.b+2.b+3.a”, “1.c+2.b+3.a”, “1.e+2.b+3.a”, and “1.f+2.b+3.a”

| Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.c + 2.a + 3.a |  |  | 1.e + 2.a + 3.a |  |  | 1.f(1.c+simplified refinement) + 2.a + 3.a |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.41 | 2.35 | 2.64 | -0.39 | 2.17 | 2.48 | -0.53 | 2.30 | 2.65 |
| Class B | -0.85 | 1.37 | 2.15 | -0.82 | 1.47 | 2.19 | -1.00 | 1.33 | 2.17 |
| Class C | -0.71 | 1.45 | 1.98 | -0.67 | 1.48 | 1.96 | -0.86 | 1.35 | 1.91 |
| Class D | -0.80 | 1.42 | 1.81 | -0.75 | 1.42 | 1.76 | -0.93 | 1.40 | 1.76 |
| Class E | -1.10 | 3.09 | 3.02 | -0.88 | 2.98 | 2.90 | -1.16 | 3.19 | 3.14 |
| All | -0.76 | 1.85 | 2.28 | -0.70 | 1.83 | 2.22 | -0.89 | 1.82 | 2.28 |
| EncT[%] | 101% |  |  | 101% |  |  | 101% |  |  |
| DecT[%] | 103% |  |  | 102% |  |  | 101% |  |  |
| Intra Loco |  |  |  |  |  |  |  |  |  |
|  | 1.c + 2.a + 3.a |  |  | 1.e + 2.a + 3.a |  |  | 1.f(1.c+simplified refinement) + 2.a + 3.a |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.90 | -0.76 | -0.10 | -0.85 | -0.90 | -0.25 | -1.05 | -0.91 | -0.20 |
| Class B | -1.09 | -1.14 | -0.36 | -0.98 | -1.05 | -0.33 | -1.21 | -1.25 | -0.42 |
| Class C | -1.11 | -0.94 | -0.58 | -1.03 | -0.90 | -0.56 | -1.24 | -1.06 | -0.74 |
| Class D | -1.12 | -0.90 | -0.74 | -1.05 | -0.98 | -0.79 | -1.25 | -1.02 | -0.93 |
| Class E | -1.26 | -0.58 | -0.89 | -0.97 | -0.36 | -0.85 | -1.31 | -0.71 | -1.03 |
| All | -1.09 | -0.89 | -0.51 | -0.98 | -0.87 | -0.53 | -1.21 | -1.02 | -0.63 |
| EncT[%] | 103% |  |  | 102% |  |  | 103% |  |  |
| DecT[%] | 105% |  |  | 105% |  |  | 104% |  |  |

Indicating as an additional mode (3.b)

- With DCT (2.a+3.b)

Table 3 “1.b+2.a+3.b”, “1.c+2.a+3.b”, “1.e+2.a+3.a”, and “1.f+2.a+3.b”

| Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.c + 2.a + 3.a |  |  | 1.e + 2.a + 3.a |  |  | 1.f(1.c+simplified refinement) + 2.a + 3.a |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.66 | 0.15 | 0.30 | -0.56 | 0.21 | 0.26 | -0.72 | 0.14 | 0.26 |
| Class B | -0.81 | -0.28 | -0.30 | -0.70 | -0.13 | -0.17 | -0.90 | -0.30 | -0.29 |
| Class C | -0.53 | -0.10 | 0.04 | -0.43 | 0.01 | 0.17 | -0.60 | -0.12 | 0.03 |
| Class D | -0.55 | -0.02 | 0.07 | -0.45 | 0.06 | 0.17 | -0.63 | -0.04 | 0.04 |
| Class E | -0.91 | -0.15 | 0.22 | -0.71 | -0.12 | 0.22 | -0.97 | -0.12 | 0.27 |
| All | -0.69 | -0.09 | 0.04 | -0.57 | 0.00 | 0.11 | -0.76 | -0.10 | 0.03 |
| EncT[%] | 104% |  |  | 101% |  |  | 104% |  |  |
| DecT[%] | 101% |  |  | 100% |  |  | 100% |  |  |
| Intra Loco |  |  |  |  |  |  |  |  |  |
|  | 1.c + 2.a + 3.a |  |  | 1.e + 2.a + 3.a |  |  | 1.f(1.c+simplified refinement) + 2.a + 3.a |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.88 | -0.61 | -0.44 | -0.76 | -0.60 | -0.42 | -0.94 | -0.63 | -0.47 |
| Class B | -0.77 | -0.96 | -0.88 | -0.60 | -0.83 | -0.74 | -0.81 | -0.99 | -0.91 |
| Class C | -0.55 | -0.79 | -0.82 | -0.42 | -0.64 | -0.71 | -0.59 | -0.83 | -0.88 |
| Class D | -0.58 | -0.80 | -0.79 | -0.46 | -0.67 | -0.72 | -0.62 | -0.83 | -0.86 |
| Class E | -0.85 | -0.77 | -0.82 | -0.60 | -0.56 | -0.65 | -0.87 | -0.87 | -0.90 |
| All | -0.72 | -0.80 | -0.75 | -0.57 | -0.68 | -0.65 | -0.76 | -0.84 | -0.81 |
| EncT[%] | 107% |  |  | 106% |  |  | 108% |  |  |
| DecT[%] | 102% |  |  | 101% |  |  | 102% |  |  |

- With DST (2.b+3.b)

Table 4 “1.b+2.b+3.b”, “1.c+2.b+3.b”, “1.e+2.b+3.a”, and “1.f+2.b+3.b”

| Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.c + 2.a + 3.a |  |  | 1.e + 2.a + 3.a |  |  | 1.f(1.c+simplified refinement) + 2.a + 3.a |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.01 | 0.12 | 0.47 | -0.92 | 0.14 | 0.46 | -1.08 | 0.21 | 0.61 |
| Class B | -1.26 | -0.56 | -0.14 | -1.15 | -0.40 | -0.03 | -1.36 | -0.49 | 0.02 |
| Class C | -1.02 | -0.22 | -0.05 | -0.93 | -0.10 | 0.06 | -1.12 | -0.21 | 0.00 |
| Class D | -1.05 | -0.37 | -0.26 | -0.96 | -0.24 | -0.19 | -1.15 | -0.35 | -0.19 |
| Class E | -1.18 | 1.03 | 1.15 | -0.95 | 1.06 | 1.19 | -1.25 | 1.19 | 1.30 |
| All | -1.11 | -0.08 | 0.17 | -0.99 | 0.02 | 0.24 | -1.20 | -0.01 | 0.28 |
| EncT[%] | 105% |  |  | 105% |  |  | 105% |  |  |
| DecT[%] | 101% |  |  | 101% |  |  | 101% |  |  |
| Intra Loco |  |  |  |  |  |  |  |  |  |
|  | 1.c + 2.a + 3.a |  |  | 1.e + 2.a + 3.a |  |  | 1.f(1.c+simplified refinement) + 2.a + 3.a |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.57 | -2.27 | -1.88 | -1.45 | -2.30 | -1.91 | -1.65 | -2.32 | -1.93 |
| Class B | -1.38 | -2.42 | -2.11 | -1.22 | -2.31 | -2.04 | -1.45 | -2.46 | -2.14 |
| Class C | -1.27 | -2.04 | -2.07 | -1.13 | -1.93 | -1.99 | -1.33 | -2.07 | -2.14 |
| Class D | -1.27 | -2.14 | -2.18 | -1.15 | -2.03 | -2.13 | -1.34 | -2.19 | -2.25 |
| Class E | -1.33 | -1.76 | -1.98 | -1.05 | -1.61 | -1.91 | -1.36 | -1.84 | -2.04 |
| All | -1.37 | -2.16 | -2.05 | -1.21 | -2.07 | -2.00 | -1.43 | -2.21 | -2.11 |
| EncT[%] | 109% |  |  | 110% |  |  | 109% |  |  |
| DecT[%] | 104% |  |  | 104% |  |  | 103% |  |  |

# Conclusion

The followings are conclusions of this contribution derived from the simulation results.

- Table 2 and 4 show that DST gives more coding efficiency than DCT when used for planar prediction.

- Table 3 and 4 show that the JCTVC-D235 indication method gives more coding efficiency than substituting DC mode.

- Table 1, 2, 3, and 4 show that refinement process gives consistent additional gains on top of different planar prediction variants.

In summary, the best combination for planar prediction is the combination of the following tools

- DST of JCTVC-D235

- Simplified refinement process of 1.f

- Indication method of JCTVC-D235

# References

[1] A. Tabatabai, K. Chono, M. Coban, K. Kei, K. Sugimoto, A. Tanizawa “Description of Core Experiment 6 (CE6): Intra Prediction Improvement,” JCT-VC Doc. JCTVC-D606, Daegu, Jan 2011.

[2] J. Lainema, K. Ugur and O. Bici, “Planar intra coding for improved subjective video quality,” JCT-VC Doc. JCTVC-D326, Daegu, Jan 2011.

[3] Y. Jeon, S. Park, B. Jeon, “Non-directional intra prediction for coding efficiency improvement,” JCT-VC Doc. JCTVC-D083, Daegu, Jan 2011.

[4] S. Kanumuri, TK Tan, F. Bossen, “Enhancements to Intra Coding,” JCT-VC Doc. JCTVC-D235, Daegu, Jan 2011.

[5] S. Kanumuri and F. Bossen, “CE6.e/f: Planar mode experiments and results,” JCT-VC Doc. JCTVC-E321, Geneva, Mar 2011.

[6] O. Bici, J. Lainema, and K. Ugur, “CE6.e: Nokia report on planar intra prediction,” JCT-VC Doc. JCTVC-E317, Geneva, Mar 2011.

[7] F. Bossen, " Common test conditions and software reference configurations" Doc. JCTVC-D600, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Daegu, KR, January 2011

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).