| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E169r1 |
| --- | --- |

| Title: | X-Check of E-156: Simplification of transform coefficient coding in LCEC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kazushi Sato, W12F 1-11-1 GCO Osaki, Shinagawa, 141-0032 Tokyo, Japan | Tel: Email: | +81 3 5435 3308 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

This document provides x-check result of Sharp’s proposal on transform coefficient coding in LCEC, JCTVC-E156.

# Simulation Condition

Software implemented on HM2.0 is provided by the proponent of [1], and the experiment has been conducted under Linux environment by the author of this document. Configuration setting is same as the one described in [2]. Only {Intra, Lowdelay, Randomaccess}_LowComplexity conditions have been tested, as the proposal is just related to LCEC and not to CABAC.

# Simulation Result

Summary of results with Intra, Randomaccess, Lowdelay conditions are shown in Table2.1, 2.2 and 2.3 respectively.

Complete results are shown in JCTVC-E169r1.xls.

As can be seen the proposed method does not have an impact on coding efficiency. With proposed method if-branch in LCEC can be reduced and contributes to complexity reduction both with HW and SW implementations.

Compared with the result with E-156, slight increase in decoding time can be observed. This may due to the instability of our simulation environment.

Table 2.1

|  | Intra LoCo |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.1 |
| Class C | 0.0 | 0.0 | 0.0 |
| Class D | -0.1 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.1 |
| All | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 108% |  |  |

Table 2.2

|  | Random access LoCo |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | 0.1 |
| Class B | 0.0 | 0.1 | 0.0 |
| Class C | 0.0 | -0.2 | 0.1 |
| Class D | 0.0 | -0.1 | 0.0 |
| Class E |  |  |  |
| All | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 106% |  |  |

Table 2.3

|  | Low delay LoCo |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.0 | 0.0 | 0.2 |
| Class C | -0.1 | -0.1 | -0.2 |
| Class D | 0.1 | 0.1 | 0.1 |
| Class E | 0.1 | -0.6 | 0.3 |
| All | 0.0 | -0.1 | 0.1 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 103% |  |  |

# Conclusion

Results with the proposal [1] have been verified.

The author recommends this proposal be adopted as it can reduce complexity on coefficient coding of LCEC without loss in coding efficiency.

# References

[1] “Simplification of transform coefficient coding in LCEC”, JCTVC-E156, Mar 2011

[2]” Common test conditions and software reference configurations”, JCTVC-D600, Jan 2011