| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-Exxx |
| --- | --- |

| Title: | CE6.a : Bi-Intra Prediction using Slope information |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. W. Seo, J. K. Han (Sejong University) J.Lim (SKT) 910, Choongmu-Gwan, Sejong University, 98 Gunja-Dong, Gwangjin-Gu, Seoul, 143-747, Korea 9-1 Sunae-Dong, Bundang-gu, Seongnam City, Kyeonggi-Do, 463-784, Korea | Tel: Email: | +82-2-3408-3739 sachonwon@gmail.com hjk@sejong.ac.kr +82-31-710-5196 jeongyeon@sktelecom.com |
| Source: | Sejong University and SK telecom |  |  |

_____________________________

# Abstract

This contribution states bi-intra prediction scheme which is used with offset and reference smoothing.  Since the proposed technique considers the concept of slope and uses bi-prediction like bilinear interpolation, it increases the coding efficiency of HM. In this document, the experimental results under the common test condition defined by JCTVC-D600 are reported. For I slice only coding structure, the average BD-rate gains are 0.7% on low complexity condition and 0.4% on high efficiency condition. At the previous meeting (4th meeting Daegu, Korea), this scheme had been introduced with 2.4% and 0.6% coding gain for intra low complexity and intra high efficiency conditions, respectively. Reduction of coding gain is caused by changes of entropy coding implemented in HM2.0 which is different from that of HM1.0. Since there was not enough time to optimize the signaling method of BIP which is related to the entropy coding, the coding gain of bi-intra prediction scheme decreases. It is required to optimize the signaling scheme for improving coding efficiency further.

# Introduction

In the previous meeting (4th meeting Daegu, Korea), bi-intra prediction using slope information (BIP) had been proposed [1]. It consists of four algorithms, (1) using offset, (2) bi-intra prediction, (3) reference pixel smoothing, and (4) embedded flags. These tools decrease the residual data resulting from intra prediction. Thus, the coding efficiency is increased by them. In this contribution, the scheme using the calculated offset is modified. The details of algorithms are described in the section 2.

# Algorithm description

2.1. Bi-Intra Prediction (BIP)

The proposed BIP uses the concept of slope to increase the performance of Unified Intra Prediction (UIP) [2]. In this contribution, BIP is applied only to 0 ~ 9th modes except for DC mode to decrease the encoding complexity while the BIP had been applied for all modes except for DC mode in JCTVC-D287 [1]. For the considered direction modes, we calculate the difference between two neighboring parts of the current block. One neighboring part is next to the reference line and other is next to the last horizontal (or vertical) line of the current block according to the prediction direction. In case of vertical prediction directions, the difference is calculated between two left neighboring parts of the last horizontal line and the corresponding reference line as in Fig. 1. For horizontal prediction directions, upper-extended parts of the last vertical line and the corresponding vertical reference line are used to calculate the difference. The lines to get the difference value are denoted by red blocks in Fig. 1. The averaged difference between them (red pixels) is offset which provides the information for slope. The slope is not encoded since the decoder can calculate it.

After offsets are calculated, the last line of the current block can be predicted by adding the offset to the precition values provided by UIP, where the calculated offset is added after it has been scaled according to the locations of the predicted pixels since the distances between reference pixels and the predicted pixels on the last line are different according to the position of the predicted pixel. When the distances between the reference pixels and the predicted pixels of the last row (or column line) are same, the offset is added to the last line without modification. If some of the distances are different from other value, the offset scaled according to the distance is used.

And then, the other lines of the current block are predicted by bi-linear interpolation with the neighboring pixels and the prediction values of the last line. Note that the size of the neighbor lines (red block) used to calculate offset is half of that of the current block.

Fig.  1. Example of BIP for a 4x4 block

2.2. Offset Clipping

When the offset is too large, the offset would not proper to predict the current block. So, the offset value is clipped as follows.

Offset = Clip( -20<<B_inc, 20<<B_inc, offset)

2.3. Reference Pixel Smoothing

In bi-intra prediction, the coding efficiency can be degraded when the difference between the predicted last line and their adjacent reference is large. This case is occurred when the difference between B and C is large in Fig. 2. This coding loss can be prevented by smoothing them as follows.

B‘=(A + 2 * B + C)/4

C‘=(B + 2 * C + D)/4

Fig.  2. Reference pixel smoothing

2.4. Flag embedded in quantized coefficeints

Intra coding blocks are encoded by UIP or the proposed BIP. Mode and algorithm are selected based on RD optimization. In this contribution, flag to indicate the selected algorithm is not encoded to reduce the generated bit rate. Instead of encoding the flag, the flag is embedded in the quantized coefficeints. When BIP is used, the sum of absolute values of quantized coefficients are even number, i.e., 0, 2, 4, ..... On the other hand, when UIP is used, the sum of absolute values of quantized coefficients is odd, i.e., 1, 3, 5, .....

# Simulation results

As for anchor, HM2.0 software is used in this simulation. Intra only coding structure is tested. The experiments in this contribution are performed under the common test condition defined by JCTVC-D600 [3].

BD-rate (ΔBitrate [%]) [4] is measured by Excel sheet (HM2_results_template.xls) provided on the JCTVC reflector. The Table 1 shows the experimental results of BIP for I slice only coding structure. This table shows that BIP provides coding gain with the reasonable increase of the encoding and decoding time.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3 | -0.5 | -0.5 | -0.6 | -0.8 | -0.8 |
| Class B | -0.3 | -0.6 | -0.6 | -0.7 | -1.0 | -0.9 |
| Class C | -0.4 | -0.9 | -0.9 | -0.6 | -1.1 | -1.0 |
| Class D | -0.5 | -1.0 | -0.9 | -0.8 | -1.0 | -1.1 |
| Class E | -0.4 | -1.0 | -0.8 | -0.7 | -1.2 | -1.2 |
| All | -0.4 | -0.8 | -0.7 | -0.7 | -1.0 | -1.0 |
| Enc Time[%] | 139% |  |  | 122% |  |  |
| Dec Time[%] | 101% |  |  | 104% |  |  |

Table 1. Intra only case

# Conclusions

From the simulations, the average BD-rate gains are 0.7% on low complexity condition and 0.4% on high efficiency condition. At the previous meeting (4th meeting Daegu, Korea), this scheme achieved 2.4% and 0.6% coding gain for intra low complexity and intra high efficiency conditions, respectively. Reduction of coding gain is caused by changes of entropy coding implemented in HM2.0 which had been adopted in the 4th Daegu meeting [5][6][7]. Because a coding flag for a PU is embedded in quantized transform coefficients as described in section 2.4, it is required to optimize the signaling method for improving coding efficiency further.

# References

- C.W. Seo, J. K. Han, J. Lim, and J. Song, “Bi-Intra Prediction using Slope information,” JCTVC-D287, 4th JCT-VC Meeting, Daegu, Korea, KR, Jan. 2011.

- JCT-VC, “WD1: Working Draft 2 of High-Efficiency Video Coding,” JCTVC-D503, 4th JCT-VC Meeting, Daegu, Korea, KR, Jan. 2011.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-D600, 4th JCT-VC Meeting, Daegu, Korea, KR, Jan. 2011.

- Gisle Bjøntegaard, “Improvements of the BD-PSNR model”, ITU-T SG16/Q6, 35th VCEG Meeting, Berlin, Germany, July 2008, Doc. VCEG-AI11.

- M. Karczewicz, X. Wang, and W. J. Chen, “CE5: Improved coefficient coding with LCEC,” JCTVC-D374, 4th JCT-VC Meeting, Daegu, Korea, KR, Jan. 2011.

- J. Lou, K. Panusopone, and L. Wang, “CE11: Report on Zigzag scan performance for CABAC on TMuC0.9,” JCTVC-D239, 4th JCT-VC Meeting, Daegu, Korea, KR, Jan. 2011.

- V. Seregin and J. Chen, “LCEC RDOQ speedup,” JCTVC-D416, 4th JCT-VC Meeting, Daegu, Korea, KR, Jan. 2011.

# Patent rights declaration(s)

Sejong University and SK Telecom may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).