| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E171 WG11 Number: m19689 |
| --- | --- |

| Title: | CE12: Verification results of Ericsson’s Proposal JCTVC-E276 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage Saori Asaka | Email: | tomoo.yamakage@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This contribution shows verification results of JCTVC-E276 (CE12.1: Ericsson deblocking filter) for CE12.  Coding efficiency results are exactly matched with the proponent’s results.  No mismatch between encoder and decoder is observed.  Relative decoding time is similar to that of proponent’s.

# Introduction

This contribution is a report of verification results of JCTVC-E276, which is a contribution to CE12.  Verification is performed under the CE12 test conditions (JCTVC-D612_r1) that refer to JCTVC-D600.  Coding efficiency results are exactly matched with the proponent’s results.  Relative decoding time is similar to those of proponent’s.

# Experimental Results

Detailed results in Excel sheet are attached.

The following table is the summary of experimental results:

|  | High Efficiency |  |  |  |  |  |  |  |  | Low Complexity |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IO |  |  | RA |  |  | LD |  |  | IO |  |  | RA |  |  | LD |  |  |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| A | -1.0 | 0.0 | 0.0 | -1.1 | 0.0 | 0.0 |  |  |  | -1.5 | 0.0 | 0.0 | -1.4 | 0.8 | 1.1 |  |  |  |
| B | -0.7 | 0.0 | 0.0 | -0.9 | -0.2 | -0.4 | -1.4 | -1.3 | -1.4 | -0.8 | 0.0 | 0.0 | -1.0 | -0.1 | -0.1 | -1.3 | -0.9 | -0.4 |
| C | -0.6 | 0.0 | 0.0 | -0.8 | -0.2 | -0.2 | -1.0 | -0.5 | -0.6 | -0.8 | 0.0 | 0.0 | -0.8 | 0.3 | 0.0 | -0.8 | 0.3 | -0.2 |
| D | -0.5 | 0.0 | 0.0 | -0.6 | -0.3 | -0.3 | -0.9 | -0.7 | -0.8 | -0.6 | 0.0 | 0.0 | -0.6 | -0.1 | 0.0 | -0.6 | 0.2 | 0.2 |
| E | -0.9 | 0.0 | 0.0 |  |  |  | -3.3 | -3.8 | -3.2 | -1.0 | 0.0 | 0.0 |  |  |  | -2.6 | -4.7 | -3.3 |
| Ave. | -0.7 | 0.0 | 0.0 | -0.9 | -0.2 | -0.2 | -1.5 | -1.4 | -1.4 | -0.9 | 0.0 | 0.0 | -0.9 | 0.2 | 0.2 | -1.2 | -1.0 | -0.7 |
| Enc. | 100% |  |  | 100% |  |  | 100% |  |  | 99% |  |  | 100% |  |  | 100% |  |  |
| Dec. | 100% |  |  | 100% |  |  | 99% |  |  | 101% |  |  | 101% |  |  | 101% |  |  |

There is no mismatch between encoder and decoder.  Relative encoding and decoding time are similar to those of proponent’s.

Source code is reviewed and confirmed.

# Conclusion

This contribution shows verification results of JCTVC-E276. Coding efficiency results are confirmed.  No mismatch between encoder and decoder is observed.  Relative encoding and decoding time are also confirmed.

# Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).