| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E174 |
| --- | --- |

| Title: | Unified planar intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Oguz Bici Jani Lainema Kemal Ugur | Email: | ext-oguz.bici@nokia.com jani.lainema@nokia.com kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This document presents a unified planar intra prediction mode combining aspects from the best performing approaches in Core Experiment 6.e. The sample prediction process is identical to the multiplication free “1.g” approach tested in CE6.e. Signaling of the planar prediction mode is done as proposed by Docomo in JCTVC-D235 and tested in test “3.a” of CE6.e and the refinement process of CE6.e’s “1.h” test is applied to the  to the reference pixels prior to prediction sample generation. It is argued that among all the alternatives tested in CE6.e this proposal brings the highest coding efficiency with complexity equal to the lowest complexity alternative. The BD-rate gains of 0.73% for Intra high efficiency and 0.86% for Intra low complexity are reported for DCT coding and BD-rate gains of 1.17% for Intra high efficiency and 1.51% for Intra low complexity are reported for DST based prediction error coding.

# Introduction

At the JCT-VC meeting in Daegu, in January 2011, Core Experiment 6.e [1] was established to measure performance of different configurations and combinations of the proposed planar intra prediction methods. Following the results of the CE6.e experiments, in this contribution, the features reported to achieve higher BD-rate gains are discussed in further detail. Next section describes each part of the proposed planar mode and the results are reported in Section 3.

# Unified Planar Intra Prediction

The unified planar intra prediction process consists of four parts as described below.

## Simplified Refinement

Prior to prediction process, a refinement process is applied on the reference samples with index N-1 in top and left reference blocks as illustrated in Figure 1. These samples are filtered with [1 2 1] filter using the neighbor samples with indices N-2 and N as in Figure 1. This process is a simplified version of the refinement process proposed in [2] and it was tested under the categories 1.f and 1.h in the CE6.e [1].

Figure 1: Illustration of reference blocks and refinement procedure

## Planar prediction

The gradient of the predicted sample surface is defined by a predicted value for the bottom-right sample of the block. This is obtained by averaging the reference samples at locations [-1, 2N-1] and [2N-1, -1] as illustrated in Figure 2. The predicted samples are then obtained as an average of two linear interpolations. This can be implemented in a multiplication free manner and is similar in nature to the implementation proposed by Docomo and described in [3]. The samples generated in this way are identical to the bilinear interpolation implementation presented in [4], with the difference of simplified prediction for the bottom-right corner sample ([4] by Nokia proposed to use DC prediction for generating that sample).

Figure 2: Prediction of bottom-right sample

The multiplication free reconstruction process can be formulated as follows:

// Delta values for single horizontal and vertical steps for each row and column

iDeltaHorStep    = ( iSampleBottomRight - iSampleBottomLeft ) >> iShift1D;

iDeltaVerStep    = ( iSampleBottomRight - iSampleTopRight   ) >> iShift1D;

iBottomRowSample = iSampleBottomLeft;

iRightColSample  = iSampleTopRight;

for(k=0;k<iWidth;k++){

iBottomRowSample += iDeltaHorStep;

iRightColSample  += iDeltaVerStep;

iDeltaHor[k]      = ( iRightColSample  - iLeftColumn[k] + iOffset1D ) >> iShift1D;

iDeltaVer[k]      = ( iBottomRowSample - iTopRow[k]     + iOffset1D ) >> iShift1D;

}

// Average of two linear interpolations to get the final planar samples

for (k=0;k<iHeight;k++){

Int iLeftRef = iLeftColumn[k];

for (l=0;l<iWidth;l++)

iTopRow[l] += iDeltaVer[l];

for (l=0;l<iWidth;l++){

iLeftRef   += iDeltaHor[k];

pDst[k*iDstStride+l] = ( iLeftRef + iTopRow[l] + iOffset2D ) >> iShift2D;

}

}

## Indication of Planar Mode

Planar prediction is applied and indicated at PU level as an additional non-directional intra mode as described in [3].

## Transform coding

Following the prediction, either DCT or DST can be utilized for prediction error coding. DST implementation is integrated as described in [3].

# Results

In this section, experimental results for various cases of the presented unified planar intra prediction mode are presented.  HM 2.0 anchors are used to compare the results. The configurations varied for the experiments are 1) Utilizing refinement (Use refinement or no refinement) and 2) DCT or DST coding. The results corresponding to these cases are presented in Tables 1 through 4.

Table 1 No refinement + DCT coding

Table 2 Refinement + DCT coding

Table 3 No refinement + DST coding

Table 4 Refinement + DST coding

# References

[1] A. Tabatabai, K. Chono, M. Coban, K. Kei, K. Sugimoto, A. Tanizawa “Description of Core Experiment 6 (CE6): Intra Prediction Improvement,” JCT-VC Doc. JCTVC-D606, Daegu, Jan 2011.

[2] Y. Jeon, S. Park, B. Jeon, “Non-directional intra prediction for coding efficiency improvement,” JCT-VC Doc. JCTVC-D083, Daegu, Jan 2011.

[3] S. Kanumuri, TK Tan, F. Bossen, “Enhancements to Intra Coding,” JCT-VC Doc. JCTVC-D235, Daegu, Jan 2011.

[4] J. Lainema, K. Ugur and O. Bici, “Planar intra coding for improved subjective video quality,” JCT-VC Doc. JCTVC-D326, Daegu, Jan 2011.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).