| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E176 |
| --- | --- |

| Title: | [AHG Screen Content Coding]Test sequences for screen content coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xingyu Zhang Oscar C. Au Dinuka Soysa Sijin Li Wei Dai Xing Wen Dept. of ECE, Hong Kong University of Science and Technology Clearwater Bay, Hong Kong | Tel: Email: | +852-2358-7053 eexyzhang@ust.hk |
| Source: | HKUST (The Hong Kong University of Science and Technology) |  |  |

_____________________________

# Introduction

This contribution describes and introduces a set of screen content video sequences that will be made available to JCT-VC and parent bodies by HKUST. HKUST proposes to adopt these sequences for the study and improvement of the effectiveness of HEVC standard for the coding of screen content.

# Content description

Three video sequences are provided. These sequences have the same resolution as Class E sequences (1280x720p). Table 1 shows the specifications of the proposed test sequences.

Table 1 Specification of 1280 × 720p format sequences

| Resolution | 1280 × 720p |
| --- | --- |
| Frame rate | 30 fps, progressive |
| Bit depth | 8 |
| Color format | YUV 4:2:0 |
| Duration | 10 seconds (300 frames) |

A brief description of the proposed test sequences is listed in Table 2, and selected frames are shown in Figure 1 to 3.

Table 2 Brief description of the sequences

| No. | Title of sequence | Brief description | Features |
| --- | --- | --- | --- |
| 1 | HKUST1 | Website browsing | - webpage scrolling vertically and horizontally - webpage zoom in and zoom out - flash animation - window size changing - rolling texts and graphics, etc. |
| 2 | HKUST2 | Slides show | - various fonts with different colors - shadows - animations - transition background - graphics - illumination change - fade in and fade out |
| 3 | HKUST3 | Document editing | - Chinese and English characters - art fonts - open new windows - text selection - typing |

(a) First frame

(b) Last frame

Figure 1: HKUST1 --- Website visiting

(a) First frame

(b) Last frame

Figure 2 : HKUST2---Slides show

(a) First frame

(b) Last frame

Figure 3 : HKUST3---Document Editing

# Conditions of Use – Copyright and Release Notes

These sequences have been generated by The Hong Kong University of Science and Technology (HKUST), and all intellectual property rights remain with HKUST. These materials shall only be used for developing high performance video coding standards in JCT-VC and in the parent bodies. The sequences should be processed in the original resolution. This copyright and permission notice shall be duplicated whenever the data are copied or distributed. These materials cannot be distributed with charge. HKUST makes no warranties with respect to the material and expressly disclaims any warranties regarding its fitness for any purpose. Unless the above conditions are agreed to by the recipient, no permission is granted for any use, copying, modification, and distribution of the data. By using these sequences the user agrees to the conditions of this copyright and disclaimer.