| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E179 |
| --- | --- |

| Title: | CE7: Cross check report of Panasonic’s proposal (Tool5 : JCTVC-E074) from Toshiba |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Akiyuki Tanizawa Jun Yamaguchi 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, JAPAN | Tel: Email: | +81-44-549-2152 akiyuki.tanizawa@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This contribution contains cross-checking results of the software containing the tool of JCTVC-D151 proposed by Panasonic on Core Experiment 7 of alternative transforms. The verification task has been done successfully and the results perfectly match those proposed in JCTVC-E074.

# Cross-checking

The software based on HM version 2.0 and the experimental results were provided on March 3 2011 by Panasonic. The software was complied fine and the cross-checking was performed on 64bit environments. The simulation has been conducted under CE common test conditions [1], [2] for both high efficiency and low complexity conditions in the case of I-slice only and random access coding structures. The encoded images match those of the decoded ones, and the PSNR and bitrate values are identical to the results provided by Panasonic. Table 1 and Table 2 show the cross-checking results. We checked the source code and confirmed that the technique integrated in the software matches JCTVC-E074 contributed by Panasonic completely.

Table 1: Cross-checking results for I slice only coding structure

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.2 | -1.3 | -1.2 | -1.6 | -0.5 | -0.4 |
| Class B | -0.9 | -1.0 | -1.1 | -1.5 | -0.7 | -0.7 |
| Class C | -1.1 | -1.2 | -1.1 | -1.7 | -1.0 | -1.0 |
| Class D | -1.1 | -1.0 | -1.0 | -1.6 | -0.8 | -0.8 |
| Class E | -1.7 | -2.2 | -2.0 | -2.4 | -1.0 | -1.3 |
| All | -1.1 | -1.3 | -1.2 | -1.7 | -0.8 | -0.8 |
| Enc Time[%] | 102% |  |  | 105% |  |  |
| Dec Time[%] | 102% |  |  | 107% |  |  |

Table 2: Cross-checking results for random access coding structure

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.6 | -0.3 | -0.5 | -0.7 | -0.2 | 0.1 |
| Class B | -0.4 | -0.4 | -0.4 | -0.7 | -0.2 | -0.2 |
| Class C | -0.6 | -0.6 | -0.5 | -0.8 | -0.5 | -0.4 |
| Class D | -0.5 | -0.4 | -0.3 | -0.7 | 0.1 | -0.2 |
| Class E |  |  |  |  |  |  |
| All | -0.5 | -0.4 | -0.4 | -0.7 | -0.2 | -0.2 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

In the above tables, it is noted that the Dec Time was calculated the decoding time ratio without outputting the yuv file for both anchor and proposal.

The accompanied spreadsheet contains the detailed data of the cross-checking results.

# References

[1] 	F. Bossen ," Common test conditions and software reference configurations ", ITU-T and ISO/IEC JCT-VC 4th Meeting Daegu, JCTVC-D600, January 2011.

[2] 	"Core Experiment 7: Alternative Transforms ", ITU-T and ISO/IEC JCT-VC 4th Meeting, Daegu, JCTVC-D607, January 2011.