| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E181 |
| --- | --- |

| Title: | CE12 Subset2: Parallel deblocking filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masaru Ikeda Junichi Tanaka Teruhiko Suzuki Gate City Osaki West Tower East Tec, 1-11-1, Osaki, Shinagawa-ku , Tokyo, 141-0032 JAPAN | Tel: Email: | +81-3-5435-3308 Masaru.Ikeda@jp.sony.com Tanaka.Junichi@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

In this contribution, parallel deblocking filter for HEVC is proposed. The proposal includes the followings:

- Parallelization of filtering

- Line-based filtering decision

- Parallelization of filtering and filtering decision

Parallelization of filtering can provide both frame-based processing for SW and LCU-based processing for HW. Y BD-rate for the parallelization of fitering (item2) is 0.0/0.0/0.0/0.0/0.0/0.0% for AI/AI_LC/RA/RA_LC/LD/LD_LC in average. Subjective quality is the almost same as HM-2.0. Run-time increases slightly and they are small enough, and additional memory isn’t required, but slight latency is required in LCU-based processing.

Line-based filtering decision can decide strong filter/weak filter/no filtering for each line in 8x8 block boundary. Y BD-rate for line-based filtering decision with parallelization of filtering (item3) is -0.1/-0.1/-0.2/-0.2/-0.4/-0.3% for AI/AI_LC/RA/RA_LC/LD/LD_LC in average. Subjective quality is improved in such as BQMall. Run-time increases slightly and they are small enough, additional memory isn’t required and the modification is quite simple.

Parallelization of filtering and filtering decision can provide further flexibility than parallelization of filtering. Y BD-rate for the parallelization of filtering and filtering decision (item1) is 0.1/0.1/0.1/0.1/0.1/0.1% for AI/AI_LC/RA/RA_LC/LD/LD_LC in average.

# Introduction

In HM-2.0 horizontal filter (across vertical edge) is dependent on vertical filter (across horizontal edge) between #n Largest Coding Unit (LCU) and #n+1 LCU because the filtering is first performed across vertical edges, then across horizontal edges in LCU. As shown Figure 1 the red circles show pixels performed by both vertical filter across horizontal edges (H1_0, H1_1) in #n LCU and horizontal filter across vertical edges (V2_0, V2_1) in #n+1 LCU. Vertical filtering (across horizontal edges which is shown as H1_0, H1_1) in #n LCU needs to be completed before horizontal filtering (across vertical edges which is shown as V2_0, V2_1) in #n+1 LCU, and so it is difficult to make deblocking filter parallel.

And edge decision for block boundary in HM-2.0 has two problems. First, the line used for the decision doesn’t correspond with the filtering for all lines in 8x8. Second, HM-2.0 has two-stage process in the decision, in which the filtering on/off is decided first and then the strong/weak filtering is decided.

Figure1-1: Dependency of vertical filtering and horizontal filtering between LCUs

Figure1-2: Edge decision for block boundary and selection of strong/weak filter

# Proposal

## Parallelization of filtering

### Basic idea

We (Sony) propose to make deblocking filter parallel by changing the order of horizontal filtering (across vertical edge) and vertical filtering (across horizontal edge). The way to change is as follows. First, horizontal filter is performed across all vertical edges in processing frame, and then operates across all horizontal edges in the frame.

By doing above the dependency between the same directional filters (horizontal filters/vertical filters) is removed, and so horizontal (vertical) filtering can be performed in parallel in processing frame.

Our (Sony’s) proposal can provide both frame-based processing for SW and LCU-based processing for HW.

### Frame-based processing

It is considered that the multi-core processing in SW increases more, and frame-based parallelization of filtering becomes more significant. The case of processing with 4 cores is shown in Figure 2-1. First, 4 cores perform horizontal filtering in parallel (as shown in 1, 2 and 3), and then perform vertical filtering in parallel (as shown in 4, 5 and 6), and so 4 cores can be used effectively.

Figure2-1: Case of processing with 4 cores

### LCU-based processing

It is considered that most of deblocking filter for HW is LCU-based processing. Our (Sony’s) proposal can provide LCU-based processing as shown in Figure2-2, Figure2-3 and Figure2-4.

- Horizontal filtering is performed across vertical edges in #n-LCU in Figure2-2.

- Vertical filtering is performed across horizontal edges in #n-LCU except for ones (as shown with pink squares) with pixels filtered by horizontal filtering in #n+1-LCU in Figure2-3

- Horizontal filtering is performed across vertical edges in #n+1-LCU, and then vertical filtering is performed across the rest of horizontal edges in #n-LCU in Figure2-4.

A little latency for #n-LCU is required because of above 3, but the latency is small enough.

Figure2-2: Horizontal filtering for #n-LCU

Figure2-3: Vertical filtering for #n-LCU

Figure2-4: Horizontal filtering for #n+1-LCU and vertical filtering for #n-LCU

## Line-based filtering decision

We (Sony) propose to decide whether each line for 8x8 block boundary is strong filtering, weak filtering or not. By doing this, strong filter/weak filter/filtering OFF can be decided for each line.

The definition of variable di in HM-2.0 is modified for each line. The variable di for each line is derived as follows:

di = | p2i – 2*p1i + p0i | + | q2i – 2*q1i + q0i |  ( i=0..7)

The filtering decision is modified as follows because of the definition of variable di.

- BS_decision is the same as HM-2.0.

- Filtering decision (parameters such as β, tC are the same as HM-2.0):

- If di is less than ( β >> 1 ), the following is applied (edge decision for block boundary):

- If di is less than ( β >> 3 ), | p3 – p0 | + | q0 – q3 | is less than ( β >> 3 ) and | p0 – q0 | is less than ( 5*tC + 1 ) >> 1, strong filter is applied.

- Otherwise, weak filter is applied.

  - Otherwise, the filtering is OFF.

Figure2-5: Line-based filtering decision in filtering decision for luma

## Parallelization of filtering and decision

We (Sony) propose to make filtering decision parallel in addition to parallelization of filtering (see 2.1) by centering the position for filtering decision as shown Figure2-6.

By doing above the dependency between all filtering decisions is removed, the flexibility increases.

Figure 2-6: Modification of the position for filtering decision

# Results

Experiment was carried out based on HM-2.0 and the configuration and common test condition are as JCTVC-D600 [1].

Both encoding and decoding time were measured on the platform which has Core i7 CPU2.8GHz, 8GB memory and the Linux 64bit (called Cent OS).

## Parallelization of filtering

### BD-rate and measurement of enc/dec-time

The results of BD-rate and measurement of enc/dec-time is shown in Table3-1, Table3-2 and Table3-3.

BD-rate for Y is 0.0 or -0.1% for all conditions (AI/AI_LC/RA/RA_LC/LD/LD_LC) and classes in average, and so the parallelization of filtering has the almost same performance as anchor.

Enc/dec-time for all conditions is under +4% in average. So the parallelization of filtering has the almost same run-time as anchor.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | -0.1 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

Table 3-1: Results for intra

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | -0.1 |
| Class B | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 104% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 3-2: Results for randomaccess

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | 0.0 | 0.0 | -0.1 | 0.0 |
| Class C | 0.0 | 0.1 | 0.0 | 0.0 | 0.1 | -0.3 |
| Class D | -0.1 | 0.0 | 0.1 | 0.0 | -0.1 | 0.1 |
| Class E | 0.0 | -0.4 | 0.3 | 0.0 | -0.3 | 0.1 |
| All | 0.0 | 0.0 | 0.1 | 0.0 | -0.1 | 0.0 |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 104% |  |  | 102% |  |  |

Table 3-3: Results for lowdelay

### Subjective quality

Subjective quality was evaluated with using below conditions and equipments.

Conditions

- Kimono1 randomaccess QP=37

- Kimono1 lowdelay_loco QP=32

- BQMall randomaccess QP=37

- BQMall lowdelay QP=32

- Vidyo3 lowdelay QP=37

- Vidyo3 lowdelay_loco QP=32

Equipments

- Monitor: EIZO FlexScan SX3031W

- Processor : Intel Core i5 750 BOX

- RAM : DCDDR3-4GB-1333

- Graphic Card : ENGTX285 2DI 1GD3

The difference from anchor is negligible, and so Sony’s proposal has no impact on subjective quality.

## Parallelization of filtering and line-based filtering decision

### BD-rate and measurement of enc/dec-time

The results of BD-rate and measurement of enc/dec-time is shown in Table3-4, Table3-5 and Table3-6.

Y BD-rate is -0.1/-0.1/-0.2/-0.2/-0.4/-0.3% for AI/AI_LD/RA/RA_LC/LD/LD_LC in average and has some gains for all conditions and classes. So this proposal brings about better performance than anchor.

Enc/dec-time for all conditions is under +3% in average. So this proposal has the almost same run-time as anchor.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.2 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
| Class B | -0.1 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
| Class C | -0.1 | 0.0 | 0.0 | -0.1 | -0.1 | -0.1 |
| Class D | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 | -0.1 |
| Class E | -0.2 | 0.0 | 0.0 | -0.1 | -0.1 | 0.0 |
| All | -0.1 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

Table 3-4: Results for intra

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.2 | -0.1 | 0.0 | -0.2 | 0.0 | 0.0 |
| Class B | -0.3 | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 |
| Class C | -0.2 | -0.1 | -0.1 | -0.2 | -0.1 | 0.0 |
| Class D | -0.1 | 0.0 | -0.1 | -0.1 | -0.1 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | -0.2 | 0.0 | 0.0 | -0.2 | -0.1 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

Table 3-5: Results for randomaccess

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.4 | -0.1 | -0.4 | -0.4 | -0.2 | 0.0 |
| Class C | -0.3 | 0.0 | 0.0 | -0.3 | -0.1 | -0.3 |
| Class D | -0.2 | -0.2 | 0.0 | -0.2 | -0.1 | 0.5 |
| Class E | -0.8 | -0.6 | 0.0 | -0.3 | -0.3 | -0.2 |
| All | -0.4 | -0.2 | -0.1 | -0.3 | -0.2 | 0.0 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  |

Table 3-6: Results for lowdelay

### Subjective quality

Subjective quality was evaluated with the same conditions and equipments as “Parallelization of filtering”.  It was confirmed that there were improved points in the line-based filtering decision. Decode and analysis images for BQMall in both HM-2.0 and line-based filtering decision (which doesn’t include parallelization of filtering ) are shown as below. Line-based filtering decision (which doesn’t include parallelization of filtering ) was compared with anchor to confirm the effect of only line-based filtering decision. And analysis image has both red and green mark, the red mark shows the pixels filtered by strong filter and the green mark shows the pixels filtered by weak filter.

Below result has 2 improved points which are shown by pink circle. Left one shows that image quality for line-based filtering decision is kept because weak filter for line-based filtering decision is selected instead of strong filter for anchor. Right one shows that image quality for line-based filtering decision is also kept because no filter for line-based filtering decision is selected instead of weak filter for anchor.

Figure3-1: Decode image for BQMall with intra_loco and QP=37 (Left: HM-2.0, Right: line-base filtering decision)

Figure3-2: Analysis image for BQMall with intra_loco and QP=37 (Left: HM-2.0, Right: line-based filtering decision)

## Parallelization of filtering and filtering decision

The results of BD-rate and measurement of enc/dec-time is shown in Table3-7, Table3-8 and Table3-9.

BD-rate for Y is under 0.4% for all conditions (AI/AI_LC/RA/RA_LC/LD/LD_LC) and classes in average, and so the parallelization of filtering and filtering decision has slight loss, but the loss is small enough.

Enc/dec-time for all conditions is under +2% in average. So the parallelization of filtering and filtering decision has the almost same run-time as anchor.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 |
| Class B | 0.1 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | -0.1 |
| Class D | 0.1 | 0.0 | 0.0 | 0.1 | 0.0 | -0.1 |
| Class E | 0.1 | 0.0 | 0.0 | 0.1 | -0.1 | 0.0 |
| All | 0.1 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |

Table3-7: Results for intra

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.2 | 0.2 | 0.0 | 0.0 | 0.0 |
| Class B | 0.1 | 0.1 | 0.1 | 0.1 | 0.0 | 0.0 |
| Class C | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.1 | -0.1 | -0.1 | 0.1 | -0.1 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.1 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table3-8: Results for randomaccess

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.2 | 0.3 | 0.0 | 0.2 | -0.1 | 0.2 |
| Class C | 0.1 | 0.2 | 0.0 | 0.0 | -0.1 | -0.2 |
| Class D | 0.0 | 0.3 | -0.2 | 0.1 | 0.2 | 0.2 |
| Class E | 0.4 | -0.3 | 0.5 | 0.3 | 0.4 | 0.5 |
| All | 0.1 | 0.2 | 0.1 | 0.1 | 0.1 | 0.2 |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |

Table3-9: Results for lowdelay

# Conclusion

In this contribution, the followings are proposed:

- Parallelization of filtering

- Line-based filtering decision

Parallelization of filtering can provide both frame-based processing for SW and LCU-based processing for HW. BD-rate and subjective quality are the almost same as HM-2.0. The difference for complexity such as run time, additional memory and so on is small enough.

Line-based filtering decision can decide strong filter/weak filter/no filtering for each line in 8x8 block boundary. BD-rate and subjective quality is improved. The difference for complexity such as run time, additional memory and so on is small enough.

Therefore we (Sony) recommend that above proposals are adopted into HM.

# References

[1]JCT-VC, “Common test conditions and software reference configurations”, JCTVC-D600,4th. JCT-VC Meeting, Daegu, KR, January 2011

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).