| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March 2011 | Document: JCTVC-E183 WG11 Number: m19701 |
| --- | --- |

| Title: | Cross-verification report of proposal on RQT root location (JCTVC-E364) |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JCT-VC] |  |  |
| Purpose: | [Report] |  |  |
| Author(s) or Contact(s): | Keiichi Chono Hirofumi Aoki Yuzo Senda 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 431 7595 chono@ct.jp.nec.com hao@bu.jp.nec.com y-senda@bc.jp.nec.com |
| Source: | [Information and Media Processing Labs, NEC Corporation] |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results of proposal on RQT root location described in JCTVC-E364. Cross-verification task has been successfully completed, and it is confirmed that the BD-rate results match those provided by proponent.

- Introduction

JCTVC-D250 proposes an efficient transform unit representation scheme in which given an inter PU, its TU size is set to the size of the largest possible square block that can fit into the PU. Hereafter the scheme is referred as Implicit Inter Residual Quad Tree (IIRQT). This contribution reports cross-verification results of HM2.0-based IIRQT as described in JCTVC-E364.

# Software analysis

Through the software inspection, it was confirmed that software implementation of IIRQT is identical to JCTVC D250 description and that 16x16 and 32x32 HADs are additionally introduced into the software, as indicated by the proponents.

- Results

Software provided by Motorola was successfully compiled by Microsoft Visual C++ 2008, and its 64-bit encode and decode executables were tested on Windows-based computing platform below.

| Computing platform | Usage |
| --- | --- |
| Windows 7 64-bit Professional Intel Xeon 3.33GHz and Memory 32GB | Up to 12 processes for encoding and decoding ran at the same time. |

In order to see the performance of large HADs as well as IIRQT, the following three simulations are conducted according to the common test coding conditions described in JCTVC-D600:

IIRQT: Enable IIRQT.

HAD: Enable large HADs

IIRQT+HAD: Enable both IIRQT and large HADs.

Table 1 shows the summary of three results. Attached Excel sheets, 20110225-HM-2.0-Anchor-vs-MOTOROLA-IIRQT.xls, 20110225-HM-2.0-Anchor-vs-HAD.xls, and 20110225-HM-2.0-Anchor-vs-IIRQT+HAD.xls, contain the detailed results. It was confirmed that BD-PSNR/Rate values of IIRQT are the same as those provided by proponent.

Table 1: Summary of simulation results.

- Reference

- K. Panusopone, X. Fang, and L. Wang, "Evaluation of RQT in HM and related TU representation", JCTVC-E364, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March 2011.

- Krit Panusopone, Xue Fang, and Limin Wang, "Efficient Transform Unit Representation," JCTVC-D250, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-D600, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011.

# Patent rights declaration(s)

NEC Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).