| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March 2011 | Document: JCTVC-E185 WG11 Number: m19703 |
| --- | --- |

| Title: | CE6: Cross-verification report of planar intra prediction (JCTVC-D235) |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JCT-VC] |  |  |
| Purpose: | [Report] |  |  |
| Author(s) or Contact(s): | Keiichi Chono Hirofumi Aoki Yuzo Senda 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 431 7595 chono@ct.jp.nec.com hao@bu.jp.nec.com y-senda@bc.jp.nec.com |
| Source: | [Information and Media Processing Labs, NEC Corporation] |  |  |

_____________________________

# Abstract

This contribution presents software analysis and simulation results of JCTVC-D235 and its related planar intra predictions. Based on the simulation results, it is recommended that if HM3.0 adopts planar intra prediction, it should adopt JCTVC-D235 planar intra prediction as an additional prediction mode in PU level.

- Introduction

The CE6.e (Planar intra prediction) [1] has three test items below in order to evaluate performance of different configurations and combinations of proposed planar intra prediction methods. Test item 1 excluding 1.e, test item 2, and test item 3 except 3.c were evaluated by using NTT DOCOMO software. This contribution reports the software analysis and its simulation results.

Test item 1: Performance comparison of planar intra prediction schemes (by replacing DC mode with planar intra prediction mode)

1.a.	Planar prediction as defined in JCTVC-D326 [5], disabling the adjustment for corner sample (bilinear prediction of samples)

1.b.	Planar prediction as defined in JCTVC-D326 with the JCTVC-D083 [3] refinement step

1.c.	Planar prediction as defined in JCTVC-D235  [4] (averaging two linear interpolations)

1.d.	Plane prediction as defined in JCTVC-D026 [2] (TMuC plane mode)

1.e.	Planar prediction as defined in JCTVC-D326, disabling the adjustment for corner sample (multiplication-free version of 1.a)

1.f.	Planar prediction as defined in JCTVC-D235 with a modified version of the JCTVC-D083 refinement step

Test item 2: Performance comparison of HM DCT and DST for planar intra prediction error signal

2.a.	DCT

2.b.	DST

Test item 3: Performance comparison of planar intra prediction mode signaling methods

3.a.	PU level indication substituting DC mode

3.b.	PU level signaling as an additional prediction mode as defined in JCTVC-D235

3.c.	CU level indication jointly for all color components as defined in JCTVC-D326

- Software analysis

The following functions are confirmed from the software provided by NTT DOCOMO:

- Support three planar intra predictions as described in CE6 document for evaluating 1.a, 1.b, and 1.c ;

- Replace DC intra prediction for luma and chroma PUs by planar intra prediction for evaluating the test item 1 and 2;

- Apply DST to planar intra prediction errors of luma and chroma TUs smaller than 32x32 for evaluating the test item 1 and 2;

- Use and signal planar intra prediction mode as an additional intra prediction mode in PU level for the evaluating test item 3; single bit syntax is additionally transmitted after intra mode index =2 as described in JCTVC-D235.

In addition, the following two functions added to the NTT DOCOMO software by LG and NOKIA are confirmed:

- Replace 1.a planar intra prediction with a multiplier-free version for evaluating 1.e; (Note that Planar intra prediction signals are identical.)

- Refine top-right and bottom-left samples with [1 2 1]/4 filter prior to generating JCTVC-D235 planar intra prediction as inferred from the description of JCTVC-D083 for evaluating 1.f evaluation.

- Simulation results

Software provided by NTTDOCOMO with the LG and NOKIA modifications was successfully compiled by Microsoft Visual C++ 2008, and its 64-bit encode and decode executables were tested on Windows-based computing platform below.

| Computing platform | Usage |
| --- | --- |
| Windows 7 64-bit Professional Core i7 2.8GHz and Memory 12GB | Up to 4 processes for encoding and decoding ran at the same time. |

Test items described in Table1 were tested, and it was confirmed that BD-rates are identical to those of proponents in all test items.

Table 1: List of test items and Excel file

Based on the simulation results, the following are remarked:

- D235 and its variant (D235 with the D083 refinement step) seem to be good planar intra prediction methods in terms of the balance of coding performance and complexity;

- DST seems to be a good transform for planar intra prediction error signal;

- Introducing planar mode as an additional prediction mode in PU level seems to be a better approach than replacing DC mode with planar mode.

# Subjective quality analysis

We compared Anchor, 1.c+3.b, and 1.c+2.b+3.b RA results with QP=32 of “Kimono” sequence by using a 56inch 4kx2k display (Sony SRM 560). Anchor and a test case (either 1.c+3.b or 1.c+2.b+3.b) were displayed in a side-by-side arrangement. Attached file, “NEC-NTTDOCOMO-Planar-pics.pdf”, shows some representative screen shots of coded results. It was observed that blocking artifacts in the face and sky regions are significantly reduced by both 1.c+3.b and 1.c+2.b+3.b, especially in Low Complexity setting, and that the visual qualities of 1.c+3.b and 1.c+2.b+3.b are very comparable.

Based on the subjective quality analysis, the following should be noted:

- Planar intra prediction reduces blocking artifacts in smooth image areas;

- DST and DCT on the top of planar intra prediction do not differentiate visual appearance of smooth image areas.

# Conclusions

This contribution has presented software analysis and simulation results of JCTVC-D235 and its related planar intra predictions.

- Reference

- JCT-VC, “Core Experiment 6: Intra Prediction Improvement”, JCTVC-D606, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011.

- Guichun Li, Lingzhi Liu , Nam Ling, Jianhua Zheng, and Philipp Zhang, "CE6.a.5: Santa Clara University and Hisilicon Report on Block Based Intra Prediction", JCTVC-D026

- Yongjoon Jeon, Seungwook Park, and Byeongmoon Jeon, "Non-directional intra prediction for coding efficiency improvement", JCTVC-D083, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011.

- Sandeep Kanumuri, TK Tan, and Frank Bossen, "Enhancements to Intra Coding", JCTVC-D235, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011.

- Jani Lainema, Kemal Ugur, and Oguz Bici, "Planar intra coding for improved subjective video quality", JCTVC-D326, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-D600, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011.

# Patent rights declaration(s)

NEC Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).