| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March 2011 | Document: JCTVC-E189 WG11 Number: m19707 |
| --- | --- |

| Title: | Cross-verification report on memory bandwidth reduction MC filter (JCTVC-E129) |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JCT-VC] |  |  |
| Purpose: | [Report] |  |  |
| Author(s) or Contact(s): | Keiichi Chono Hirofumi Aoki Yuzo Senda 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 431 7595 chono@ct.jp.nec.com hao@bu.jp.nec.com y-senda@bc.jp.nec.com |
| Source: | [Information and Media Processing Labs, NEC Corporation] |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results of memory bandwidth reduction MC filter described in JCTVC-E129. Cross-verification task has been successfully completed, and it is confirmed that the BD-rate results match those provided by proponent.

- Software analysis

The following modifications in HM luma sub-pel MC process are confirmed through the analysis of software provided by SONY:

- Switch filter coefficients according to the sample position in an MC block:

  - If the sample position to be interpolated locates on around MC block boundaries, its sample value at the sub-pel position determined by motion vector is interpolated by using modified versions of the HM DCITF filter coefficients.

  - Otherwise, its sample value is interpolated by using the HM DCITF filter coefficients.

- The modified DCITF filter coefficients are designed not to refer to outside the sample values at full-pel positions referred by the interpolation with the HM DCITF filter coefficients.

The software supports different sets of variety of modified DCITF filter coefficients. NEC tested one set whose memory access to reference picture memory is the same as that by using 6-tap and 4-tap filters for horizontal and vertical interpolations, as depicted in Figure 1.

Figure 1	Example of MC reference samples for 4x4 MC block.

# Simulation results

Software provided by SONY [1] was successfully compiled by Microsoft Visual C++ 2008, and its 64-bit encode and decode executables were tested on Windows-based computing platform below.

| Computing platform | Usage |
| --- | --- |
| Windows 7 64-bit Professional Core i7 2.8GHz and Memory 12GB | Up to 4 processes for encoding and decoding ran at the same time. |

Simulation was conducted according to the common test coding conditions described in JCTVC-D600 [2] Attached Excel sheet, NEC-HM-1.0rev569-noyuv-Anchor-vs-SonyMC1.xls, contains the detailed results. It was confirmed that BD-PSNR/Rate values obtained are the same as those of provided by proponent.

# Conclusion

It is confirmed that JCTVC-E129 results are accurate.

- Reference

- Kenji Kondo and Teruhiko Suzuki, "Memory bandwidth reduction MC filter", JCTVC-E129, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March 2011.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-D600, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011.

# Patent rights declaration(s)

NEC Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).