| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E190 |
| --- | --- |

| Title: | Adaptive loop filtering using directional activity |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kei Kawamura Tomonobu Yoshino Sei Naito 2-1-15, Ohara, Fujimino-shi, Saitama, JAPAN | Tel: Email: | +81 49 278 7411 ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI R&D Laboratories) |  |  |

_____________________________

# Abstract

HM2.0 employs QC-ALF as an adaptive loop filtering approach. QC-ALF classifies each pixel based on activity of decoded pixel value, and Wiener filter coefficients are calculated for every category. However, QC-ALF doesn't consider activity direction when evaluating the activity. Thus, it is difficult to design an appropriate loop filter for the texture with various edge directions. This contribution introduces a loop filtering approach that considers the activity direction to evaluate the activity from the result of Laplacian operation. Experimental result shows BD-rate against HM2.0 reaches -0.3% and -0.4% for LD/HE and RA/HE, respectively.

# Introduction

QC-ALF implemented in HM2.0 classifies each pixel into up to 16 categories based on activity of decoded pixel value, and Wiener filter coefficients are calculated for every category. However, since QC-ALF doesn't consider the activity direction, inappropriate filter might be assigned especially for the content with various edge directions. To overcome the problem, directional activity should be considered in the filter design process.

From this perspective, this contribution proposes an adaptive loop filtering approach that considers directional activity. This approach evaluates both horizontal and vertical activity, respectively. The detail is shown in Section 2.

# Adaptive Loop Filtering using Directional Activity (ALF-DA)

This approach utilizes Laplacian operation for activity evaluation as in the case of QC-ALF. However, ALF-DA independently evaluates horizontal and vertical activity, respectively. Equation (1) and (2) indicates evaluation of horizontal and vertical activity, respectively. xi,j denotes pixel value at pixel position (i,j), Hi,j and Vi,j denotes activity value of horizontal and vertical direction, respectively.

Hi,j = (xi+1,j-1 + 2xi+1,j + xi+1,j+1) - (xi-1,j-1 + 2xi-1,j + xi-1,j+1)						(1)

Vi,j = (xi-1,j+1 + 2xi,j+1 + xi+1,j+1) - (xi-1,j-1 + 2xi,j-1 + xi+1,j-1)						(2)

In, ALF-DA, the activity for each pixel is calculated by the absolute value of Hi,j and Vi,j as horizontal and vertical component, respectively. The activity value is scaled into range among 0 to 15. Each pixel is categorized into N types of subset based on N-1 threshold parameters. Wiener filter coefficients are calculated for each categorized subset, and the corresponding filter coefficients are applied to each decoded pixel.

Actually, in this contribution, each pixel is categorized into 3 subsets based on 2 threshold parameters (2.0 and 7.0). For descriptive purposes, "Category 1" indicates the lowest category, and "Category 3" indicates the highest category.

In this situation, there are 9 cases based on combination of horizontal and vertical 3 types of category. Table 1 shows relationship between category combination and filter case. Therefore, the number of encoding-pass is 9.

Table 1 Categorization

|  |  | Horizontal activity |  |  |
| --- | --- | --- | --- | --- |
|  |  | category 1 | category 2 | category 3 |
| Vertical activity | category 1 | Case 1 | Case 2 | Case 3 |
|  | category 2 | Case 4 | Case 5 | Case 6 |
|  | category 3 | Case 7 | Case 8 | Case 9 |

# Experimental Results

We conducted a coding experiment to evaluate the coding performance. The proposed scheme introduced in Section 2 was implemented on HM2.0. Table 2 and 3 show the BD-rateY against HM2.0 under LD/HE and RA/HE, respectively.

Table 2 : summary of the results for LD/HE

|  | LD/HE |
| --- | --- |
|  | Y BD-rate |
| Class A |  |
| Class B | -0.39 |
| Class C | -0.11 |
| Class D | -0.19 |
| Class E | -0.64 |
| All | -0.32 |
| Enc Time[%] | 103% |
| Dec Time[%] | 110% |

Table 3 : summary of the results for RA/HE

|  | RA/HE |
| --- | --- |
|  | Y BD-rate |
| Class A | -0.40 |
| Class B | -0.64 |
| Class C | -0.23 |
| Class D | -0.21 |
| Class E |  |
| All | -0.39 |
| Enc Time[%] | 103% |
| Dec Time[%] | 110% |

# Conclusion

In this contribution, an adaptive loop filtering approach using directional activity was introduced. The BD-rateY against HM2.0 reached 0.4% in the case of RA/HE while reducing the number of encoding pass. Encoding time and decoding time approximately increased 3% and 10%, respectively. This result implies that ALF has potential of further coding performance improvement while reducing the number of encoding pass. The proposed approach should be adopted as the technical issue in CE8.

# Patent rights declaration(s)

KDDI Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).