| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March 2011 | Document: JCTVC-E191 WG11 Number: m19709 |
| --- | --- |

| Title: | Cross-verification report on temporal predictor restriction method (JCTVC-E097) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Keiichi Chono Hirofumi Aoki Yuzo Senda 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 431 7595 chono@ct.jp.nec.com hao@bu.jp.nec.com y-senda@bc.jp.nec.com |
| Source: | NEC Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results of temporal predictor restriction method described in JCTVC-E097. The cross-verification tasks were successfully completed and it was confirmed that the results exactly match those presented in JCTVC-E097.

- Results

Software provided by JVC KENWOOD [1] was successfully compiled by Microsoft Visual C++ 2008, and its 64-bit encode and decode executables were tested on Windows-based computing platform below.

| Computing platform | Usage |
| --- | --- |
| OS: Windows 7 64-bit Professional CPU: Xeon X5680 3.46GHz (12 cores) Memory: 12GiB | Up to 12 processes for encoding and decoding ran at the same time. |

Simulation was conducted according to the common test coding conditions described in JCTVC-D600 [1]. Table 1 shows the summary of results. An attached spreadsheet, JCT-E191.xls, contains the detailed results. It was confirmed that BD-PSNR/Rate values are exactly same as those provided by the proponent.

Almost no gains are observed from these results, although the method potentially brings coding efficiency depending on test conditions. The difference and encoding and decoding times are negligible.

Table 1		Summary of results.

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.1 | 0.0 | 0.1 | -0.1 | 0.2 |
| Class B | 0.1 | 0.0 | -0.1 | -0.1 | -0.1 | -0.1 |
| Class C | 0.2 | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class D | 0.4 | 0.2 | 0.1 | 0.3 | -0.1 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.2 | 0.1 | 0.0 | 0.1 | -0.1 | 0.0 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.1 | -0.2 | -0.3 | -0.2 | -0.3 | -0.1 |
| Class C | 0.4 | 0.4 | 0.1 | 0.2 | 0.1 | 0.0 |
| Class D | 0.2 | -0.2 | -0.6 | 0.2 | -0.1 | -0.2 |
| Class E | -0.2 | -0.2 | 0.0 | -0.4 | -0.3 | -0.2 |
| All | 0.1 | 0.0 | -0.2 | 0.0 | -0.1 | -0.1 |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 98% |  |  |

# Software analysis and remarks

It was confirmed that the software disables the use of temporal predictor candidates of Inter PU partitions smaller than or equal to the size of motion vector storage in accordance with the JCTVC-E097 description. The number of access times to the temporal predictor at the worst case can be reduced by the proposal, as described in JCTVC-E097.

- Reference

- Hideki Takehara and Shigeru Fukushima, "Temporal predictor restriction harmonized with motion vector compression", JCTVC-E097, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March 2011.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-D600, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011.

# Patent rights declaration(s)

NEC Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).