| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, 16-23 March, 2011 | Document: JCTVC-E193 WG11 Number: m19711 |
| --- | --- |

| Title: | CE8.1: Deblocking parameter adjustment for 1-input ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage1 Saori Asaka1 Takashi Watanabe1 Takeshi Chujoh1 Ching-Yeh Chen2 Chih-Ming Fu2 Chia-Yang Tsai2 Yu-Wen Huang2 Shawmin Lei2 Marta Karczewicz3 In Suk Chong3 | Email: | tomoo.yamakage@toshiba.co.jp |
| Source: | Toshiba Corporation1, MediaTek Inc.2 and Qualcomm3 |  |  |

_____________________________

# Abstract

This contribution proposes appropriate parameters for deblocking filter to improve coding efficiency for CE8 Subtest 1: Number of inputs.  In this contribution, both  and tc in the current HM 2 design are offset slightly so that the impact on subjective picture quality is negligible.  When using  offset of -2 and tc offset of -4, coding efficiency improvements are 0.7% for I Only case and 0.5 % for Random Access and Low delay cases for High Efficiency configuration described in JCTVC-D600 and JCTVC-D608.  Encoding and decoding time are the same as those for the anchor.  In Annex, there are additional results of coding efficiency improvement for combination of  and tc offsets using the first 2 seconds of each test sequence.  Results are verified by Ericsson.

# Introduction

At the Deagu meeting in Jan. 2011, a contribution (JCTVC-D192[1]) reported that there would be an interaction between adaptive loop filter and deblocking filter.  The contribution modified  value drastically (e.g. -10) and obtained coding efficiency improvements of 0.8% for I Only, 0.5% for Random Access and 1.0% for Low Delay cases.  There was a comment that modifying  value might affect subjective picture quality.

In this contribution, both  and tc in the current HM 2 design are offset slightly so that the impact on subjective picture quality is negligible.  When using  offset of -2 and tc offset of -4, coding efficiency improvement is 0.7% for I Only case and 0.5 % for Random Access and Low delay cases for High Efficiency configuration described in JCTVC-D600.  Encoding and decoding time are the same as those for the anchor.  In Annex, there are additional results of coding efficiency improvement for combination of  and tc offsets using the first 2 seconds of each test sequence.  Results are verified by Ericsson.

# Modification points from HM 2.0

Since the current HM 2.0 software does not support interface to offset  or tc, an interface to support offsetting is introduced.  In addition, slice header syntax of WD1 should be modified to support tc offsetting instead of  offsetting.

When adjusting the offsets, values of 0, -2, and -4 are tested for the first 2 seconds of each test sequence to determine an appropriate offsets.  These results are shown in Annex.  As a result, offsets of  = -2 and tc = -4 are chosen considering the balance of coding efficiency and subjective picture quality.

There is no additional modification to the current HM 2.0 software other than offsetting  and tc (e.g. number of coefs., filter shapes, encoding technique, etc.).

| slice_header( ) { | Descriptor |
| --- | --- |
| first_tb_in_slice | ue(v) |
| ………… |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_tc_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |
| } |  |
| } |  |

# Experimental results for full length sequences

Experiments are conducted based on the test condition JCTVC-D600 [2] and JCTVC-D608 [3] for High Efficiency cases.  Offsets of  = -2 and tc = -4 are used.  Table 1 shows experimental results.  Coding efficiency improvement is 0.7% for I Only case and 0.5 % for Random Access and Low delay cases with no impact of encoding and decoding time.  Regarding the subjective picture quality, an informal viewing was conducted by using CG-301W by the proponent.  Several sequences that were used for informal subjective viewing in TE10 and previous CE8 are used to compare the anchor and the proposal.  Viewing results are similar to those of TE10 and previous CE8, which means that there is not a significant difference between the anchor and the proposal.

Table 1  Results for High Efficiency conditions

| High Efficiency |  |  | IO | RA | LD |
| --- | --- | --- | --- | --- | --- |
| A | S01 | Traffic | -1.0 | -1.2 |  |
|  | S02 | PeopleOnStreet | -1.1 | -0.3 |  |
|  | Sxx | Nebuta | -0.9 | -0.4 |  |
|  | Syy | SteamLocomotive | -0.9 | -0.8 |  |
| B | S03 | Kimono | -0.6 | -0.2 | -0.1 |
|  | S04 | ParkScene | -1.2 | -1.0 | -1.2 |
|  | S05 | Cactus | -0.8 | -0.7 | -0.6 |
|  | S06 | BasketballDrive | -0.7 | -0.1 | -0.1 |
|  | S07 | BQTerrace | -0.6 | -0.7 | -1.0 |
| C | S08 | BasketballDrill | -0.8 | -0.5 | -0.4 |
|  | S09 | BQMall | -0.6 | -0.4 | -0.3 |
|  | S10 | PartyScene | -0.4 | -0.2 | -0.2 |
|  | S11 | RaceHorses | -0.6 | -0.2 | -0.1 |
| D | S12 | BasketballPass | -0.7 | -0.1 | -0.1 |
|  | S13 | BQSquare | -0.2 | -0.2 | -0.4 |
|  | S14 | BlowingBubbles | -0.5 | -0.3 | -0.5 |
|  | S15 | RaceHorses | -0.8 | -0.1 | -0.3 |
| E | S16 | Vidyo1 | -0.7 |  | -1.1 |
|  | S17 | Vidyo3 | -0.8 |  | -1.3 |
|  | S18 | Vidyo4 | -0.8 |  | -1.0 |
| Class A |  |  | -1.0 | -0.7 |  |
| Class B |  |  | -0.8 | -0.6 | -0.6 |
| Class C |  |  | -0.6 | -0.3 | -0.2 |
| Class D |  |  | -0.5 | -0.2 | -0.3 |
| Class E |  |  | -0.8 |  | -1.1 |
| Total |  |  | -0.7 | -0.5 | -0.5 |
| Encoding time |  |  | 100% | 100% | 100% |
| Decoding time |  |  | 100% | 100% | 100% |

# Conclusion

In this contribution, it is shown that using  offset of -2 and tc offset of -4 improve coding efficiency (0.7% for I Only case and 0.5 % for Random Access and Low delay cases) with negligible impact on subjective picture quality.  It is recommended to use  offset of -2 and tc offset of -4 for the common test conditions.  In addition it is recommended to add an interface to support offsetting of  and tc to HM software and modify WD text as appropriate.  Results are verified by Ericsson.

# References

[1]	T. Yamakage, S. Asaka, A. Tanizawa, T. Watanabe, T. Chujoh, “Analysis on the interaction between deblocking filtering and in-loop filtering”, JCTVC-C192_r2, January 2011.

[2]	F. Bossen, “Common test conditions and software reference configurations”, JCTVC-D600, January 2011.

[3]	T. Yamakage, Y.-W. Huang, I. S. Chong, T. Ikai, M. Narroschke, W. L. Lai, A. Tabatabai, M. Budagavi, F. Kossentini, “Description of CE8: Non-deblocking loop filtering”, JCTVC-D608_r1, January 2011.

# Patent rights declaration(s)

Toshiba Corporation, MediaTek Inc. and Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex  Experimental results for short length (2 sec.) sequences

| Intra Only |  |  BD-rate [%] |  |  |  | Deblocked pixels (strong/weak) [%] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  offset |  |  |  |  offset |  |  |  |
|  |  | 0 | -2 | -4 | -6 | 0 | -2 | -4 | -6 |
| tc offset | 0 | 0.0 | -0.2 | -0.4 | -0.5 | 70.1 (23.5/46.6) |  | 64.2 (20.1/44.2) | 61.0 (18.5/42.5) |
|  | -2 | -0.6 | -0.7 | -0.8 | -0.9 |  |  |  |  |
|  | -4 | -0.6 | -0.7 | -0.8 | -0.9 | 70.1 (22.9/47.2) | 67.4 (21.3/46.1) | 64.2 (19.7/44.6) |  |
|  | -6 | -1.0 | -1.1 | -1.1 | -1.1 | 70.1 (21.9/48.2) |  |  | 61.0 (17.6/43.4) |

| Random Access |  |  BD-rate [%] |  |  |  | Deblocked pixels (strong/weak) [%] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  offset |  |  |  |  offset |  |  |  |
|  |  | 0 | -2 | -4 | -6 | 0 | -2 | -4 | -6 |
| tc offset | 0 | 0.0 | -0.1 | -0.3 | -0.4 | 69.0 (18.3/50.6) |  | 63.0 (14.7/48.3) | 59.8 (13.0/46.8) |
|  | -2 | -0.3 | -0.4 | -0.4 | -0.5 |  |  |  |  |
|  | -4 | -0.4 | -0.5 | -0.5 | -0.5 | 68.9 (17.8/51.1) | 66.9 (16.1/49.9) | 63.0 (14.4/48.6) |  |
|  | -6 | -0.5 | -0.5 | -0.5 | -0.5 | 68.8 (17.2/51.7) |  |  | 59.7 (12.4/47.3) |

| Random Access |  |  BD-rate [%] |  |  |  | Deblocked pixels (strong/weak) [%] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  offset |  |  |  |  offset |  |  |  |
|  |  | 0 | -2 | -4 | -6 | 0 | -2 | -4 | -6 |
| tc offset | 0 | 0.0 | -0.2 | -0.4 | -0.5 | 69.0 (19.5/49.5) |  | 63.7 (15.6/48.2) | 60.6 (13.4/47.2) |
|  | -2 | -0.3 | -0.4 | -0.5 | -0.6 |  |  |  |  |
|  | -4 | -0.4 | -0.5 | -0.5 | -0.6 | 68.9 (18.9/50.0) | 66.4 (17.1/49.3) | 63.6 (15.2/48.4) |  |
|  | -6 | -0.5 | -0.5 | -0.5 | -0.5 | 68.9 (18.1/50.7) |  |  | 60.5 (12.9/47.6) |