| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, 16-23 March, 2011 | Document: JCTVC-E194_r1 WG11 Number: m19712 |
| --- | --- |

| Title: | Unified Chroma Filter Shape with Luma for ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage1 Takashi Watanabe1 Takeshi Chujoh1 Ching-Yeh Chen2 Chih-Ming Fu2 Chia-Yang Tsai2 Yu-Wen Huang2 Shawmin Lei2 Marta Karczewicz3 In Suk Chong3 | Email: | tomoo.yamakage@toshiba.co.jp takashi39.watanabe@toshiba.co.jp takeshi.chujoh@toshiba.co.jp chingyeh.chen@mediatek.com chihming.fu@mediatek.com chiayang.tsai@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com martak@qualcomm.com ichong@qualcomm.com |
| Source: | TOSHIBA Corporation1, MediaTek Inc.2 and Qualcomm3 |  |  |

_____________________________

# Abstract

This contribution presents coding results of replacing adaptive loop filters for chroma with the same shape as luma’s.  There is no coding efficiency loss for luma, but 0.2-0.7% losses for chroma, while saving the decoding time with 1-4%.  Coding results were confirmed by TI.  Encoding and decoding time have discrepancy which can be attributed to difference of the operating environment.  It is recommended to replace the current rectangular shape with the same shape as luma in order to reduce the complexity.

# Introduction

At the Deagu meeting in Jan. 2011, it was suggested to study to use the same shapes for chroma adaptive loop filtering (ALF) as luma’s instead of the rectangular shape in HM 2.  This contribution reports coding results of replacing the current rectangular shape the same shape as luma.  5x5-tap diamond filter is only used for chroma in the experiment.

Based on the common test conditions in JCTVC-D600 and relevant CE 8 description JCTVC-D608, a test is conducted.  There is no coding efficiency loss for luma, but 0.2-0.7% losses for chroma, while saving the decoding time with 1-4%.  Coding results were confirmed by TI.  Encoding and decoding time have discrepancy which can be attributed to difference of the operating environment.

Since this replacement does not only reduce the decoding time, but also would simplify the implementation and its verification, it is recommended to replace the current rectangular shape with the same shape as luma in order to reduce the complexity.

# Experimental Results

Experiments are conducted based on the test condition JCTVC-D600 [1] and JCTVC-D608 [2] for High Efficiency cases.  The modification point of the HM 2.0 software is only the shape of the chroma filtering.  No encoding strategy nor other portion is modified.

Table 1 shows experimental results.  Coding efficiency for luma is unchanged, but it is observed that BD-rate for chroma is 0.2-0.7% on average (positive value means loss of coding efficiency).  Additional results using 7x7 diamond filter is provided in the attached Excel sheet.

Table 1  Results for High Efficiency conditions

|  | IO |  |  | RA |  |  | LD |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.1 | 0.1 | 0.1 | 0.3 | 0.0 |  |  |  |
| Class B | 0.0 | 0.4 | 0.2 | 0.0 | 0.7 | 0.4 | 0.0 | 0.7 | 0.6 |
| Class C | 0.0 | 0.4 | 0.6 | 0.0 | 0.4 | 0.9 | 0.0 | 0.5 | 1.2 |
| Class D | 0.0 | 0.1 | 0.2 | 0.0 | 0.1 | 0.2 | -0.1 | 0.2 | 0.0 |
| Class E | 0.0 | 0.0 | 0.3 |  |  |  | -0.1 | 1.2 | 0.9 |
| Total | 0.0 | 0.2 | 0.3 | 0.0 | 0.4 | 0.4 | 0.0 | 0.6 | 0.7 |
| Encoding time | 99% |  |  | 100% |  |  | 100% |  |  |
| Decoding time | 96% |  |  | 98% |  |  | 99% |  |  |

# Conclusion

In this contribution, coding results of replacing adaptive loop filters for chroma with the same shape as luma’s are reported.  5x5-tap diamond filter is only used for chroma in the experiment.  Since there is no coding efficiency loss in luma and small loss in chroma, it is recommended to replace the current rectangular shape with the same shape as luma in order to reduce the complexity.

# References

[1	F. Bossen, “Common test conditions and software reference configurations”, JCTVC-D600, January 2011.

[2	T. Yamakage, Y.-W. Huang, I. S. Chong, T. Ikai, M. Narroschke, W. L. Lai, A. Tabatabai, M. Budagavi, F. Kossentini, “Description of CE8: Non-deblocking loop filtering”, JCTVC-D608_r1, January 2011.

# Patent rights declaration(s)

TOSHIBA Corporation, MediaTek Inc. and Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).