| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E196 WG11 Number: m19714 |
| --- | --- |

| Title: | Wavefront Parallel Processing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Félix Henry (Orange Labs) Stéphane Pateux (Orange Labs) | Tel: Email: | +33 2 99 12 45 58 felix.henry@orange-ftgroup.com stephane.pateux@orange-ftgroup.com |
| Source: | Orange Labs |  |  |

_____________________________

# Abstract

This contribution describes a method to perform parallel encoding and decoding of video using HEVC. As recommended in JCTVC-D243, interleaved lines of LCU are processed by the encoding/decoding threads. In order to limit the degradation of performance, a wavefront pattern of processing ensures that spatial and motion vectors dependency is fully preserved, as recommended in JCTVC-D073. On the other hand, a single additional probability buffer is used to synchronize CABAC along the second column of LCUs. It is asserted that this method provides a fixed encoding/decoding design with thread-independent bitstream content. The average BD-rate degradation is +0.7% (intra +0.1%, random access +0.8%, low delay +1.3%). It is also asserted that this method can be viewed as an extension of Entropy Slices with LCU-limit.

# Introduction

Multi-core architectures become more widely available on a variety of devices including mobile terminals, and these terminals are no longer used for content display, but also for content production. Consequently, efficient parallelism of the HEVC video encoder and decoder are important features of the future HEVC standard.

The concept of wavefront processing of the LCU has already been proposed for the decoding process of the buffered symbols when using Entropy Slices (ES) such as defined in JCTVC-D070 “Lightweight slicing for entropy coding” [1]. Using wavefront processing allows maintaining spatial, motion vectors, and in-loop filter dependency between LCUs. This dependency is mandatory to maintain decent rate-distortion performance. We propose to extend the wavefront processing pattern to the entropy coding to achieve full Wavefront Parallel Processing (WPP) of the LCUs, without requiring the step of buffering the symbols after entropy decoding.

# Sequential LCU processing in HEVC

In HEVC encoding and decoding, the LCUs are processed in raster scan order, with the CABAC entropy codec updating its probabilities on the fly. This is represented on Fig. 1. In order to encode the current LCU, the left, top-left, top, and top-right LCUs need to be available (already decoded) in order to use spatial and motion information for the Intra and Inter prediction. This is the pixel and MV dependency, represented on Fig. 1 by the light grey arrows. On the other hand, when starting the entropy coding of the symbols, the CABAC probabilities used are simply the ones that are available after the previous LCU (left LCU) has been processed. In particular, due to the raster scan order, the first LCU of a line uses the CABAC probabilities available after the processing of the last LCU from the previous line.

Fig. 1 - Illustration of sequential raster scan processing of the LCUs in a frame.

# Wavefront processing of the LCUs (ideally)

Wavefront processing of the LCU is a straightforward approach to try and achieve parallelism for encoding and decoding. Fig. 2 shows an example of wavefront processing of the LCUs with 4 threads. Due to the spatial and MV dependency of HEVC, there must be a shift of 2 LCUs between processed LCUs of adjacent lines.

However, due to the CABAC probabilities dependency of the first LCU of each line (dependent on the processing of the last LCU of the previous line), it is not possible to start threads 2, 3, 4 before the thread processing the upper line has finished, thus preventing efficient parallelism. This is illustrated on Fig. 2.

To solve this problem, it would be possible, for instance, to re-initialize the CABAC probabilities at the beginning of each line of LCUs. This has been proposed in document JCTVC-D073 “Periodic inits for wavefront coding functionality” [2]. However, a large penalty in R/D performance is incurred: (+2.8% measured in JCTVC-D073 with HM0.9 [2], +4.4% in our tests with HM2.0 and entropy slices limited to the number of LCUs per line). This loss is due to the fact that the CABAC probabilities adapted to the specificities of the current image need to be learned again after each re-initialization, and this takes some time (typically, a few LCUs).

Fig. 2 - Illustration of desired wavefront processing of LCUs, unavailable probabilites at the start of each line prevent the threads from starting the encoding/decoding.

# Wavefront Parallel Processing (proposal)

In order to reduce the performance penalty, one needs to avoid losing the probabilities that have already been acquired. To achieve this in a simple way, we propose to initialize the CABAC probabilities of the first LCU of each line with the probabilities obtained after the second LCU of the upper line is processed. This proposal is illustrated on Fig. 4. This allows for a quick learning of the probabilities along the first column of LCUs. Moreover, this can be performed without modifying the wavefront structure, because the second LCU of the upper line is always available, as a consequence of the spatial and MV dependency. To achieve this, a single addition probability buffer is required. When a thread starts to process the first LCU of the line, it reads the probabilities from the buffer. When a thread has finished to process the second LCU of a line, it writes its probabilities into the buffer. Due to the wavefront approach, this guarantees that the probabilities are read and written in the proper line order.

Fig. 3 - Illustration of the proposed method. At the start of each line, the CABAC probabilities are synchronized with the probabilities of the upper right LCU.

# Advantages

It is asserted that the proposed method has the following advantages.

## Fixed encoding and decoding design

The size, shape, and number of parallel processed areas do not change between each frame (as it does when using entropy slices with bins limit termination). We assert that this is useful for H/W design. Also, it allows to guarantee that if N processing units are available, N threads are used on each frame, be the sequence of class D or class A. We assert that the increasing availability of multi-core mobile devices makes parallel encoding and decoding of moderate resolutions a desired feature.

## Works both for encoder and decoder

Since the design is fixed, it is possible to use WPP when decoding, but also when encoding (this is not possible using entropy slices with bin limit termination because the encoder does not know when a thread is terminated). We assert that parallel encoding will also become useful, for instance on multi-core mobile device with high resolution video capture capabilities (smartphones, tablets) and/or for mobile videoconference.

## Parallelizes entropy codec AND pixel codec

In the proposed approach, the full LCU processing is parallelized (entropy processing and pixel processing). In particular, during decoding, there is no need to buffer the decoded symbols on the whole frame before starting the decoding process (entropy slices with bins limit require to buffer symbols of the full frame before decoding process can start).

## Size of “useful” bitstream is independent from number of threads

Using WPP, the binary stream produced by each LCU has the same size for any number of threads, because the encoding uses a predefined structure (then same symbols are encoded with the same probabilities). Therefore the size of the  “useful” part of the bitstream  does not depend on the number of threads. The main difference in size comes from the substream wrapping: substream headers and terminating bits have to be written for each substream. Also, transcoding the bitstream to any number of threads is only an entropy level operation.

## Processes interleaved lines of LCUs

As described by a previous study by Texas Instrument (JCTVC-D243 “Analysis of entropy slice approaches” [3]), we assert that processing interleaved lines of LCUs is good for:

- throughput (because interleaved lines of LCU process similar areas and « cross » the same type of content)

- memory bandwidth (entropy encoder/decoder and pixel encoder/decoder both run in parallel)

# Results

We integrated Wavefront Parallel processing (WPP) in HM2.0 and tested with the following conditions:

- High efficiency (for CABAC use)

- Intra, Low delay, Random access

- JCTVC-D600 [4] conditions

Moreover, since each thread generates its own substream, it is necessary to provide substream pointers in the bitstream, so that the decoder can find the beginning of each bitstream and start decoding immediately. We have implemented a very simple (unoptimized) bitstream pointer mechanism. The penalty incurred for one substream is due to the size of the pointer and the fill-in bits written at the end of the bitstream for byte alignment. We have measured this penalty to be less than 0.1% per subtream. It is certainly possible to further reduce this by a more clever substream pointing mechanism. The performance figures shown here have been produced for 2 substreams.

As shown on table 1, the average loss is 0.7% (0.1% in Intra, 0.8% in Random Access, and 1.3% in Low Delay, mostly due to class E sequence).

Table 1. Summary RD results of WPP versus HM2.0 anchors

# Syntax

We propose to support WPP with the following syntax and semantics

| seq_parameter_set_rbsp( ) { | Descriptor |  |  |
| --- | --- | --- | --- |
| wavefront_parallelism; | u(1) |  |  |
| if( wavefront_coding = = 1 ){ |  |  |  |
| number_of_threads; | u(8) |  |  |
| } |  |  |  |
| ... |  |  |  |
| } |  |  |  |
| slice_header( ) { |  | Descriptor |  |
| ... |  |  |  |
| if( wavefront_parallelism == 1){ |  |  |  |
| for(i=0;i<number_of_threads-2;i++){ |  |  |  |
| substream_length; |  | u(v) |  |
| } |  |  |  |
| } |  |  |  |

The propose syntax modifications take into account up to 256 threads (substreams per slice). The wavefront_parallelism syntax element is set to one if and only if wavefront parallelism is activated. In this case, head slice header contains the substream pointer substream_length to start immediate parallel decoding.

# Conclusion

We propose Wavefront Parallel Processing (WPP) for HEVC video encoding and decoding. The proposed approach combines moderate R/D loss (0.7%) with a number of advantages:

- fixed parallel design,

- applicable to encoding and decoding,

- parallelization of the full LCU processing (memory bandwidth efficiency)

- size of useful bitstream is independent from number of threads

- interleaved lines of LCUs (throughput efficiency)

Moreover, the proposed approach can be viewed as an extension of Entropy Slices with LCU-limit as implemented in HM2.0.

Given the significance of parallelization for HEVC standard, we submit WPP to JCT-VC for adoption into the HM.

# Patent rights declaration(s)

Orange Labs may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- JCTVC-D070, “Lightweight slicing for entropy coding”, 4th JCT-VC Meeting, Daegu, January 2011.

- JCTVC-D073, “Periodic inits for wavefront coding functionality”, 4th JCT-VC Meeting, Daegu, January 2011.

- JCTVC-D243, “Analysis of entropy slice approaches”, 4th JCT-VC Meeting, Daegu, January 2011.

- JCTVC-D600, “Common test conditions and software reference configurations”, 4th JCT-VC Meeting, Daegu, January 2011.

# Appendix: Modification of HEVC Working Draft 2 to include WPP

9.3	CABAC parsing process for slice data

[Ed. (TW) needs to be extended]

This process is invoked when parsing syntax elements with descriptor ae(v) in subclauses 7.3.4 and 7.3.5 when entropy_coding_mode_flag is equal to 1.

Inputs to this process are a request for a value of a syntax element and values of prior parsed syntax elements.

Output of this process is the value of the syntax element.

When starting the parsing of the slice data of a slice in subclause 7.3.4, the initialisation process of the CABAC parsing process is invoked as specified in subclause 9.3.1.

In case syntax element wavefront_parallelism  is equal to one, when  starting the parsing of  a coding tree as specified in subclause 7.3.5,  if  x0 is equal to zero and  y0 is different from zero and  log2CUSize  is equal to log2MaxCUSize   the synchronization process of the CABAC parsing process is invoked as specified in subclause  9.3.x, otherwise if   x0 is equal to log2MaxCUSize+1   zero and  y0 is different from zero and  log2CUSize  is equal to log2MaxCUSize   the memorization  process of the CABAC parsing process is invoked as specified in subclause  9.3.y .

The parsing of syntax elements proceeds as follows:

For each requested value of a syntax element a binarization is derived as described in subclause 9.3.2.

The binarization for the syntax element and the sequence of parsed bins determines the decoding process flow as described in subclause 9.3.3.

For each bin of the binarization of the syntax element, which is indexed by the variable binIdx, a context index ctxIdx is derived as specified in subclause 9.3.3.1.

For each ctxIdx the arithmetic decoding process is invoked as specified in subclause 9.3.3.2.

The resulting sequence ( b0..bbinIdx ) of parsed bins is compared to the set of bin strings given by the binarization process after decoding of each bin. When the sequence matches a bin string in the given set, the corresponding value is assigned to the syntax element.

In case the request for a value of a syntax element is processed for the syntax element mb_type and the decoded value of mb_type is equal to I_PCM, the decoding engine is initialised after the decoding of any pcm_alignment_zero_bit and all pcm_sample_luma and pcm_sample_chroma data as specified in subclause 9.3.1.2.

9.3.x Synchronization process of context variables

Inputs of this process are variables TableStateSync and TableMPSSync contain the values of the variables n  and m  used in the initialisation of context variables that are assigned to all syntax elements in subclauses 7.3.4 and 7.3.5 except for the end-of-slice flag.

Outputs of this process are the initialised CABAC context variables indexed by ctxIdx.

For each context variable, the two variables pStateIdx and valMPS are initialised: given two table entries n,m, pStateIdx is initilialized to n and valMPS is initialized to m.

9.3.y Memorization process of context variables

Inputs of this process are the CABAC context variables indexed by ctxIdx.

Output of this process are variables TableStateSync and TableMPSSync contain the values of the variables n  and m  used in the synchronization process of context variables that are assigned to all syntax elements in subclauses 7.3.4 and 7.3.5 except for the end-of-slice flag.

For each context variable, given two table entries n,m, n is initilialized to pStateIdx n and m is initialized to valMPS.

9.3.1	Initialisation process

Outputs of this process are initialised CABAC internal variables.

The processes in subclauses 9.3.1.1 and 9.3.1.2 are invoked when starting the parsing of the slice data of a slice in subclause 7.3.4.

The process in subclause 9.3.1.2 is also invoked after decoding any pcm_alignment_zero_bit and all pcm_sample_luma and pcm_sample_chroma data for a macroblock of type I_PCM.

9.3.1.1	Initialisation process for context variables

Outputs of this process are the initialised CABAC context variables indexed by ctxIdx.

Tables ABC to DEF contain the values of the variables n and m used in the initialisation of context variables that are assigned to all syntax elements in subclauses 7.3.4 and 7.3.5 except for the end-of-slice flag.

For each context variable, the two variables pStateIdx and valMPS are initialised.

NOTE 1 – The variable pStateIdx corresponds to a probability state index and the variable valMPS corresponds to the value of the most probable symbol as further described in subclause 9.3.3.2.

The two values assigned to pStateIdx and valMPS for the initialisation are derived from SliceQPY, which is derived in Equation [Ed. (TW): insert text]. Given the two table entries ( m, n ), the initialisation is specified by the following pseudo-code process:

preCtxState = Clip3( 1, 126, ( ( m  Clip3( 0, 51, SliceQPY ) ) >> 4 ) + n )

if( preCtxState  <=  63 )  {

pStateIdx = 63 − preCtxState

valMPS = 0	(9 4)

} else {

pStateIdx = preCtxState − 64

valMPS = 1

}