| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E197 WG11 Number: m19715 |
| --- | --- |

| Title: | Comments on the Common Test Conditions for ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ming Li Ping Wu Lei Wang Wen Zhang | Tel: Email: | +86-25-88014641 li.ming42@zte.com.cn ping.wu@zte.com.cn wang.lei121@zte.com.cn zhang.wen4@zte.com.cn |
| Source: | ZTE Corporation |  |  |

_____________________________

# Abstract

In this document, two alternative configurations are proposed to reduce the ALF complexity. The first one is to disable ALF for the frames that are not used as reference for random access (RA) configurations. Since hierarchical QP assignment is adopted in low complexity (LD) cases, the other one is to apply ALF to I-frames and the P/B frames using the smallest QP, that currently equals to QPI + 1. Based on the experimental results, it is suggested to adopt these two configurations to the high efficiency (HE) cases in the common test conditions, i.e. RAHE and low delay (LD) HE (LDHE).

# Introduction

ALF is integrated in HEVC to reduce the quantization noise in the reconstructed pictures, and contributes to the high coding performance achieved by HEVC codec. In the current common test conditions recommended in JCTVC-D600, ALF is enabled for coding every frame when using HE configurations, i.e. RAHE and LDHE. For LC configurations (i.e. RALC, LDLC), ALF is turned off for all frames because of its complexity. To tackle the complexity problem, two configurations for ALF are proposed to reduce the computational burden as

- For RA cases using the dyadic hierarchical prediction structure, ALF is applied to the frames which are employed as reference for coding successive frames.

- For LD cases using hierarchical QP assignment, ALF is applied to I-frames and the P/B frames with the smallest QP (currently QPI + 1).

# Experimental Results

The proposed method was evaluated using the common test conditions described in JCTVC-D600. Two inter configurations (i.e. RAHE, RALC) were employed to generate anchors. The proposed configurations are applied to HM 2.0 codec. The test platform is HP server E5506/2.13 GHz with Windows NT server 2003. The results are summarized in Table 1.

Table 1. Experimental Results

|  |  | Random access |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.6 | -0.1 | -0.1 |
| Class B | 0.4 | -0.1 | -0.1 |
| Class C | 0.2 | -0.1 | -0.1 |
| Class D | 0.3 | -0.1 | -0.1 |
| Class E |  |  |  |
| All | 0.3 | -0.1 | -0.1 |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 69% |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 2.1 | 0.7 | 0.4 |
| Class C | 1.2 | 0.6 | 0.9 |
| Class D | 1.0 | -0.3 | -0.2 |
| Class E | 1.3 | -0.8 | -0.2 |
| All | 1.4 | 0.1 | 0.2 |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 70% |  |  |

From the experimental results, it is observed that the proposed configurations achieve well trade-off between coding efficiency and complexity for RAHE and LDHE.

# Conclusions

In this document, two alternative configurations for ALF are proposed for the inter frame test conditions recommended in JCTVC-D600. Based on the experimental results, it is suggested to adopt the proposed configurations to RAHE and LDHE test conditions for the trade-off between coding efficiency and complexity.

# Patent rights declaration(s)

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).