| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Genève, CH, 16-23 March, 2011 | Document: JCTVC-E198 |
| --- | --- |

| Title: | Sub-LCU level delta QP signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masaaki Kobayashi, Masato Shima 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan | Tel: Email: Email: | +81-3-3758-2111 kobayashi.masaaki517@canon.co.jp shima.masato@canon.co.jp |
| Source: | Canon Inc. |  |  |

_____________________________

# Abstract

This contribution proposes sub-LCU level QP control functionality and alternative delta QP signaling methods.  It is claimed in this contribution that the LCU level (64x64) QP control implemented in current HM is too coarse comparing to the small block level (16x16) QP control adapted by former video coding standards such as AVC.  Accordingly, this contribution supports the concept of sub-LCU level QP control which was already presented in the former contributions, JCTVC-D038/D308.  In addition, this contribution introduces several alternative delta QP calculation methods which can be used with the sub-LCU level QP control.

# Introduction

The concept of CU and LCU has been employed to HEVC.  Especially 64x64 size LCU brings about high coding efficiency.  However, current HM is capable of controlling QP at 64x64 LCU level or slice level while AVC is capable of controlling QP at 16x16 MB level.
In former contributions, JCTVC-D308 presented the necessity of sub-LCU level QP control, while JCTVC-D038 introduced methods to calculate delta QP.

There have already been many AVC-based products such as camcorders and digital still cameras available in the market.  Video coding functionality of such products is capable of controlling QP at 16x16 block level granularity.  Fig. 1-1 shows one frame of 1920x1080 encoded video data recorded by a camcorder currently available in the market with the AVC encoding functionality.  Fig. 1-2 shows the QP variation in one frame presented in Fig. 1-1.  It can be observed from Fig. 1-2 that QP is controlled at 16x16 block level according to the image region characteristics.

Fig. 1-1 One frame of AVC video data recorded by a camcorder available in the market

Fig. 1-2 QP distribution of the frame presented in Fig. 1-1

In general, there are several reasons to control QP at block level.  One of them is to improve visual image quality as already described in JCTVC-D308.  Another reason is to control bitrates for recoding to media or transmitting via network.  In addition, block level QP control functionality is especially important to adjust network transmission delay.  Furthermore, in recent years, object recognition functionality is being adopted for many camera products, and they tend to utilize the object information to control image quality of each object region.  In this way, QP control functionality at small blocks has widely been used, and it is becoming more significant.

In JCTVC- D038, delta QP is calculated as the difference between QP of current CU and QP of previously coded CU.  The larger magnitude of delta QP is, the more bits will be required.  Coded bit size of delta QP is not negligible especially when QP is controlled elaborately for rate control.  Therefore, the method to calculate QP is significant to reduce the amount of coded bits as JCTVC- D038 also asserted.

# Alternative Methods of Calculating Delta QP

JCTVC-D038 introduced several methods to calculate delta QP.  In addition to them, other methods of calculating delta QP should be studied as well because:
(1) The variation of QP is depending on rate control method.
(2) CU structure of the current HEVC design is complex.
In this contribution, three alternative methods are introduced, and those methods should also be examined in the CE along with the delta QP calculating methods already presented.

## Method 1

Delta QP is calculated as the difference between the QP of current CU and the QP of previously coded QP with the same size as the current CU.  It means calculating delta QP at each CU size respectively.  Fig. 2-3 shows an example of possible CU partition structure in current HM design.

The detailed description of calculating delta QP is shown as follows:

(1) If the size of current CU is 64x64,
delta QP is calculated as the difference between the QP of current CU and the QP of previously coded 64x64 CU.

(2) If the size of current CU is 32x32,
delta QP is calculated as the difference between the QP of current CU and the QP of previously coded 32x32 CU.
Example using Fig. 3-1)
-Delta QP of A8 is the difference between QP of A8 and QP of A7.

(3) If size of current CU is 16x16,
delta QP is calculated as the difference between the QP of current CU and the QP of previously coded 16x16 CU.
Example using Fig. 3-1)
	-Delta QP of B2 is the difference between QP of B2 and QP of B1.
	-Delta QP of B3 is the difference between QP of B3 and QP of B2.

(4) If size of current CU is 8x8,
delta QP is calculated as the difference between the QP of current CU and the QP of previously coded 8x8 CU.
Example using Fig. 3-1)
	-Delta QP of C3 is the difference between QP of C3 and QP of C2.
	-Delta QP of C5 is the difference between QP of C5 and QP of C4.

(5) If size of current CU is 4x4,
delta QP is calculated as the difference between the QP of current CU and the QP of previously coded 4x4 CU.

Fig. 2-3 Possible CU partition and its delta QP calculation

## Method 2

Delta QP is calculated as the difference between the QP of current CU and the QP calculated from QPs of left and top CUs with the same "PredMode" as the current CU. "PredMode" is defined in JCTVC-C403, and it indicates whether CU is inter, intra, or other mode.  Fig. 2-4 shows an example of "PredMode" and position of neighboring CU to explain this method.

The detailed description of calculating delta QP is shown as follows:

(1) If "PredMode" of current CU is the same as "PredMode" of left CU but not the same as “PredMode” of top CU,
	delta QP is calculated as the difference between the QP of current CU and the QP of left CU.

(2) If "PredMode" of current CU is the same as "PredMode" of top CU but not the same as “PredMode” of left CU,
	delta QP is calculated as the difference between the QP of current CU and the QP of top CU.

(3) If "PredMode" of left CU and top CU are same and "PredMode" of current CU is same as it,
	delta QP is calculated as the difference between the QP of current CU and the average of QPs of left CU and top CU.

(4) If "PredMode" of current CU is different from "PredMode" of left CU or top CU.
	delta QP is calculated as the difference between the QP of current CU and the QP of left CU.

Fig. 2-4 Delta QP calculation by PredMode

## Method 3

Delta QP is calculated as the difference between the QP of current CU and the QP of one of the neighboring CUs indicated by IntraPredMode.  Fig. 2-3 shows position of neighboring CUs to be referenced for calculating delta QP.  The calculating method and neighboring CU are specified by the value of IntraPredMode shown in Fig. 2-6.  Fig. 2-6 shows intra prediction direction indicated by IntraPredMode.  This method is applicable when "PredMode" is intra. If "PredMode" is not intra, delta QP is calculated as the difference between the QP of current CU and the QP of left CU.

The detailed description of calculating delta QP is shown as follows:

(1) If IntraPredMode is 9, 33, 17, 32, 8, 31, 16, 30, 1, 29, 15, 28, 7, or 2,
	delta QP is calculated as the difference between the QP of current CU and the QP of left CU.

(2) If IntraPredMode is 27, 14, 26, or 3,
	delta QP is calculated as the difference between the QP of current CU and the average of QPs of left CU and top_left CU.

(3) If IntraPredMode is 18, 10, or 19,
	delta QP is calculated as the difference between the QP of current CU and the average of QPs of top_left CU and top CU.

(4) If IntraPredMode is 4, 20, 11, 21, 0, 22, 12, 23, or 5
	delta QP is calculated as the difference between the QP of current CU and the QP of top CU.

(5) If IntraPredMode is 24, 13, 25, or 6
	delta QP is calculated as the difference between the QP of current CU and the average of QPs of top CU and top_right CU.

(6) If IntraPredMode is 2,
	delta QP is calculated as the difference between the QP of current CU and the average of QPs of top CU and left CU.

Fig. 2-5 Delta QP calculation by intra prediction

Fig. 2-6  Intra prediction direction indicated by IntraPredMode

# Conclusion

Current AVC-based products widely deployed in the market are capable of controlling QP at 16x16 block level. In the current HEVC design, however, is not capable of controlling QP at sub-LCU level. HEVC should also provide fine-granularity QP control functionality which has been practically utilized by current video coding products available in the market.
In addition, practical use of rate control and its effect on delta QP coding need to be studied further under the condition of current CU structure.  Hence, it is recommended to establish a CE to study such items.

# Patent rights declaration(s)

Canon Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).