| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E203 |
| --- | --- |

| Title: | Constrained intra prediction scheme for flexible-sized prediction units in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Viktor Wahadaniah ChongSoon Lim Sue Mon Thet Naing Block 1022 Tai Seng Avenue #06-3530 Singapore 534415 | Tel: Email: | +65 6550 5415 viktor.wahadaniah@sg.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution presents the HM-2.0-based experimental results of the constrained intra prediction scheme initially proposed in JCTVC-D094. By adapting the constrained intra prediction scheme to HEVC characteristics, the proposed scheme is reported to give average BD-rate gains of  0.2% (Y),  0.3% (U),  0.3% (V) for random access, high efficiency setting,  0.3% (Y),  0.5% (U),  0.6% (V) for random access, low complexity setting,  0.1% (Y),  0.1% (U), 0.2% (V) for low delay, high efficiency setting, and  0.1% (Y),  0.2% (U),  0.1% (V) for low delay, low complexity setting, as compared to the AVC-style constrained intra prediction scheme adopted in HM 2.0 software. It was reported that the encoding and decoding times of the proposed scheme are the same as the AVC-style scheme. It is suggested to adopt this proposal into HM 3.0. It is also recommended to investigate constrained intra prediction design for other intra prediction schemes besides directional intra prediction scheme.

# Introduction

Constrained intra prediction has been shown to be beneficial for error resilience especially in reducing error propagation from spatially-predicted intra-coded samples ([1], [2]). Constrained intra prediction restricts the intra prediction process for a target prediction unit to use only the intra-coded neighboring samples. H.264/MPEG-4 AVC features a constrained intra prediction scheme in which specific intra prediction modes are disabled when corresponding reference samples are not intra-predicted. An adaptation of AVC-style constrained intra prediction scheme into HEVC is presented in JCTVC-D086 ([2]). A modified constrained intra prediction scheme adapted to flexible-sized prediction units in HEVC is presented in JCTVC-D094 ([3]).

This contribution presents the experimental results of the constrained intra prediction scheme of JCTVC-D094 based on implementation onto HM 2.0 software. Based on these results, this contribution recommends adoption of the proposed constrained intra prediction scheme into HM 3.0. Section 2 reviews the constrained intra prediction scheme presented in JCTVC-D094. Sections 3 and 4 present the experimental setting and results. Conclusion and recommendation is presented in Section 5.

# Constrained intra prediction for flexible-sized prediction units

As shown in [2], constrained intra prediction is useful in reducing visual artifacts caused by spatial error drifts. As a consequence of the reduction in available reference samples and intra prediction modes, it is reported in [2] that AVC-style constrained intra prediction scheme introduces coding efficiency loss on TMuC 0.9 in the form of average luma BD-rate drops of 1.4% to 2.1%.

H.264 constrained intra prediction requires every reference sample (except from top-right neighboring locations) to be intra-predicted. Such a requirement is appropriate for AVC, where intra/inter prediction type is specified for each 16x16 macroblock unit. In contrast, intra/inter prediction type in HEVC is specified for flexible-sized prediction units whose sizes range from 4x4 to 64x64. As such, there are instances where some intra prediction reference samples are intra-coded while others are inter-coded, as shown in Figure 1 below.

Figure 1 - Example of H.264-style constrained intra prediction in an inter picture

When AVC-style constrained intra prediction scheme is used for the above example, all intra prediction modes except DC mode is disabled. Even for the DC intra prediction mode, the partially available intra-predicted reference samples (green colored samples in Figure 1) are not used in calculating prediction sample values.

The above example suggests that it may be beneficial to use the partially available intra-predicted reference samples in a constrained intra prediction scheme for HEVC. One such constrained intra prediction scheme is proposed in this document. The proposed scheme substitutes the reference sample values at inter-predicted sample locations with values derived from the reference sample values at nearby intra-predicted sample locations. After the substitution, the inter-predicted reference sample locations are considered to be available for use as intra prediction reference samples. As a result, all intra prediction modes remain available as candidate modes and an encoder still has the freedom to select a most efficient mode using a decision scheme such as the rate-distortion optimized scheme.

For the example in Figure 1, the proposed scheme substitutes the reference samples at inter-prediction sample locations as follows:

- p[ -1, 23 ] to p[ -1, 8 ] are substituted with the value (p[ -1, 24 ] + p[ -1, 7 ] + 1) >> 1).

- p[ -1, -1 ] to p[ 11, -1 ] are substituted with the value (p[ -1, 0 ] + p[ 12, -1 ] + 1) >> 1).

- p[ 16, -1 ] to p[ 31, -1 ] are substituted with the value p[ 15, -1 ].

# Experimental setting

The proposed constrained intra prediction scheme was implemented into HM 2.0 software (branch version HM-2.0-dev-nec revision 597). In the current implementation, substitution of left and top-left reference samples using top reference samples are not performed when left and top-left reference samples are not available (for example, located outside picture edge). Similarly, substitution of top and top-left reference samples using left reference samples are not performed when top and top-left reference samples are not available. The 64-bit encoder and decoder executable binaries were generated using Microsoft Visual C++ 2010.

Simulations were conducted on all six test conditions as specified in JCTVC-D600 ([5]), namely:

- Intra, High Efficiency

- Intra, Low Complexity

- Random Access, High Efficiency

- Random Access, Low Complexity

- Low Delay, High Efficiency

- Low Delay, Low Complexity

Full test sequences and quantization parameter values as specified in [5] were used. Three sets of simulations were conducted on the full test set:

- HM 2.0 anchor with constrained intra prediction disabled.

- AVC-style constrained intra prediction scheme

- Proposed constrained intra prediction scheme.

# Experimental results

Experimental results are shown in Table 1, Table 2 and Figure 2. It can be observed from the results that the proposed scheme produces higher coding efficiency compared to AVC-style constrained intra prediction scheme over all test settings in general. In addition, the encoding and decoding times for random access and low delay settings suggest that the proposed scheme has negligible effect on the computational complexity.

It is noted that in intra settings, the decoding times of the proposed scheme is longer than AVC-style scheme. This is due to the fact that the current implementation performs the same reference sample checking and substitution steps for both intra and inter pictures. In practice, reference sample checking and substitution steps related to intra vs. non-intra prediction modes can be skipped in intra pictures, as all prediction samples are intra-predicted. Therefore, the decoding time increase of the proposed scheme would become immaterial.

Table 1 - Coding efficiency of AVC-style constrained intra prediction compared to HM 2.0 anchor with constrained intra prediction disabled

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 101% |  |  | 98% |  |  |
| Dec Time[%] | 101% |  |  | 104% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 1.63 | 3.40 | 3.17 | 2.43 | 2.72 | 3.45 |
| Class B | 1.95 | 3.87 | 3.79 | 2.59 | 4.04 | 3.64 |
| Class C | 3.07 | 5.26 | 5.35 | 3.62 | 5.05 | 5.19 |
| Class D | 2.38 | 4.25 | 4.16 | 2.75 | 3.37 | 3.78 |
| Class E |  |  |  |  |  |  |
| All | 2.24 | 4.17 | 4.10 | 2.83 | 3.81 | 3.99 |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 2.08 | 6.30 | 6.94 | 3.04 | 9.20 | 9.82 |
| Class C | 3.40 | 8.36 | 8.73 | 4.39 | 9.53 | 10.76 |
| Class D | 2.23 | 5.87 | 6.27 | 3.03 | 7.24 | 7.34 |
| Class E | 0.98 | 4.28 | 4.07 | 1.45 | 6.52 | 5.73 |
| All | 2.24 | 6.33 | 6.68 | 3.08 | 8.29 | 8.67 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 97% |  |  | 94% |  |  |

Table 2 - Coding efficiency of proposed constrained intra prediction scheme compared to AVC-style constrained intra prediction scheme

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 105% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.2 | -0.1 | -0.1 | -0.2 | -0.7 |
| Class B | -0.2 | -0.3 | -0.3 | -0.3 | -0.5 | -0.5 |
| Class C | -0.3 | -0.4 | -0.5 | -0.5 | -0.7 | -0.9 |
| Class D | -0.2 | -0.3 | -0.2 | -0.4 | -0.6 | -0.6 |
| Class E |  |  |  |  |  |  |
| All | -0.2 | -0.3 | -0.3 | -0.3 | -0.5 | -0.6 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | -0.1 | -0.2 | -0.1 | -0.3 | -0.2 |
| Class C | -0.1 | -0.1 | -0.1 | -0.2 | -0.3 | -0.3 |
| Class D | 0.0 | 0.2 | -0.6 | -0.1 | -0.3 | 0.1 |
| Class E | -0.1 | -0.7 | -0.1 | -0.1 | 0.3 | 0.0 |
| All | -0.1 | -0.1 | -0.2 | -0.1 | -0.2 | -0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Figure 2 - Per-sequence coding efficiency of proposed constrained intra prediction scheme compared to AVC-style constrained intra prediction scheme

# Conclusions

This contribution provides HM-2.0-based experimental results for a proposed constrained intra prediction scheme adapted to flexible-sized prediction units in HEVC video coding scheme. Results show that the proposed scheme provides per-sequence BD-rate gain of up to 1.0% (luma component) and 2.0% (chroma component) as compared to AVC-style constrained intra prediction scheme, without increasing the encoding and decoding times. It can be concluded that the proposed scheme is better-adapted to HEVC characteristics than AVC-style constrained intra prediction. Therefore, we would firstly propose to adopt the proposed scheme into HM 3.0. Secondly, we would also recommend investigation of constrained intra prediction design for other intra prediction schemes besides directional intra prediction scheme.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- T. Wiegand, G.J. Sullivan, G. Bjontegaard, A. Luthra, "Overview of the H.264/AVC video coding standard," IEEE Transactions on Circuits and Systems for Video Technology, Vol. 13 Issue 7, pp. 560 - 576, July 2003.

- H. Aoki, K. Chono, X. Jing, V. Wahadaniah, C.S. Lim, S.M.T. Naing, "Constrained intra prediction for reducing visual artifacts caused by lossy decoder-side memory compression," JCT-VC document JCTVC-D086, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 4th meeting, Daegu, KR, January 2011.

- V. Wahadaniah, C.S. Lim, S.M.T. Naing, "Constrained Intra Prediction Scheme for Flexible-Sized Prediction Units in HEVC," JCT-VC document JCTVC-D094, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 4th meeting, Daegu, KR, January 2011.

- T. Wiegand, W.J. Han, B. Bross, J.R. Ohm, G.J. Sullivan, "WD2: Working Draft 2 of High-Efficiency Video Coding," JCT-VC document JCTVC-C503, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 4th meeting, Daegu, KR, January 2011.

- F. Bossen, "Common test conditions and software reference configurations," JCT-VC document JCTVC-C600, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 4th meeting, Daegu, KR, January 2011.

# Appendix: Proposed text changes for Working Draft

A portion of the text from subclause 8.3.3.1 of Working Draft 2 (JCTVC-D503 [4]) is replaced as follows. Original text:

When any samples p[ x, -1 ], with x = 0..nS-1 are marked as “not available for intra prediction,” samples p[ x, -1 ], with x = 0..nS-1 are marked as “not available for intra prediction.”

When any samples p[ -1,  y ], with y = 0..nS-1 are marked as “not available for intra prediction,” samples p[ -1,  y ], with y = 0.. nS-1 are marked as “not available for intra prediction.”

When any samples p[ x, -1 ], with x = nS..nS*2-1 are marked as “not available for intra prediction” and the sample p[ nS-1, -1 ] is marked as “available for intra prediction” the sample value of p[ nS-1, -1 ] is substituted for samples p[ x, -1 ], with x = nS..nS*2-1 and samples p[ x, -1 ], with x = nS..nS*2-1 are marked as “available for intra prediction”.

When any samples p[ -1, y ], with y = nS..nS*2-1 are marked as “not available for intra prediction” and the sample p[ -1, nS-1 ] is marked as “available for intra prediction” the sample value of p[ -1, nS-1 ] is substituted for samples p[ -1, y ], with y = nS..nS*2-1 and samples p[ -1, y ], with y = nS..nS*2-1 are marked as “available for intra prediction”.

For all remaining samples p[ x, y ] marked as “not available for intra prediction,” the following applies.

P[ x, y ] = 1 << ( ( BitDepthY + increased_bit_depth_luma ) – 1 )	(8-16)

Modified text in red:

The reference sample substitution process is performed according the following ordered steps:

- The following steps are performed for each sample p0[ x0, y0 ] with x0 = -1, y0 = -1..nS*2-1 and x0 = 0..nS*2-1, y0 = -1 not marked as "available for intra prediction":

- Two nearest samples p1 and p2 marked as “available for intra prediction” are searched as follows:

  - If x0 = -1 and y0 = -1,

  - If the treeblock address tbAddrN corresponding to p[ -1, -1] is not available, p1 and p2 are considered as not present.

  - Otherwise, the nearest sample p1 marked as "available for intra prediction" is searched among the samples p[ -1, y1 ] sequentially from y1 = 0 to y1 = nS*2-1, and the nearest sample p2 marked as "available for intra prediction" is searched among the samples p[ x2, -1 ] sequentially from x2 = 0 to x2 = nS*2-1.

  - Otherwise, if x0 = 0..nS*2-1 and y0 = -1,

  - The nearest sample p1 marked as "available for intra prediction" is searched among the samples p[ x1, -1 ] sequentially from x1 = x0-1 to x1 = 0. When p1 is not present among all p[ x1, -1 ] with x = x0-1..0 and the treeblock address tbAddrN corresponding to p[ -1, -1] is available, p1 is searched among the samples p[ -1, y1 ] sequentially from y1 = -1 to y1 = nS*2-1.

  - The nearest sample p2 marked as "available for intra prediction" is searched among the samples p[ x2, -1 ] sequentially from x2 = x0+1 to x2 = nS*2-1.

  - Otherwise (if x0 = -1 and y0 = 0..nS*2-1),

  - The nearest sample p1 marked as "available for intra prediction" is searched among the samples p[ -1, y1 ] sequentially from y1 = y0+1 to y1 = nS*2-1.

  - The nearest sample p2 marked as "available for intra prediction" is searched among the samples p[ -1, y2 ] sequentially from y2 = y0-1 to y2 = 0. When p2 is not present among all p[ -1, y2 ] with y = y0-1..0 and the treeblock address tbAddrN corresponding to p[ -1, -1 ] is available, p2 is searched among the samples p[ x2, -1 ] sequentially from x2 = -1 to x2 = nS*2-1.

- The sample p0[ x0, y0 ] is modified as follows:

  - When both p1 and p2 are present, the sample p0[ x0, y0 ] is marked as "to be marked as available for intra prediction" and the following applies:

p0[ x0, y0 ] = ( ( p1 + p2 + 1 ) >> 1)		(8-16)

  - Otherwise, when p1 is present and p2 is not present, p0[ x0, y0 ] is marked as "to be marked as available for intra prediction" and the value of p1 is substituted for the value of p0[ x0, y0 ].

  - Otherwise, when p1 is not present and p2 is present, p0[ x0, y0 ] is marked as "to be marked as available for intra prediction" and the value of p2 is substituted for the value of p0[ x0, y0 ].

  - Otherwise (when both p1[ x1, y1 ] and p2[ x2, y2 ] are not present), the following applies:

p0[ x0, y0 ] = 1 << ( ( BitDepthY + increased_bit_depth_luma ) – 1 )	(8-17)

- All samples p[ x, y ] marked as “to be marked as available for intra prediction” are marked as “available for intra prediction”.