| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E204 WG11 Number: m19722 |
| --- | --- |

| Title: | CE9: Results of Tests B, C, D and T |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Core Experiment Result Report |  |  |
| Author(s) or Contact(s): | B. Bross D. Marpe H. Schwarz T. Wiegand Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49 30 31002 622 benjamin.bross@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports test results of core experiment 9 evaluating motion vector derivation and skip tools. All four tested variants are using merge skip instead of AMVP skip. Test T achieves average BD rates from -2% for low delay high efficiency (LD HE) to -1.1% for low delay low complexity (LD LC) compared to the HM2.0 anchor with AMVP skip. For all tests, the encoder runtimes are ranging from 97% to 103% of the anchor encoder runtimes.

# Tested Proposals

## Test B: HM2.0 with Merge Skip

In HM2.0, the skip mode uses the AMVP method to derive motion information which is slice dependent. For P slices, a motion vector predictor (MVP) index is signaled for list0 prediction. For B slices, a MVP index is signaled for list0 prediction and a MVP index is signaled for list1 prediction. The reference index is always set to 0, the motion vector(s) is(are) equal to the MVP(s) and no residual is transmitted.

In test B, the merge method is used to derive motion information. A merge index is signaled. The motion information is fully derived from the neighboring block indexed by the merge index.

## Test C: HM2.0 with Merge Skip using HM2.0 AMVP Candidate Positions

In HM2.0, the merge method uses the motion information of neighboring blocks. The spatial neighboring blocks are located above, left, top left and below left of the current block.

In test C, the locations of the spatial neighboring blocks for the merge method are the same as for the AMVP method which are described in JCTVC-D231. Furthermore the AMVP based skip mode is replaced by the merge skip mode as described in test B.

## Test D: HM2.0 with Merge Skip using Merge Candidate Positions

In test D, the locations of the spatial neighboring blocks for the AMVP method are the same as for the merge method. Furthermore the AMVP based skip mode is replaced by the merge skip mode as described in test B.

## Test T: HM2.0 with Merge Skip using JCTVC-D125 and JCTVC-D274 improvements

In test T, the following was tested:

The spatial candidates used in inter AMVP were modified according to:

JCTVC-D125 section 2.3 as described in JCTVC-D609 [1]

The temporal motion vector predictor used in inter merge and inter AMVP was modified according to:

JCTVC-D125 section 2.1 (1 T MVP*) as described in JCTVC-D609 [1]

JCTVC-D274 section 3’ as described in JCTVC-D609 [1]

Furthermore the AMVP based skip mode is replaced by the merge skip mode as described in test B.

# Merge Skip related Changes in WD2

## Syntax

In 7.3.7	Prediction unit syntax:

WD2:

| prediction_unit( x0, y0, log2PUWidth, log2PUHeight, PartIdx , InferredMergeFlag ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( NumMVPCand( L0 ) > 1 ) |  |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if( NumMVPCand( L1 ) > 1 ) |  |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |

Suggested change:

| prediction_unit( x0, y0, log2PUWidth, log2PUHeight, PartIdx , InferredMergeFlag ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( NumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |

## Decoding Process

In 8.4.2.1	Derivation process for motion vector components and reference indices:

WD2:

-	If PredMode is equal to MODE_SKIP and slice_type is equal to P, the derivation process for luma motion vectors for skipped prediction units in P slices in subclause 8.4.2.1.1 is invoked with the luma location ( xP, yP ), variables nPSW and , nPSH as inputs and the output being the luma motion vector mvL0 and reference index refIdxL0, and the prediction list utilization flag predFlagL0 is set equal to 1. Both components of mvL1 are set to 0, refIdxL1 is set equal to -1 and predFlagL1 is set equal to 0.

-	Otherwise, if PredMode is equal to MODE_SKIP and slice_type is equal to B, the derivation process for luma motion vectors for skipped prediction units in B slices in subclause 8.4.2.1.2 is invoked with the luma location ( xP, yP ), variables nPSW and nPSH as inputs and the output being the luma motion vectors mvL0, mvL1, the reference indices refIdxL0, refIdxL1, and the prediction list utilization flags predFlagL0 and predFlagL1.

Suggested change:

-	If PredMode is equal to MODE_SKIP, the derivation process for luma motion vectors for merge mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xP, yP ), variables nPSW and , nPSH and the partition index PartIdx as inputs and the outputs being the luma motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, the prediction utilization flags predFlagL0 and predFlagL1.

The sections

8.4.2.1.1	Derivation process for luma motion vectors for skipped prediction units in P slices

8.4.2.1.2	Derivation process for luma motion vectors for skipped prediction units in B slices

can be deleted.

# Simulation Environment and Results

## Software and Settings

The presented proposals have been integrated into HEVC HM 2.0 with default macro configuration[2]. The binaries were compiled on a 64 bit Ubuntu 10.10 Linux using the included make files and gcc 4.4.5. The coding conditions used for the simulations are specified in [3]. These default HM2.0 settings, both for configuration file and compile-time, are used in this test as anchor. The following macros have been added:

MTK_AMVP_TEMPORAL_MVP			// CE9 test B:0 C:0 D:0 T:1

MTK_AMVP_SMVP_DERIVATION			// CE9 test B:0 C:0 D:0 T:1

MTK_MERG_TEMPORAL_MVP			// CE9 test B:0 C:0 D:0 T:1

MTK_TMVP_DERIVATION			// CE9 test B:0 C:0 D:0 T:0

MTK_MERG_TCTR_DERIVATION			// CE9 test B:0 C:0 D:0 T:0

MTK_AMVP_TCTR_DERIVATION			// CE9 test B:0 C:0 D:0 T:0

MTK_TMVP_DERIVATION_TCTR_AFTER_TH	// CE9 test B:0 C:0 D:0 T:1

PANASONIC_AMVPTEMPORALMOD		// CE9 test B:0 C:0 D:0 T:1

PANASONIC_AMVPADAREFIDX			// CE9 test B:0 C:0 D:0 T:0

HHI_MRG_SKIP		// CE9 test B:1 C:1 D:1 T:0 - replaces the AMVP based skip by merge based skip

HHI_MRG_CAND_FOR_AMVP	// CE9 test B:0 C:0 D:1 T:0 - use merge candidates for inter amvp and skip amvp

HHI_MRG_AMVP_HM2_0_CAND 	// CE9 test B:0 C:1 D:0 T:0 - use AMVP HM2.0 candidates for inter merge and skip merge

## Hardware

Three different types of computers have been used to run the simulations  all running a 64 bit Ubuntu 10.10 Linux operating system. They are listed in Table 2-1.

Table 2-1	 Computers used to run the simulations.

| Type | Purpose | Cores | CPU | RAM |
| --- | --- | --- | --- | --- |
| 1 | Decoding on 1 core without yuv | 1x4 | 4 x Intel(R) Core(TM)2 Extreme CPU Q9300 @ 2.53GHz | 4 GB |
| 2 | Encoding for test C and HM2.0 anchor to be compared with | 2x6 | 2 x 6 x Intel(R) Xeon(R) CPU X5670 @ 2.93GHz | 48 GB |
| 3 | Encoding for test B, D, T and HM2.0 anchor to be compared with | 2x4 | 2 x 4 x Intel(R) Xeon(R) CPU X5570 @ 2.93GHz | 32 GB |

## Results

The coding efficiency results in this section are given in terms of BD-rate. The values are averaged over all sequences from one class. BD-PSNR, PSNR values and bit rates for every test point can be found in the attached Excel spreadsheets JCTVC-E204-TestB.xls, JCTVC-E204-TestC.xls, JCTVC-E204-TestD.xls and JCTVC-E204-TestT.xls.

Table 2-2	Test B: HM2.0 with Merge Skip

Table 2-3	 Test C: HM2.0 with Merge Skip using HM2.0 AMVP Candidate Positions

Table 2-4	Test D: HM2.0 with Merge Skip using Merge Candidate Positions

Table 2-5	Test T: HM2.0 with Merge Skip using JCTVC-D125 and JCTVC-D274 improvements

# References

- Joel Jung, Benjamin Bross, Peisong Chen, Woo-Jin Han, “Description of Core Experiment 9 (CE9): MV Coding and Skip/Merge operations”, JCTVC-D609, Daegu, January, 2011.

- JCT-VC, “HEVC Test Model Software 2.0”, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/2.0

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-D600, Daegu, January, 2011.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).