| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E206 |
| --- | --- |

| Title: | In-loop filter based on non-local means filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masaaki Mtsumura Yukihiro Bandoh Seishi Takamura Hirohisa Jozawa 1-1, Hikari-no-oka, Yokosuka-shi, Kanagawa 239-0847, JAPAN | Tel: Email: | +81-46-859-3521 matsumura.masaaki@lab.ntt.co.jp |
| Source: | NTT (Nippon Telegraph and Telephone) Corporation |  |  |

_____________________________

# Abstract

This contribution reports the performance of a technique that utilizes an adaptive denosing filter as the in-loop filter of HM codec. In the proposed method, a denoising filter called non-local mean (NLM) filter is added in a deblocking filer process of the encoder/decoder in addition to original deblocking filter of HM2.0. The proposal was implemented in HM2.0-r598 software (for High Efficiency) to evaluate its performance. Compared to the anchor of HM2.0-r598, the average BD-rate gains were “intra (Y:-1.6%, U:-1.4% and V:-1.8%)”, “lowdelay (Y:-2.9%, U:-2.5% and V:-3.3%)” and “random access (Y:-1.9%, U:-2.8% and V:-2.8%)”, respectively. The maximum gain was about "lowdelay (Y:-4.1%, U:-8.8% and V:-8.9%)" for the sequence “BasketballDrill”.

# Introduction

The in-loop filter that supports to reduce the distortion of local decoded picture is one of important coding tools for improving coding efficiency of HM. HM2.0 adopts two kinds of in-loop filter; de-blocking filter and Wiener-based filter. Current deblocking filter is a kind of local smoothing operation which refers pixels located near each target pixel and generates filtered pixel as its output. The locality of the operation restricts the performance of the filtering. Such local operation is a kind of restriction of its filter design. This contribution proposes a denosing operation based on non-local mean (NLM) filter [1] as an in-loop filter operation. The proposed filter compensates the shortcoming current in-loop filter based on the local operations, because of the characteristic of its non-local operation. As a result, coding efficiency can be improved by introducing the proposed filter.

# Algorithm description

## Encoding algorithm

This subsection describes the algorithm of the proposed adaptive filter that is used in the previous step of adaptive loop filter (ALF) process of HM2.0. Firstly, an encoder generates two kinds of filtered pictures, one is generated by the deblocking filter of HM2.0 and the other is generated by NLM filter. Secondly, the encoder segments a frame into blocks, and selects which filtered picture is more suitable for each block. The selected filter is indicated using filter mode information (filter mode 0: NLM filter, filter mode1: deblocking filter). The quad-tree structure is used to represent the segmentation information. The quad-tree structure and filter mode information allocated to the inner node and the leaf node of the quad-tree are determined based on RD cost minimization. Fig.1(A) illustrates a block segmentation in a quad-tree structure, where red-colored regions and green-colored ones indicate blocks associated with “filter mode 0” and those associated with “filter mode 1”, respectively. Thirdly, based on the results of filter mode selection, the following adaptive process is performed to the local decoded picture. Pixels within a block associated with “filter mode 0” in local decoded picture are replaced with those generated by NLM filter. Finally, the deblocking filter is applied to the output of the above process.

The extra overhead of this algorithm is the information of the quad-tree structure, filter modes of each block and a denoising parameter controls the strength of NLM filter.

Fig. 1: Pocessing flow for encoder.

## Decoding algorithm

This subsection describes the decoding algorithm corresponding to the Encoding algorithm described in section 2.1. After decoding the information of the quad-tree structure, filter modes of each block and a denoising parameter controls the strength of NLM filter, the NLM filter is applied to the blocks of “filter mode 0”, and generates a picture Fig.2(A'). Then, decoder applies the deblocking filter of HM2.0 to the picture Fig.2(A'). This adaptive filter is used in the previous step of ALF process of HM2.0. The output of this adaptive filter becomes the input signal for ALF process.

Fig. 2: Processing flow for decoder.

## Non-local means filter (NLM filter)

This subsection provides the overview of the NLM filter [1] that is used in the proposed method. The NLM filter is a non-linear operation based on a similarity between pixel neighborhoods. The basic idea of the NLM filter is that pixel neighborhoods with high similarity provide large weight for filtering.

Fig.3 illustrates basic algorithm of NLM filter. When a pixel at the position of P is the target pixel of the NLM filter, encoder/decoder sets the square neighborhood (red region in the Fig.3) of target pixel and a reference region (the blue region in Fig.3) for computing the weight factor of the filter. NLM filter computes the similarity between the square neighborhood (red region in the Fig.3) of target pixel and the square neighborhood (ex. green region in the Fig.3) in the reference region, in terms of sum of square difference. Using the similarity, NLM filter computes weight factor for the square neighborhood in the reference region. The weighting summation based on the weight factor is the output of the NLM filter.

Fig. 3: Example of target pixel and reference region of NLM filter.

# Experimental results

The proposal has been implemented in HM 2.0-r598. The configuration is subject to the common test condition [2].

## Objective evaluation

Table below presents BD-Rate [3] and encoder/decoder complexities compared to HM2.0-r598 anchors.

Table 1. Experimental results for NLM filter compared to HM2.0-r598

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.7 | -1.0 | -0.9 |
| Class B | -1.3 | -1.3 | -2.0 |
| Class C | -2.1 | -2.6 | -3.0 |
| Class D | -1.3 | -2.1 | -3.1 |
| Class E | -1.8 | 0.6 | 0.5 |
| All | -1.6 | -1.4 | -1.8 |
| Enc Time[%] | 145% |  |  |
| Dec Time[%] (single-thread) | 152% |  |  |
| Dec Time[%] (multi-thread) | 126% |  |  |
|  | Random access |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.6 | -2.0 | -1.9 |
| Class B | -2.1 | -2.6 | -2.1 |
| Class C | -2.3 | -4.1 | -4.0 |
| Class D | -1.5 | -2.5 | -3.2 |
| Class E |  |  |  |
| All | -1.9 | -2.8 | -2.8 |
| Enc Time[%] | 118% |  |  |
| Dec Time[%] (single-thread) | 124% |  |  |
| Dec Time[%] (multi-thread) | 114% |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -2.3 | -2.5 | -2.9 |
| Class C | -3.1 | -4.8 | -5.3 |
| Class D | -2.7 | -2.1 | -2.8 |
| Class E | -4.1 | 0.1 | -1.8 |
| All | -2.9 | -2.5 | -3.3 |
| Enc Time[%] | 112% |  |  |
| Dec Time[%] (single-thread) | 131% |  |  |
| Dec Time[%] (multi-thread) | 117% |  |  |

In the case of intra slices, the encoder often employs NLM filter (filter mode 0) in a lot of blocks on the picture, for recovering the distortion of a picture. Therefore, the average decoding time shows worse than inter slices.

NLM filter is suitable for parallel processing by multi-core CPU and vector operations such as SSE (Streaming SIMD Extensions) and AVX (Advanced Vector Extensions). As a optional information, average decoding time of multi-thread processing (quad-core Intel i7 HT off) is given in table 1. As shown in Table 1, the decoding time for NLM filter can be reduced by about 46% on average, compared to single-thread processing.

## Subjective evaluation

A: Original

B: HM2.0-r598 anchor

C: Proposed

Fig. 4: Subjective evaluation - BasketballDrill, Lowdelay HE, QP 37, frame #499 -

Fig. 4 shows the subjective quality comparison between HM anchor and the proposed method. From the viewpoint of subjective quality, the proposed method provides an advantage over HM anchor as follows:

- Excessive smoothing is avoided (esp. green lines).

- Mosquito noise is reduced around marked edges.

# Conclusion

In this contribution, an in-loop filter based on non-local means filter was proposed. Compared to the anchor of HM2.0-r598, the average BD-rate gains were “intra (Y:-1.6%, U:-1.4% and V:-1.8%)”, “lowdelay (Y:-2.9%, U:-2.5% and V:-3.3%)” and “random access (Y:-1.9%, U:-2.8% and V:-2.8%)”, respectively. Speed-up that avoids deterioration will be important future work of this technique. Recommend the proposed scheme to be studied within CE8 to find the optimal combination of current in-loop filter.

# References

- A. Buades, B. Coll, and J. M. Morel, “A Non Local Algorithm for Image Denoising,” Proc. IEEE Int. Conf. on Computer Vision and Pattern Recognition, vol. 2, pp. 60-65, June 2005.

- F. Bossen, “Common test conditions and software reference configurations,” JCTVC-D600, 4th. JCT-VC Meeting, Daegu, KR, 20-28 January, 2011.

- G. Bjontegaard, “Calculation of average PSNR differences between RD-curves,” ITU-T SG16 Q.6 Document, VCEG-M33, 13th VCEG Meeting, Austin, Texas, USA, April 2001.

# Patent rights declaration(s)

NTT Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).