| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E207 |
| --- | --- |

| Title: | CE6.b.2: Cross check report of BBC's proposal (JCTVC-E130) from Toshiba |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Akiyuki Tanizawa Taichiro Shiodera 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, JAPAN | Tel: Email: | +81-44-549-2152 akiyuki.tanizawa@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This contribution contains cross-checking results of the software containing the tool of JCTVC-D191 proposed by BBC on Core Experiment 6 subset b of intra prediction improvement. In this contribution, the simulation results for combined intra scheme with parallel intra prediction scheme proposed in JCTVC-D074 are reported and cross-verified.

The verification task has been done successfully and the results perfectly match those proposed in JCTVC-E130.

# Cross-checking

In this contribution, the simulation results for combined intra scheme (CIP) proposed in JCTVC-E130 with parallel intra prediction scheme (PIP) are reported and cross-verified.

The software based on HM version 2.0 and the experimental results were provided on March 9 2011 by BBC. The software was complied fine and the cross-checking was performed on 64bit environments. The simulation has been conducted under CE common test conditions [1], [2] for both high efficiency and low complexity conditions in the case of I-slice only.

## Configurations

The proponents require two test cases in the following;

- This setting enables CIP on Set 1 of Parallel Intra, and on remaining 4x4 PUs (Set 2 when Parallel Intra is applied on 8x8 CU that is split into four 4x4 PUs); this setting improves losses introduced by parallel intra, without any increase in complexity (no additional RD searches needed);

#define CIP_BLOCK_DEF 4

#define CIP_BLOCK_CODE 0

- This setting uses CIP on same blocks as in the previous setting, and additionally uses CIP optionally on remaining PUs of size 8x8 and 16x16; optional CIP means that additional RD searches are needed and hence the encoder complexity is increased; we want to demonstrate that we can keep new functionalities of parallel intra with similar BD-rate results as without this new feature. In this case CIP can compensate for possible losses introduced by Parallel Intra.

#define CIP_BLOCK_DEF 4

#define CIP_BLOCK_CODE 16

## Cross-verification results

The encoded images match those of the decoded ones, and the PSNR and bitrate values are identical to the results provided by BBC. Table 1 shows the cross-checking results of combined intra prediction and Table 2 shows the cross-checking results of combination with combined intra prediction and parallel intra prediction proposed by Sharp.

We checked the source code and confirmed that the technique integrated in the software matches JCTVC-E130 contributed by BBC completely.

Table 1: Cross-checking results of test case 1 for I slice only coding structure

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | -0.4 | -0.4 | -0.1 | -0.3 | -0.4 |
| Class B | 0.3 | -0.4 | -0.4 | 0.4 | -0.5 | -0.5 |
| Class C | 0.1 | -0.5 | -0.5 | 0.2 | -0.6 | -0.6 |
| Class D | 0.1 | -0.5 | -0.5 | 0.2 | -0.5 | -0.5 |
| Class E | 0.6 | -0.4 | -0.5 | 0.6 | -0.4 | -0.5 |
| All | 0.2 | -0.4 | -0.4 | 0.2 | -0.5 | -0.5 |
| Enc Time[%] | 103% |  |  | 110% |  |  |
| Dec Time[%] | 102% |  |  | 107% |  |  |

Table 2: Cross-checking results of test case 2 for I slice only coding structure

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3 | -2.4 | -2.3 | -1.6 | -3.2 | -3.2 |
| Class B | 0.2 | -1.3 | -1.4 | -0.1 | -1.7 | -1.9 |
| Class C | 0.3 | -1.0 | -1.1 | 0.1 | -1.2 | -1.2 |
| Class D | 0.3 | -1.0 | -0.9 | 0.0 | -1.0 | -1.0 |
| Class E | 0.7 | -1.8 | -1.7 | -0.1 | -2.3 | -2.1 |
| All | 0.2 | -1.5 | -1.5 | -0.3 | -1.9 | -1.9 |
| Enc Time[%] | 116% |  |  | 133% |  |  |
| Dec Time[%] | 102% |  |  | 109% |  |  |

In the above tables, it is noted that the Dec Time was calculated the decoding time ratio without outputting the yuv file for both anchor and proposal.

The accompanied spreadsheet contains the detailed data of the cross-checking results.

# References

[1] 	F. Bossen ," Common test conditions and software reference configurations ", ITU-T and ISO/IEC JCT-VC 4th Meeting Daegu, JCTVC-D600, January 2011.

[2] 	"Core Experiment 6: Intra Prediction Improvement ", ITU-T and ISO/IEC JCT-VC 4th Meeting, Daegu, JCTVC-D606, January 2011.