| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E208 |
| --- | --- |

| Title: | CE6.d: Cross check report of Sharp's proposal (JCTVC-E315) from Toshiba |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Akiyuki Tanizawa Taichiro Shiodera 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, JAPAN | Tel: Email: | +81-44-549-2152 akiyuki.tanizawa@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This contribution contains cross-checking results of the software containing the tool of JCTVC-D074 proposed by SHARP Corporation on Core Experiment 6 subset d of intra prediction improvement. The software contains two levels of parallel intra prediction, 2X parallelism and 8X parallelism.

The verification task has been done successfully and the results perfectly match those proposed in JCTVC-E315.

# Cross-checking

The software based on HM version 2.0 and the experimental results were provided on March 3 2011 by SHARP. The software was complied fine and the cross-checking was performed on 64bit environments. The simulation has been conducted under CE common test conditions [1], [2] for both high efficiency and low complexity conditions in the case of I-slice only, random access and low delay coding structures.

## Configurations

The proponents require two test cases in the following;

- Parallel intra prediction with 2X parallelism

#define SHARP_PARR_INTRA     1

#define SHARP_PARR_INTRA_8X  0

- Parallel intra prediction with 8X parallelism

#define SHARP_PARR_INTRA     1

#define SHARP_PARR_INTRA_8X  1

## Cross-checking results

The encoded images match those of the decoded ones, and the PSNR and bitrate values are identical to the results provided by SHARP. Table 1, Table 2 and Table 3 show the cross-checking results of 2X parallelism. Table 4, Table 5 and Table 6 show the cross-checking results of 8X parallelism. It is noted that the parallel intra prediction scheme is applied over 720p sequences. Encoding and decoding results for class C and class D are identical to the anchor results.

We checked the source code and confirmed that the technique integrated in the software matches JCTVC-E315 contributed by SHARP completely.

Table 1: Cross-checking results of 2X parallelism for I slice only coding structure

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.4 | 0.3 | 0.3 | 0.5 | 0.4 | 0.3 |
| Class B | 0.4 | 0.2 | 0.3 | 0.6 | 0.3 | 0.3 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.7 | 0.4 | 0.4 | 1.0 | 0.5 | 0.5 |
| All | 0.3 | 0.2 | 0.2 | 0.4 | 0.2 | 0.2 |
| Enc Time[%] | 103% |  |  | 106% |  |  |
| Dec Time[%] | 101% |  |  | 104% |  |  |

Table 2: Cross-checking results of 2X parallelism for random access coding structure

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.5 | -0.3 | 0.3 | 0.2 | 0.1 |
| Class B | 0.2 | 0.1 | 0.1 | 0.3 | 0.2 | 0.2 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.1 | 0.1 | 0.0 | 0.2 | 0.1 | 0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 3: Cross-checking results of 2X parallelism for low delay coding structure

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.1 | 0.3 | 0.1 | 0.1 | -0.1 | 0.3 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.2 | 0.4 | 0.2 | -0.1 | 0.5 |
| All | 0.0 | 0.1 | 0.1 | 0.1 | 0.0 | 0.2 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 4: Cross-checking results of 8X parallelism for I slice only coding structure

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.4 | 0.3 | 0.3 | 0.5 | 0.4 | 0.3 |
| Class B | 0.5 | 0.3 | 0.3 | 0.7 | 0.4 | 0.4 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.8 | 0.5 | 0.4 | 1.0 | 0.5 | 0.5 |
| All | 0.3 | 0.2 | 0.2 | 0.4 | 0.2 | 0.2 |
| Enc Time[%] | 103% |  |  | 106% |  |  |
| Dec Time[%] | 101% |  |  | 104% |  |  |

Table 5: Cross-checking results of 8X parallelism for random access coding structure

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.1 | -0.4 | 0.2 | 0.2 | 0.2 |
| Class B | 0.3 | 0.1 | 0.1 | 0.3 | 0.2 | 0.1 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.1 | 0.1 | -0.1 | 0.2 | 0.1 | 0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 6: Cross-checking results of 8X parallelism for low delay coding structure

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.1 | 0.2 | 0.0 | 0.1 | 0.1 | 0.3 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.2 | -0.1 | 0.8 | 0.2 | 0.4 | 0.1 |
| All | 0.1 | 0.0 | 0.1 | 0.1 | 0.1 | 0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

In the above tables, it is noted that the Dec Time was calculated the decoding time ratio without outputting the yuv file for both anchor and proposal.

The accompanied spreadsheet contains the detailed data of the cross-checking results.

# References

[1] 	F. Bossen ," Common test conditions and software reference configurations ", ITU-T and ISO/IEC JCT-VC 4th Meeting Daegu, JCTVC-D600, January 2011.

[2] 	"Core Experiment 6: Intra Prediction Improvement ", ITU-T and ISO/IEC JCT-VC 4th Meeting, Daegu, JCTVC-D606, January 2011.