| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E211 |
| --- | --- |

| Title: | Modified motion vector memory compression |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Taichiro Shiodera Akiyuki Tanizawa Takeshi Chujoh Tomoo Yamakage 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, JAPAN | Tel: Email: | +81-44-549-2152 taichiro.shiodera@toshiba.co.jp akiyuki.tanizawa@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This contribution proposes a modified motion vector memory compression (MMC) method which includes two parts. One is the reference index memory compression which reduces the reference index memory to one sixteen in same manner as motion vector memory compression. The other part is modified representative motion vector derivation which uses MV of bottom-right 4x4 block (position 12 or 15) as a representative MV instead of upper-left 4x4 block in compressed 16x16 block.

Experimental results show that proposed method can achieve the average BD-rate gain of 0.5% for random access cases and 0.6% for low delay cases without additional encoder/decoder complexity.

# Introduction

The current version of the HM test model (HM2.0) includes motion vector memory compression (MMC) [1] which reduces the MV memory resolution to one sixteenth. In current MMC, a representative MV where locates in upper-right block of 16x16 block is only stored to memory. On the other hand, the reference index (RefIdx) is not compressed and still stored to RefIdx memory for each 4x4 block. These stored data are used for temporal MV predictor in adaptive motion vector prediction (AMVP) and/or Merge.

This contribution proposes two schemes in order to improve coding efficiency, RefIdx memory compression and modified representative motion vector derivation.

# Proposed method

## Proposal 1: RefIdx memory compression

### Description

In MMC of HM2.0, only MV in motion information is compressed to one sixteen and stored to memory for each 16x16 block. However, RefIdx in motion information is not compressed and is stored to memory for each 4x4 block. This might be caused mismatch between MV and RefIdx when temporal MV predictor is used as shown in Figure 1. In this proposal, a RefIdx is also stored to memory for each 16x16 block in same manner as MV memory compression as shown in Figure 2.

Figure 1. Relationship between MV and RefIdx memory in HM2.0

Figure 2. Relationship between MV and RefIdx memory in proposed 1 method

### Results

HM software version 2.0 with the common configuration [2] is used in this experiment. Table 1 shows BD-rate [%] and encoding/decoding time compared with HM2.0 anchor. The negative value indicates gain. It observed that proposed method gives some coding gain for low delay case nonetheless RefIdx is compressed by one sixteen. This is reason why there is no mismatch between MV and RefIdx when temporal MV predictor is used. Moreover, current RefIdx memory can be reduced since it is only necessary to store a representative RefIdx to memory.

Table 1. BD-Rate[%] and relative encoding/decoding time[%] (proposal 1)

|  | RA-HE |  |  | RA-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | -0.1 | 0.0 | -0.1 | -0.1 | -0.1 | -0.2 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | LD-HE |  |  | LD-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | 0.2 | -0.1 | 0.0 | 0.1 |
| Class C | -0.1 | 0.1 | 0.0 | -0.2 | -0.1 | -0.2 |
| Class D | -0.2 | -0.2 | -0.3 | -0.1 | -0.4 | -0.1 |
| Class E | -0.1 | -0.5 | 0.2 | 0.1 | -0.3 | -0.2 |
| All | -0.1 | -0.1 | 0.0 | -0.1 | -0.2 | -0.1 |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

## Proposal 2: Modified representative motion vector derivation

### Description

In current MMC of HM2.0, block position of the representative MV to be stored is set to upper-left 4x4 block in each 16x16 block. In this proposal, the position of representative MV to be stored is set to bottom-right 4x4 block (position 12 or 15 in Figure 3).

Figure 3. Position of representative MV.

### Results

HM software version 2.0 with the common configuration [2] is used in this experiment. The negative value indicates gain. The position of representative MV is set to bottom-right 4x4 block (position 15).

Table 2 and 3 show BD-rate [%] and encoding/decoding time of proposal 2.a and 2.b compared with HM2.0 anchor. Experiments of proposal 2.a are not finished yet. The coding gain of proposal 2.b is average of 0.4%, 0.3%, 0.4% and 0.4% for RA-HE, RA-LC, LD-HE and LD-LC, respectively. This modification is only one line in source code of current HM software version 2.0 and does not need additional complexity increase.

Table 2. BD-Rate[%] and relative encoding/decoding time[%] (proposal 2.a, position 12)

|  | RA-HE |  |  | RA-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | t.b.a |  |  |  |  |  |
| Class B |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |
| All |  |  |  |  |  |  |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |

|  | LD-HE |  |  | LD-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | t.b.a |  |  |  |  |  |
| Class C |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |
| All |  |  |  |  |  |  |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |

Table 3. BD-Rate[%] and relative encoding/decoding time[%] (proposal 2.b, position 15)

|  | RA-HE |  |  | RA-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3 | -0.3 | -0.3 | -0.4 | -0.4 | -0.4 |
| Class B | -0.2 | -0.1 | -0.1 | -0.1 | -0.1 | -0.1 |
| Class C | -0.3 | -0.3 | -0.3 | -0.3 | -0.2 | -0.3 |
| Class D | -0.5 | -0.4 | -0.4 | -0.5 | -0.5 | -0.4 |
| Class E |  |  |  |  |  |  |
| All | -0.3 | -0.3 | -0.3 | -0.3 | -0.3 | -0.3 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

|  | LD-HE |  |  | LD-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.3 | -0.2 | -0.4 | -0.2 | -0.3 | 0.0 |
| Class C | -0.5 | -0.3 | -0.5 | -0.5 | -0.5 | -0.7 |
| Class D | -0.6 | -0.7 | -0.7 | -0.5 | -0.5 | -0.9 |
| Class E | -0.7 | -1.0 | -0.3 | -0.4 | -0.9 | -0.5 |
| All | -0.5 | -0.5 | -0.5 | -0.4 | -0.5 | -0.5 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

# Syntax

No change in proposal 1 and 2.

# Experimental results

Finally, experiments which includes proposal 1 and 2 (2.a or 2.b) is conducted. HM software version 2.0 with the common configuration [2] is also used. BD-rate [8] is measured by Excel sheet provided on the JCT-VC reflector. The block position of the representative MV and RefIdx to be stored is set to potision 12 (proproal 1 and 2.a) or potision 15 (proproal 1 and 2.b), respectively.

Table 3 shows the experimental results of proposal for four inter configurations . The negative value indicates gain. The BD-rate of proposal (1 and 2.a) is -0.5%, -0.5%, -0.6% and -0.6% for RA-HE, RA-LC, LD-HE and LD-LC, respectively. Encoding and decoding time is not changed compared with the anchor. The BD-rate and encoder/decoder complexity of proposal (1 and 2.b) are almost same results as proposal (1 and 2.a).

Table 4. BD-Rate[%] and relative encoding/decoding time[%] (both proposal 1 and 2.a are used)

|  | RA-HE |  |  | RA-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 |
| Class B | -0.3 | -0.1 | -0.2 | -0.2 | -0.1 | -0.1 |
| Class C | -0.5 | -0.5 | -0.5 | -0.5 | -0.3 | -0.4 |
| Class D | -0.8 | -0.7 | -0.6 | -0.8 | -0.7 | -0.7 |
| Class E |  |  |  |  |  |  |
| All | -0.5 | -0.4 | -0.4 | -0.5 | -0.4 | -0.4 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.4 | -0.1 | -0.2 | -0.3 | -0.4 | -0.1 |
| Class C | -0.7 | -0.6 | -0.6 | -0.8 | -0.7 | -0.9 |
| Class D | -0.8 | -0.8 | -1.2 | -0.8 | -0.9 | -0.7 |
| Class E | -0.6 | -0.7 | -0.4 | -0.5 | -0.5 | -0.3 |
| All | -0.6 | -0.5 | -0.6 | -0.6 | -0.6 | -0.5 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 5. BD-Rate[%] and relative encoding/decoding time[%] (both proposal 1 and 2.b are used)

|  | RA-HE |  |  | RA-LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.4 | -0.2 | -0.5 | -0.4 | -0.4 | -0.3 |
| Class B | -0.3 | -0.2 | -0.2 | -0.2 | -0.1 | -0.1 |
| Class C | -0.5 | -0.5 | -0.5 | -0.5 | -0.4 | -0.3 |
| Class D | -0.7 | -0.7 | -0.7 | -0.8 | -0.7 | -0.7 |
| Class E |  |  |  |  |  |  |
| All | -0.5 | -0.4 | -0.5 | -0.5 | -0.4 | -0.4 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.4 | -0.2 | -0.4 | -0.4 | -0.3 | -0.1 |
| Class C | -0.6 | -0.5 | -0.5 | -0.7 | -0.7 | -0.8 |
| Class D | -0.8 | -0.4 | -0.8 | -0.8 | -0.7 | -0.7 |
| Class E | -0.7 | -0.4 | -0.2 | -0.6 | -0.9 | -0.8 |
| All | -0.6 | -0.4 | -0.5 | -0.6 | -0.6 | -0.6 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 4 shows summary of average Y BD-Rate in proposal. It was observed that the combination of proposal 1 and 2(2.a or 2.b) gives additive gain.

Table 4. Summary of Y BD-Rate[%] in this proposal.

|  | RA-HE | RA-LC | LD-HE | LD-LC |
| --- | --- | --- | --- | --- |
| proposal 1 | 0.0 | 0.0 | -0.1 | -0.1 |
| proposal 2.a (pos12) | t.b.a | t.b.a | t.b.a | t.b.a |
| proposal 2.b (pos15) | -0.3 | -0.3 | -0.5 | -0.4 |
| proposal 1 and 2.a | -0.5 | -0.5 | -0.6 | -0.6 |
| proposal 1 and 2.b | -0.5 | -0.5 | -0.6 | -0.6 |

# Conclusion

In this contribution, a modified motion vector memory compression (MMC) method is proposed. Experimental results show that proposed method can achieve the average BD-rate gain of 0.5-0.6 % for all cases without additional encoder/decoder complexity.

It’s suggested that this modified motion vector memory compression is introduced in the HEVC test model (HM).

# References

[1]	Y. Su, A. Segall, "CE9: Reduced resolution storage of motion vector data," Joint Collaborative Team on Video Coding, JCTVC-D072, Daegu, January 2011.

[2]	F. Bossen, " Common HM test conditions and software reference configurations," Joint Collaborative Team on Video Coding, JCTVC-D600, Daegu, January 2011.

# Patent rights declaration(s)

TOSHIBA Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).