| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March 2011 | Document: JCTVC-E212 WG11 Number: m19731 |
| --- | --- |

| Title: | Cross-verification report on constrained intra prediction scheme for flexible-sized prediction units (JCTVC-E203) |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JCT-VC] |  |  |
| Purpose: | [Report] |  |  |
| Author(s) or Contact(s): | Keiichi Chono Hirofumi Aoki Yuzo Senda 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 431 7595 chono@ct.jp.nec.com hao@bu.jp.nec.com y-senda@bc.jp.nec.com |
| Source: | [Information and Media Processing Labs, NEC Corporation] |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results of constrained intra prediction scheme for flexible-sized prediction units described in JCTVC-D094. Cross-verification task has been successfully completed, and it is confirmed that the BD-rate results match those provided by proponent and that encode/decode runtime measurements are similar to those of proponent. It is recommended that the JCTVC-E203 CIP is integrated into HM ver 3.0 and that CIP enhancement is further studied.

# Introduction

In Daegu meeting, AVC-style Constrained Intra Prediction (CIP) described in JCTVC-D086 [1] was adapted in HM design. Since the AVC-style CIP does not account for flexible-sized prediction units of HM design, it requires every reference sample to be intra-predicted. Thus it does not fully exploit available neighboring reference samples.

JCTVC-D094 [2] and JCTVC-E203 [3] investigates the characteristics of the AVC-style CIP and proposes to fill unavailable inter-coded samples one basis of using partially available intra-coded reference samples. This contribution reports cross-verification results of the JCTVC-E203 CIP.

# Software analysis and results

Through the inspection of HM-dev-r597-based software provided by Panasonic, it was confirmed that luma and chroma reference sample values of inter-coded blocks are replaced with values calculated from those of adjacent intra-coded blocks in accordance with the description of JCTVC-D094.

The HM-dev-r597-based software was successfully compiled by Microsoft Visual C++ 2008, and its 64-bit encode and decode executables were tested on Windows-based computing platform below.

| Computing platform | Usage |
| --- | --- |
| Windows 7 64-bit Professional Intel Xeon 3.33GHz and Memory 32GB | Up to 12 processes for encoding and decoding ran at the same time. |

In order to see the performance of JCTVC-D203, its results are compared to HM-dev-r597 results using AVC-style CIP (ConstrainedIntraPred=1). Simulation was conducted according to the RA and LC common test coding conditions described in JCTVC-D600 [4] except the CIP parameter.

Table 1 shows the summary of the results. Attached Excel sheet, 20110307-HM-2.0-NEC-dev597-AnchorCIP1-vs-PanasonicCIP1.xls, contains the detailed results. It was confirmed that BD-PSNR/Rate values are the same as those of provided by proponent and that encode/decode runtime measurements are similar to those of proponent.

Table 1 - JCTVC-D203 CIP vs AVC-style CIP

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.04 | -0.24 | -0.11 | -0.10 | -0.18 | -0.66 |
| Class B | -0.15 | -0.27 | -0.32 | -0.32 | -0.50 | -0.45 |
| Class C | -0.29 | -0.39 | -0.48 | -0.48 | -0.74 | -0.89 |
| Class D | -0.25 | -0.28 | -0.24 | -0.37 | -0.59 | -0.56 |
| Class E |  |  |  |  |  |  |
| All | -0.18 | -0.29 | -0.29 | -0.32 | -0.50 | -0.63 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.06 | -0.11 | -0.15 | -0.11 | -0.30 | -0.25 |
| Class C | -0.07 | -0.13 | -0.09 | -0.17 | -0.34 | -0.33 |
| Class D | -0.03 | 0.22 | -0.62 | -0.09 | -0.26 | 0.08 |
| Class E | -0.13 | -0.68 | -0.12 | -0.11 | 0.27 | 0.02 |
| All | -0.07 | -0.14 | -0.25 | -0.12 | -0.20 | -0.14 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Remarks

The AVC-style CIP does not account for flexible-sized prediction units of HM design, and it does not fully exploit available neighboring reference samples. It seems that the JCTVC-E203 CIP is a kind of bugfix of current HM CIP design. The JCTVC-E203 CIP shows consistent BD-rates improvements relative to the AVC-style CIP without affecting encoder/decoder runtimes; it is recommended that the JCTVC-E203 CIP is integrated into HM ver 3.0 and that CIP enhancement is further studied.

- Reference

- K. Chono, H. Aoki, X. Jing, V. Wahadaniah, C.S. Lim, and S.M.T. Naing, "Constrained intra prediction for reducing visual artifacts caused by lossy decoder-side memory compression," JCT-VC document JCTVC-D086, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 4th meeting, Daegu, January 2011.

- V. Wahadaniah, C.S. Lim, S.M.T. Naing, and X. Jing, "onstrained Intra Prediction Scheme for Flexible-Sized Prediction Units in HEVC,"  JCTVC-D094, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011.

- V. Wahadaniah, C.S. Lim, S.M.T. Naing, "Constrained intra prediction scheme for variable-sized prediction units in HEVC," J JCTVC-E203, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 5th meeting, Geneva, CH, March 2011.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-D600, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011.

# Patent rights declaration(s)

NEC Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).