| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E214 |
| --- | --- |

| Title: | Cross-verification report on pulse code modulation mode for HEVC (JCTVC-E057) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Viktor Wahadaniah ChongSoon Lim Block 1022 Tai Seng Avenue #06-3530 Singapore 534415 | Tel: Email: | +65 6550 5415 viktor.wahadaniah@sg.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution presents cross-verification results on the proposal for pulse code modulation (PCM) mode for HEVC described in JCTVC-E057. Cross-verification has been successfully completed. It is verified that the BD-rate results match exactly with those provided by the proponent and the relative encoding/decoding runtime measurements are similar to those of the proponent. It is recommended that the PCM operation mode 2 proposed in JCTVC-E057 is integrated into HM 3.0 and that PCM mode enhancement in HEVC is further considered and studied.

# Introduction

Pulse code modulation (PCM) mode for HEVC was proposed by NEC in 4th JCT-VC meeting in JCTVC-D044 ([1]). The PCM mode transmits sample values of the associated prediction unit without prediction, transform coding, and entropy coding, and thus it allows encoders to adjust the number of coded bits for the prediction unit to a predetermined value or less without complicated computation.

After the 4th JCT-VC meeting, the proponent investigated the improvement of PCM mode for HEVC especially from the aspect of prediction mode signaling. The outcome of the improvement is presented in JCTVC-E057 ([2]), in which three operation modes are described.

This contribution presents cross-verification results of operation mode 1 and operation mode 2 of the PCM mode for HEVC described in JCTVC-E057.

# Software analysis and results

PCM software based on HM 2.0 were received from NEC. Through the inspection of the source codes, it was verified that the PCM implementation conforms to descriptions of the initial version JCTVC-D044 as well as the subsequent adaptations for HM 2.0 and for the various operation modes.

The 64-bit encoder and decoder executable binaries were generated using Microsoft Visual C++ 2010. Encoding and decoding simulations using the full test set according to the common test conditions in JCTVC-D600 ([3]) are performed on machines running on Windows 7 64-bit Professional with up to 6 processes running in parallel.

Table 1 shows the summary of the results. Detailed results are available in the attached MS Excel sheets. It was confirmed that the BD-PSNR and BD-rate values produced in cross-check simulations are the same as those provided by the proponent, and that the relative encoding/decoding time measurements are similar to those of the proponent. It was noted that for the common test set of JCTVC-D600, on which PCM mode is not selected by the implemented bit-based mode decision, PCM operation mode 2 introduces smaller coding efficiency loss as compared to PCM operation mode 1.

Table 1 - Coding efficiency of PCM operation mode 1 compared to HM 2.0 anchor

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.01 | 0.02 | 0.07 | 1.39 | 0.78 | 0.56 |
| Class B | 0.01 | 0.01 | 0.01 | 1.57 | 0.98 | 0.84 |
| Class C | 0.01 | 0.01 | 0.02 | 1.20 | 0.83 | 0.81 |
| Class D | 0.01 | -0.01 | 0.00 | 0.89 | 0.60 | 0.60 |
| Class E | 0.02 | 0.05 | 0.07 | 2.37 | 1.05 | 1.27 |
| All | 0.01 | 0.02 | 0.03 | 1.44 | 0.85 | 0.79 |
| Enc Time[%] | 105% |  |  | 111% |  |  |
| Dec Time[%] | 104% |  |  | 104% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.08 | 0.14 | -0.22 | 0.77 | 0.41 | 0.16 |
| Class B | -0.02 | 0.01 | -0.10 | 0.71 | 0.56 | 0.42 |
| Class C | -0.02 | -0.03 | 0.08 | 0.52 | 0.38 | 0.48 |
| Class D | 0.00 | -0.08 | -0.10 | 0.38 | 0.31 | 0.28 |
| Class E |  |  |  |  |  |  |
| All | 0.01 | 0.01 | -0.08 | 0.60 | 0.42 | 0.34 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 104% |  |  | 105% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.01 | 0.09 | -0.11 | 0.37 | 0.69 | 0.80 |
| Class C | -0.01 | 0.21 | 0.04 | 0.31 | 0.41 | 0.43 |
| Class D | -0.09 | -0.04 | -0.34 | 0.26 | 0.62 | 0.62 |
| Class E | -0.23 | 0.29 | 0.69 | 0.39 | 0.59 | 0.67 |
| All | -0.06 | 0.13 | 0.02 | 0.33 | 0.58 | 0.64 |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 105% |  |  | 103% |  |  |

Table 2 - Coding efficiency of PCM operation mode 2 compared to HM 2.0 anchor

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.00 | -0.02 | 0.01 | 0.08 | 0.12 | 0.11 |
| Class B | 0.00 | 0.00 | 0.01 | 0.11 | 0.14 | 0.12 |
| Class C | 0.00 | 0.01 | 0.01 | 0.03 | 0.03 | 0.02 |
| Class D | 0.00 | 0.00 | 0.00 | 0.01 | 0.01 | 0.02 |
| Class E | -0.01 | 0.01 | 0.03 | 0.16 | 0.19 | 0.22 |
| All | 0.00 | 0.00 | 0.01 | 0.08 | 0.10 | 0.09 |
| Enc Time[%] | 105% |  |  | 107% |  |  |
| Dec Time[%] | 103% |  |  | 109% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.02 | 0.30 | -0.12 | 0.07 | 0.08 | -0.04 |
| Class B | -0.01 | -0.03 | -0.06 | 0.06 | 0.05 | 0.04 |
| Class C | -0.02 | -0.02 | -0.06 | 0.02 | 0.03 | 0.00 |
| Class D | -0.01 | 0.03 | 0.04 | 0.00 | -0.04 | -0.02 |
| Class E |  |  |  |  |  |  |
| All | -0.01 | 0.07 | -0.05 | 0.04 | 0.03 | 0.00 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 105% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.01 | 0.14 | -0.05 | 0.05 | 0.12 | 0.29 |
| Class C | 0.00 | 0.18 | 0.13 | -0.04 | 0.21 | -0.16 |
| Class D | -0.03 | 0.03 | -0.12 | 0.06 | 0.06 | 0.07 |
| Class E | -0.16 | -0.39 | 0.51 | 0.00 | 0.36 | 0.09 |
| All | -0.04 | 0.02 | 0.08 | 0.02 | 0.17 | 0.08 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 106% |  |  | 102% |  |  |

# Remarks

Cross-verification of PCM mode for HEVC presented in JCTVC-E057 has been successfully completed. It is verified that the BD-PSNR and BD-rate results match exactly with those provided by the proponent and the relative encoding/decoding runtime measurements are similar to those of the proponent. It is recommended that the PCM operation mode 2 proposed in JCTVC-E057 is integrated into HM 3.0 and that PCM mode enhancement in HEVC is further considered and studied.

- Reference

- K. Chono, K. Senzaki, H. Aoki, J. Tajime, Y. Senda, " Pulse code modulation mode for HEVC," JCT-VC document JCTVC-D044, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 4th meeting, Daegu, KR, January 2011.

- K. Chono, H. Aoki, Y. Senda, "Pulse code modulation mode for HEVC", JCT-VC document JCTVC-E057, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting, Geneva, CH, March 2011.

- JCT-VC, “Common test conditions and software reference configurations”, JCT-VC document JCTVC-D600, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting, Daegu, KR, January 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).