| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E216_r1 |
| --- | --- |

| Title: | Bi-prediction by single motion vector using decoder-side inter-reference ME |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Motoharu Ueda JVC KENWOOD Holdings, Inc. 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-9945 ueda.motoharu@jk-holdings.com |
| Source: | JVC KENWOOD Holdings, Inc. |  |  |

_____________________________

# Abstract

This proposal focuses on the complexity analysis of the technique “Refinement motion compensation using DMVD (RMC)” proposed in the past JCT-VC meetings under CE1/TE1 activity. The conventional　bi-predictive motion compensation is executed by single motion information using decoder-side inter-reference motion estimation indirectly in this technique.

In this contribution, the decoder complexity of this technique is evaluated in the various aspects such as the worst case of memory access, and block matching operation.

The simplified algorithm is proposed to reduce the decoder complexity. The first one is to change the search range of DMVD. The second one is to change the sub-pixel filtering on decoder-side motion estimation. The last one is to restrict the PU size that can be used for DMVD.

The simulation results show that the proposed technique provides 1.5% BD-rate gain on original algorithm and 1.2% BD-rate gain on simplified algorithm against HM2.0 under random access condition.

Also the simulation results show that the proposed technique increase 42% encoder runtime and 21% decoder runtime on original algorithm and 23% encoder runtime and 14% decoder runtime on simplified algorithm against HM2.0 under random access condition.

# Introduction

At 4th JCT-VC meeting in Daegu, Refinement Motion Compensation using DMVD (RMC) implemented into TMuC0.9 (HM1.0) was examined under CE1 common test condition [1].

The advantage of RMC is that coding bits can be reduced because bi-predictive MC block is obtained by transmitting only one motion vector. On the other hand, this technique needs additional reference memory access and block matching operation for DMVD.

The runtime measurement is one of the methods to evaluate the complexity, which can estimate average of increase of the amount of the operations. However, the increase of the amount of memory access and the number of operations at worst case cannot clearly be evaluated by the runtime measurement.

In this contribution, it introduces the amount of reference memory access at the worst case and the number of operations for additional complexity measurement value. Moreover, the increase of this complexity measurement value in the case of using RMC algorithm is estimated, and the algorithm that suppressed the complexity is examined.

# Algorithm Description

## Algorithm overview

Figure 1 Conceptual diagram of RMC

The conceptual diagram of RMC is shown in Figure 1.

In this technique, a first MC block is obtained from reference picture 1. A second MC block is obtained by executing inter-reference motion estimation. Finally, bi-predictive MC block (RMC block) is generated as combination of the first MC block and the second MC block.

Figure 2 The relationship of vectors used in RMC

Figure 2 shows the relationship between the motion vector derived by DMVD and motion vectors used for bi-predictive prediction in RMC.

In RMC, MC block for reference picture 1 is extracted by transmitted vector.

The transmitted motion vector for uni-predictive prediction is rounded to integer accuracy.

DMVD process is performed between reference picture 1 and reference picture 2 using template block extracted by rounded motion vector.

On the decoder, MC block for reference picture 2 is extracted by addition of transmitted motion vector and motion vector derived by DMVD process.

The difference between the transmitted motion vector and the motion vector rounded to integer means the phase shift vector for both references.

On the encoder, after simulate DMVD process on decoder-side bi-predictive motion estimation is performed for detecting the phase shift vector.

MV1 shows the motion vector between reference picture 1 and target picture, at the same time it is contained the common phase information for reference picture 1 and reference picture 2 using for the bi-predictive prediction after DMVD process.

## Complexity analysis of the decoder

To estimate the actual increase of the decoder complexity using RMC, additional complexity measurements are introduced as follows.

1) The reference memory access for motion compensation

2) The sub-pixel filtering operation for DMVD

3) The block matching operation for DMVD

### Reference memory access

For estimating the amount of the access of the reference memory for motion compensation, the decoder HW model as shown in Figure 3 is assumed. The decoder HW reads necessary pixels for motion compensation reference picture memory to cash memory at low access speed at first. After that, the decoder HW executes sub-pixel filtering to generate motion compensate signals using pixels in the cash memory with high access speed.

To consider the bottleneck of the reference memory access, the worst case of necessary pixels should be considered as the complexity measurement value for the reference memory access.

Figure 3 The assumption model of the reference memory access at the decoder HW

Figure 4 shows the necessary reference memory access area for luminance motion compensation. In current HEVC algorithm (HM2.0), DCTIF 8tap filter is used for interpolation filtering of quarter-pixel accuracy luminance motion compensation.

Figure 4 Memory access area of luminance motion compensation

If the horizontal pixels on prediction unit are equal to block_size_x and vertical pixels on prediction unit are equal to block_size_y and the number of coefficients of interpolation filter is equal to filter_taps, the necessary reference memory access area (NEMA) is as follows.

NEMA = (filter_taps + block_size_x - 1)*(filter_taps + block_size_y - 1)

The worst case of the reference memory access per decoding pixels should be considered for making amends to decode all assumed bitstream in real-time. Therefore, the reference memory access per decoding pixels (access ratio) is estimated by each prediction unit size and the access ratio of the worst case is using the indication of the memory access complexity.

Table1 shows the reference memory access ratio against prediction unit size of motion compensation.

Table 1 Reference memory access ratio against Prediction Unit Size

| MC blocksize | (pixels) | uni-predictive | bi-predictive | access ratio |
| --- | --- | --- | --- | --- |
| 4x4 | 16 | 121 | 242 | 15.13 |
| 4x8/8x4 | 32 | 165 | 330 | 10.31 |
| 8x8 | 64 | 225 | 450 | 7.03 |
| 8x16/16x8 | 128 | 345 | 690 | 5.39 |
| 16x16 | 256 | 529 | 1058 | 4.13 |
| 16x32/32x16 | 512 | 897 | 1794 | 3.50 |
| 32x32 | 1024 | 1521 | 3042 | 2.97 |
| 32x64/64x32 | 2048 | 2769 | 5538 | 2.70 |
| 64x64 | 4096 | 5041 | 10082 | 2.46 |

The extended area for the interpolation filter is same for all prediction unit size and the access ratio depends on the number of reference picture using for motion compensation, the worst case of the reference memory access is on 4x4PU bi-prediction.

In the case of 4x4PU bi-prediction, access ratio is equal to 15.13. It means that 15.13 times memory access against memory access for decoded block is needed for the motion compensation at the worst case.

In the RMC, the reference memory access for the first reference picture is same as normal motion compensation because the template block of DMVD can get in the memory access area for normal motion compensation always.

For the second reference picture, inter-reference motion estimation is executed based on search center position and added phase shift vector after the result of inter-reference motion estimation. After that, sub-pixel filtering for motion compensation of the second reference picture is executed using extended area for interpolation filter.

Figure 5 shows the necessary reference memory access area of second reference in the RMC including luminance motion compensation.

Figure 5 Memory access area of second reference using RMC

Notes: If additional memory access can be executed after full pixel search of DMVD without considering memory access time, the total memory access can be decreased than Figure 5.

If the horizontal pixels on prediction unit are equal to block_size_x and vertical pixels on prediction unit are equal to block_size_y and the number of coefficients of sub-pixel filter is equal to filter_taps and horizontal full pixel search range of DMVD is equal to full_pixel_search_range_x and vertical full pixel search range of DMVD is equal to full_pixel_search_range_y, the reference memory access area (NEMA_RMC) is as follows.

NEMA_RMC = (filter_taps + (full_pixel_search_range_x + 1)*2 + block_size_x)

*(filter_taps + (full_pixel_search_range_y + 1)*2 + block_size_y)

Table2 shows the reference memory access ratio against prediction unit size of motion compensation using the original RMC algorithm (same algorithm as JCTVC-D120).

Table 2 Reference memory access ratio against Prediction Unit Size using RMC

| MC blocksize | (pixels) | 1st reference | 2nd reference | Total (pixels) | access ratio |
| --- | --- | --- | --- | --- | --- |
| 4x4 | 16 | 121 | 400 | 521 | 32.56 |
| 4x8/8x4 | 32 | 165 | 480 | 645 | 20.16 |
| 8x8 | 64 | 225 | 576 | 801 | 12.52 |
| 8x16/16x8 | 128 | 345 | 768 | 1113 | 8.70 |
| 16x16 | 256 | 529 | 1024 | 1553 | 6.07 |
| 16x32/32x16 | 512 | 897 | 1536 | 2433 | 4.75 |
| 32x32 | 1024 | 1521 | 2304 | 3825 | 3.74 |
| 32x64/64x32 | 2048 | 2769 | 3840 | 6609 | 3.23 |
| 64x64 | 4096 | 5041 | 6400 | 11441 | 2.79 |

In the case of 4x4PU bi-prediction, access ratio is equal to 32.56. It means that 32.56 times memory access against memory access for decoded block is needed for the motion compensation using RMC (full_pixel_search_range_x and full_pixel_search_range_y are equal to 3 by the setting of previous proposal) at worst case.

### Sub-pixel filtering operation of DMVD

In current HEVC algorithm (HM2.0), DCTIF 8tap interpolation filter is defined by the shift operation and the addition operation [2].

Table 3 shows DCTIF 8tap filter coefficients for 1/4 pixel MV accuracy and the number of operations for each sub-pixel position.

Table 3 DCTIF 8tap filter coefficients for 1/4 pixel MV accuracy and the number of operations

| position | coefficents (6bits) / 64 | shift operation | add operation |
| --- | --- | --- | --- |
| 1/4 | { -1, 4, -10, 57, 19, -7, 3, -1 } | 4 | 12 |
| 1/2 | { -1, 4, -11, 40, 40, -11, 4, -1 } | 4 | 9 |

Notes: The number of shift operations for the 1/4 position seems to be equal 5 in the HM2.0 software.

In the RMC, full-pixel and half-pixel and quarter-pixel hierarchical block matching is used for inter-reference motion estimation of DMVD. After full-pixel result of block matching, half-pixel block matching is executed at 8 positions around the result full-pixel as shown in Figure 6.

Figure 6 related areas for executing sub-pixel filtering

The half-pixel filtering for the half-pixel block matching is calculated half-pixel accuracy reference block for 8 positions using vertical filtering and horizontal filtering.

Table 4 shows the number of operations per pixel for half-pixel filtering using DCTIF 8tap filter.

Table 4 The number of operations per pixel for half-pixel filtering using DCTIF 8tap

| half-pixel filtering |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| position | num | num filter_v |  | num filter_h |  | shift operation | add operation |
| 1/2 , 1 | 2 | 0 |  | 2 |  | 8 | 18 |
| 1, 1/2 | 2 | 2 |  | 0 |  | 8 | 18 |
| 1/2, 1/2 | 4 | 16 |  | 4 |  | 80 | 180 |
|  |  |  |  |  | total | 96 | 216 |

The quarter-pixel block matching is executed at 8 positions around the half-pixel result as same manner as the half-pixel block matching.

Table 5 shows the number of operations per pixel for quarter-pixel filtering using DCTIF 8tap filter.

Table 5 The number of operations per pixel for quarter-pixel filtering using DCTIF 8tap

| quarter-pixel filtering |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| position | num | num filter_v |  | num filter_h |  | shift operation | add operation |
| 1/4 , 1or1/2 | 2 | 0 |  | 2 |  | 8 | 24 |
| 1or1/2, 1/4 | 2 | 2 |  | 0 |  | 8 | 24 |
| 1/4, 1/4 | 4 | 16 |  | 4 |  | 80 | 240 |
|  |  |  |  |  | total | 96 | 288 |

Totally, 192 times of shift operations and 504 times of add operations per pixel are needed on prediction unit using the bi-prediction using the original RMC algorithm.

### Block matching operation of DMVD

In the RMC, inter-reference motion estimation algorithm is hierarchical block matching of full-pixel and half-pixel and quarter-pixel accuracy.

In the previous proposal, DMVD full-pixel search range of DMVD is ±3 pixels. There are 49 search points on full-pixel search and 8 search points on half-pixel search and 8 search points on quarter-pixel search. Add operations for the number of pixels and absolute operations for the number of pixels and summation operations for the number of pixels minus one are needed for the block matching process.

Table 6 shows the number of operations per pixel for block matching of DMVD.

Table 6 The number of operations per pixel for block matching of DMVD

| full-pixel search point | 49 |
| --- | --- |
| half-pixel search point | 8 |
| quarter-pixel search point | 8 |
| add operation per 4x4PU | 16 |
| absolute operation per 4x4PU | 16 |
| summation (=add) operation per 4x4PU | 15 |
| Total operation per pixel | 190.9375 |

Totally, 190.9375 operations per pixel are needed on prediction unit using the bi-prediction using the original RMC algorithm.

## Simplified algorithm for considering the decoder complexity

Based on the results of the increase of the decoder complexity, the simplified algorithm is considered for decrease decoder complexity in aspect of the measurement of the reference memory access and block matching operations as follows.

1) Search range limitation of inter-reference motion estimation

2) Decrease number of operations for interpolation filter

3) Limited RMC algorithm adaptation for small prediction unit

### Limited search range of DMVD

The necessary reference memory access area is depended on the DMVD search range. Therefore, full-pixel search range of DMVD is set to ±1 pixel (total ±1.75 pixels) in the simplified algorithm of RMC.

Table 7 shows the reference memory access ratio against prediction unit size of motion compensation in the case of search range is equal to±1.75 pixels.

Table 7 Reference memory access ratio in the case of search range is equal to ±1.75 pixels

| MC blocksize | (pixels) | 1st reference | 2nd reference | Total (pixels) | access ratio |
| --- | --- | --- | --- | --- | --- |
| 4x4 | 16 | 121 | 256 | 377 | 23.56 |
| 4x8/8x4 | 32 | 165 | 320 | 485 | 15.16 |
| 8x8 | 64 | 225 | 400 | 625 | 9.77 |
| 8x16/16x8 | 128 | 345 | 560 | 905 | 7.07 |
| 16x16 | 256 | 529 | 784 | 1313 | 5.13 |
| 16x32/32x16 | 512 | 897 | 1232 | 2129 | 4.16 |
| 32x32 | 1024 | 1521 | 1936 | 3457 | 3.38 |
| 32x64/64x32 | 2048 | 2769 | 3344 | 6113 | 2.98 |
| 64x64 | 4096 | 5041 | 5776 | 10817 | 2.64 |

The reference memory access ratio is about 27% decrease against the original RMC algorithm in the case of full-pixel search range is equal to ±1 pixel.

Also, the number of operations of block matching is decrease in the case of full-pixel search range is equal to ±1 pixel. Table 8 shows the number of operations per pixel for block matching of DMVD in the case of search range is equal to±1.75 pixels.

Table 8 The number of operations per pixel in the case of search range is equal to ±1.75 pixels

| full-pixel search point | 9 |
| --- | --- |
| half-pixel search point | 8 |
| quarter-pixel search point | 8 |
| add operation per 4x4PU | 16 |
| absolute operation per 4x4PU | 16 |
| summation (=add) operation per 4x4PU | 15 |
| Total operation per pixel | 73.4375 |

The number of operations of block matching is about 61% decrease against the original RMC algorithm in the case of full-pixel search range is equal to ±1 pixel.

### Bi-linear filtering for sub-pixel ME of DMVD

The number of operations for interpolation filter is depended on the number of taps of filter. Therefore, bi-linear filter is introduced to use interpolation filter of DMVD.

Table 9 shows bi-linear filter coefficients for 1/4 pixel MV accuracy and the number of operations for each sub-pixel position.

Table 9 bi-linear filter coefficients for 1/4 pixel MV accuracy and the number of operation

| position | coefficients (2bits) / 4 | shift operation | add operation |
| --- | --- | --- | --- |
| 1/4 | { 3, 1 } | 1 | 2 |
| 1/2 | { 2, 2 } | 1 | 1 |

Table 10 shows the number of operations per pixel for half-pixel filtering using bi-linear filter.

Table 10 The number of operation per pixel for half-pixel filtering using bi-linear filter

| half-pixel filtering |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| position | num | num filter_v |  | num filter_h |  | shift operation | add operation |
| 1/2 , 1 | 2 | 0 |  | 2 |  | 2 | 2 |
| 1, 1/2 | 2 | 2 |  | 0 |  | 2 | 2 |
| 1/2, 1/2 | 4 | 4 |  | 4 |  | 8 | 8 |
|  |  |  |  |  | total | 12 | 12 |

Table 11 shows the number of operations per pixel for quarter-pixel filtering using bi-linear filter.

Table 11 The number of operation per pixel for quarter-pixel filtering using bi-linear filter

| quarter-pixel filtering |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| position | num | num filter_v |  | num filter_h |  | shift operation | add operation |
| 1/4 , 1or1/2 | 2 | 0 |  | 2 |  | 2 | 4 |
| 1or1/2, 1/4 | 2 | 2 |  | 0 |  | 2 | 4 |
| 1/4, 1/4 | 4 | 4 |  | 4 |  | 8 | 16 |
|  |  |  |  |  | total | 12 | 24 |

Totally, 24 times of shift operations and 36 times of add operations per pixel are needed on prediction unit using the bi-prediction using bi-linear interpolation filter for DMVD.

The total number of operations per pixel for bi-linear interpolation filter is 60 and 91% decrease against the original RMC algorithm.

### Restriction of the PU size for RMC

In the case of small prediction unit, the reference memory access ratio becomes bigger than large prediction unit. Therefore, the prediction unit which size is smaller than 8x8PU is not allowed to be RMC mode.

Table 12 shows the reference memory access ratios of restricted RMC

Table 12 Reference memory access ratios of restricted RMC

| MC blocksize | (pixels) | 1st reference | 2nd reference | Total (pixels) | access ratio |
| --- | --- | --- | --- | --- | --- |
| 4x4 | 16 | 121 | 121 | 242 | 15.13 |
| 4x8/8x4 | 32 | 165 | 165 | 330 | 10.31 |
| 8x8 | 64 | 225 | 576 | 801 | 12.52 |
| 8x16/16x8 | 128 | 345 | 768 | 1113 | 8.70 |
| 16x16 | 256 | 529 | 1024 | 1553 | 6.07 |
| 16x32/32x16 | 512 | 897 | 1536 | 2433 | 4.75 |
| 32x32 | 1024 | 1521 | 2304 | 3825 | 3.74 |
| 32x64/64x32 | 2048 | 2769 | 3840 | 6609 | 3.23 |
| 64x64 | 4096 | 5041 | 6400 | 11441 | 2.79 |

In the case of 8x8PU using RMC algorithm, the memory access ratios are less than the access ratio of 4x4PU for normal motion compensation.

Table13 shows the reference memory access ratios of restricted RMC and search range of DMVD is equal to±1.75pixels.

Table 13 Reference memory access ratios of restricted RMC and search range equal ±1.75 pixels

| MC blocksize | (pixels) | 1st reference | 2nd reference | Total (pixels) | access ratio |
| --- | --- | --- | --- | --- | --- |
| 4x4 | 16 | 121 | 121 | 242 | 15.13 |
| 4x8/8x4 | 32 | 165 | 165 | 330 | 10.31 |
| 8x8 | 64 | 225 | 400 | 625 | 9.77 |
| 8x16/16x8 | 128 | 345 | 560 | 905 | 7.07 |
| 16x16 | 256 | 529 | 784 | 1313 | 5.13 |
| 16x32/32x16 | 512 | 897 | 1232 | 2129 | 4.16 |
| 32x32 | 1024 | 1521 | 1936 | 3457 | 3.38 |
| 32x64/64x32 | 2048 | 2769 | 3344 | 6113 | 2.98 |
| 64x64 | 4096 | 5041 | 5776 | 10817 | 2.64 |

The simplified RMC algorithm is used these three changed parameters of DMVD algorithm.

Table 14 shows the difference between the original algorithm of RMC and the simplified algorithm.

Table 14 the difference between the original algorithm of RMC and the simplified algorithm

| Specification | Original RMC algorithm (same as JCTVC-D120) | Simplified RMC algorithm (decrease the complexity) |
| --- | --- | --- |
| Search range of inter-reference ME | ±3.75 pixels | ±1.75 pixels |
| Sub-pixel ME accuracy | 1/4 pixel | 1/4 pixel |
| Interpolation filter for inter-reference ME | Same as MC (DCTIF 8tap) | bi-linear filter |
| Matching technique | SAD (luminance only) | SAD (luminance only) |
| DMVD block size | Same size as PU | Same size as PU |
| DMVD size limitation | Allowed all size of PU | Not used for 4x4/4x8/8x4PU |
| Worst case of memory access ratio | 32.56 (RMC mode in 4x4PU) | 15.13 (normal bi-pred in 4x4PU) |
| Number of operations for DMVD | 886.9375 times / pixel | 133.4375 times / pixel |

## Software Implementation

### Software

This proposed technique is implemented into HM2.0 software. The original RMC algorithm and the simplified RMC algorithm can be executed by changing the configuration file settings. The parameters related the difference of algorithm send by the slice header.

### Syntax structure

The modified syntax based on Working Draft 2 of HEVC [3] is as follows.

Slice header

| slice_header( ) { | Descriptor |
| --- | --- |
| first_tb_in_slice | ue(v) |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| frame_num | u(v) |
| if( IdrPicFlag ) |  |
| idr_pic_id | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| pic_order_cnt_lsb /* | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( nal_ref_idc != 0 ) |  |
| dec_ref_pic_marking( ) |  |
| if(slice_type = = B){ |  |
| rmc_enable_flag | u(1) |
| if(rmc_enable_flag){ |  |
| Rmc_search_range | u(2) |
| Rmc_max_cu_depth_minus1 | ue(v) |
| Rmc_filter_type | ue(v) |
| } |  |
| } |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| Cabac_init_idc | ue(v) |
| slice_qp_delta | se(v) |
| if( adaptive_loop_filter_enabled_flag ) |  |
| alf_param() |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| } else |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| } |  |

rmc_enable_flag specifies whether refinement motion compensation process is applied or not for the slice.

If rmc_enable_flag is equal to 1, RMC mode is enabled in the slice.

If rmc_enable_flag is equal to 0, RMC mode is disabled in the slice.

rmc_search_range specifies the size of the search range used in DMVD process in RMC. Actual size of search range RmcDmvdSearchRangeSize is derived as follows:

RmcDmvdSearchRangeSize = (1 << (rmc_search_range + 1)) - 1

rmc_max_cu_depth_minus1 specifies the maximum hierarchy depth for RMC units. The variable MaxRmcUnitSize is derived as follows:

MaxRmcUnitSize = rmc_max_cu_depth_minus1 + 1

rmc_filter_type specifies the filter type of interpolation filter using DMVD process

0 : same interpolation filter as motion compensation, 1 : bi-linear filter, 2 : reserved

prediction_unit

| prediction_unit( x0, y0, log2PUWidth, log2PUHeight, PartIdx , InferredMergeFlag ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( NumMVPCand( L0 ) > 1 ) |  |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if( NumMVPCand( L1 ) > 1 ) |  |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( !prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { /* MODE_INTER */ |  |
| if( !InferredMergeFlag && NumMergeCand > 0 ) |  |
| merge_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( merge_flag[ x0 ][ y0 ] && NumMergeCand > 1 ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] = = Pred_LC ) { |  |
| if( num_ref_idx_lc_active_minus1 > 0 ) |  |
| ref_idx_lc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if ( rmc_pu_enable[ log2PUWidth ] [ log2PUHeight ] && |  |
| rmc_idx_enable_lc[ ref_idx_lc[ x0 ][ y0 ] ] ) |  |
| rmc_flag | u(1) ¦ ae(v) |
| mvd_lc[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_lc[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( LcToLx ) > 1 ) |  |
| mvp_idx_lc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| else { /* Pred_L0 or Pred_BI */ |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if ( rmc_pu_enable[ log2PUWidth ][ log2PUHeight ] && |  |
| rmc_idx_enable_l0[ ref_idx_l0[ x0 ][ y0 ] ] ) |  |
| rmc_flag | u(1) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( L0 ) > 1 ) |  |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] = = Pred_BI ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| mvd_l1[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l1[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( L1 ) > 1 ) |  |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

rmc_flag specifies whether RMC process is applied or not for the prediction unit.

If rmc_flag is equal to 1, RMC mode is enabled in the prediction unit.

If rmc_flag is equal to 0, RMC mode is disabled in the prediction unit.

rmc_idx_enable_l0[ ] , rmc_idx_enable_lc[ ] are flags for enabling RMC depended on relation of POC of the first reference and the second reference.

If POC of first reference is same as second reference, the flag is inferred to zero and does not use RMC.

rmc_pu_enable[ log2PUWidth ][ log2PUHeight ] is inferred to 0 in the case that RMC is disabled by the limitation of MaxRmcUnitSize.

# Experiments

## Test conditions

The random access high efficiency condition in common test conditions [4] is used.

## Experimental environment

The experimental environment used is presented in Table 15 and Table 16.

Table 15 Experimental environment for encoding

| CPU | Intel Xeon 5160 3.00GHz |
| --- | --- |
| Memory | 16GB / 12GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Professional edition SP1 |
| Executable | x64 |

Table 16 Experimental environments for decoding

| CPU | Intel Core i7 860 2.80GHz |
| --- | --- |
| Memory | 8GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Professional edition SP1 |
| Executable | X64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Table 17 presents the simulation results under random access condition for the original algorithm and the simplified algorithm.

Table 17 Simulation results under random access condition

|  | original (same as JCTVC-D120) |  |  | simplified |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.3 | -1.8 | -1.6 | -1.1 | -1.5 | -1.4 |
| Class B | -1.5 | -1.3 | -1.3 | -1.3 | -0.9 | -0.9 |
| Class C | -1.8 | -1.8 | -1.9 | -1.2 | -1.1 | -1.2 |
| Class D | -1.6 | -1.7 | -1.6 | -1.0 | -1.0 | -1.0 |
| Class E |  |  |  |  |  |  |
| All | -1.5 | -1.6 | -1.6 | -1.2 | -1.1 | -1.1 |
| Enc Time[%] | 142% |  |  | 123% |  |  |
| Dec Time[%] | 121% |  |  | 114% |  |  |

The simulation results show that average 1.5% BD-rate gain on the original RMC algorithm and 1.2% BD-rate gain on the simplified RMC algorithm against HM2.0 under random access condition.

For the simplified RMC algorithm, the simulation results show that about -0.3% BD-rate gains against the original RMC algorithm.

Also the simulation results show that 42% increase of encoder runtime and 21% increase of decoder runtime on original RMC algorithm against HM2.0 under random access condition.

For the simplified RMC algorithm, the simulation results show that 23% increase of encoder runtime and 14% increase of decoder runtime against HM2.0 and about 19% decrease of encoder runtime and about 7% decrease of decoder runtime against the original RMC algorithm.

Table 18 presents the coding efficiency comparison results of each test sequence at random access for the original algorithm and the simplified algorithm.

Table 18 Coding efficiency comparison results of each test sequence under random access condition

| sequence | original RMC algorithm |  |  | simplified RMC algorithm |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Y | BD-rate U | BD-rate V | BD-rate Y | BD-rate U | BD-rate V |
| Traffic | -1.24 | -0.91 | -0.95 | -1.07 | -0.83 | -0.82 |
| PeopoleOnStreet | -2.25 | -3.34 | -3.03 | -1.78 | -2.32 | -2.14 |
| Nebuta | -0.44 | -1.14 | -1.09 | -0.39 | -1.41 | -1.35 |
| SteamLocomotive | -1.32 | -1.62 | -1.26 | -1.11 | -1.33 | -1.46 |
| Kimono | -1.42 | -1.03 | -1.08 | -1.06 | -0.82 | -0.84 |
| ParkScene | -0.88 | -0.80 | -0.85 | -0.66 | -0.52 | -0.60 |
| Cactus | -1.54 | -1.51 | -1.47 | -1.47 | -1.14 | -1.22 |
| BasketballDrive | -2.38 | -2.15 | -2.38 | -1.77 | -1.31 | -1.64 |
| BQTerrace | -1.14 | -0.90 | -0.79 | -1.30 | -0.50 | -0.38 |
| BasketballDrill | -1.93 | -2.25 | -2.25 | -1.39 | -1.43 | -1.40 |
| BQMall | -1.69 | -1.30 | -1.29 | -1.32 | -0.75 | -0.90 |
| PartyScene | -1.24 | -1.50 | -1.46 | -0.98 | -1.04 | -0.94 |
| RaceHorces | -2.19 | -2.32 | -2.56 | -1.25 | -1.21 | -1.52 |
| BasketballPass | -2.47 | -2.65 | -2.52 | -1.69 | -1.49 | -1.40 |
| BQSquare | -0.23 | -0.43 | -0.31 | -0.33 | -0.32 | -0.37 |
| BlowingBubbles | -1.13 | -0.99 | -1.01 | -0.77 | -0.48 | -0.63 |
| RaceHorces | -2.39 | -2.75 | -2.74 | -1.39 | -1.65 | -1.67 |
| ClassA Average | -1.3 | -1.8 | -1.6 | -1.1 | -1.5 | -1.4 |
| ClassB Average | -1.5 | -1.3 | -1.3 | -1.3 | -0.9 | -0.9 |
| ClassC Average | -1.8 | -1.8 | -1.9 | -1.2 | -1.1 | -1.2 |
| ClassD Average | -1.6 | -1.7 | -1.7 | -1.1 | -1.0 | -1.0 |
| Total Average | -1.5 | -1.6 | -1.6 | -1.2 | -1.1 | -1.1 |

## Additional Results

In order to investigate the influence of each change on the simplified algorithm, the additional simulation is performed using only one change on the simplified algorithm from the original algorithm.

Table 19 presents the simulation results under random access condition for the simplified approach of this technique independently adapted.

Table 19 Additional results under random access condition

|  | DMVD search range = ±1 |  |  | bilinear filter for DMVD |  |  | No DMVD for 4x4/4x8/8x4PU |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.3 | -1.8 | -1.8 | -1.3 | -1.9 | -1.7 | -1.3 | -1.8 | -1.9 |
| Class B | -1.4 | -1.1 | -1.1 | -1.4 | -1.2 | -1.2 | -1.4 | -1.2 | -1.2 |
| Class C | -1.6 | -1.6 | -1.6 | -1.6 | -1.7 | -1.8 | -1.6 | -1.6 | -1.7 |
| Class D | -1.5 | -1.4 | -1.5 | -1.4 | -1.5 | -1.5 | -1.3 | -1.3 | -1.3 |
| Class E |  |  |  |  |  |  |  |  |  |
| All | -1.4 | -1.4 | -1.5 | -1.4 | -1.5 | -1.5 | -1.4 | -1.5 | -1.5 |
| Enc Time[%] | 140% |  |  | 139% |  |  | 125% |  |  |
| Dec Time[%] | 117% |  |  | 117% |  |  | 122% |  |  |
| worst case of memory access ratio | 23.56 (RMC mode in 4x4PU) |  |  | 32.56 (RMC mode in 4x4PU) |  |  | 15.13 (normal bi-pred in 4x4PU) |  |  |
| number of operations for DMVD | 769.4375 times / pixel |  |  | 250.9375 times / pixel |  |  | 886.9376 times / pixel |  |  |

Each change affects about 0.1% coding efficiency loss, and the change of search range of DMVD and interpolation filtering decrease the decoder runtime. However, the limitation of PU size does not affect the decoder runtime though the worst case of reference memory access decreases.

# Conclusions

In this contribution, the decoder complexity of the technique of bi-predictive motion compensation by single motion vector using decoder-side inter-reference ME is evaluated for the worst case of memory access and block matching operation.

Based on the results of the increase of the decoder complexity, the simplified algorithm is proposed to reduce the decoder complexity.

The simulation results show that the complexity of simplified algorithm is less than the complexity of original algorithm on both runtime complexity measurement and the worst case based complexity measurement with 0.3% coding efficiency loss.

The evaluation of parallelism is needed for more exact measurement of the complexity. Block matching process in proposed technique has the ability of parallelism.

We suggest that the more complexity measurements should be introduced in the various aspects such as the worst case of memory access and additional operation considering the parallelism in addition to runtime measurement.

We recommend investigating the proposed technique and the combination of the proposed technique and other related tools using the complexity measurements in CE activity.

# References

[1] “CE1: Refinement motion compensation using DMVD with merge extension “,

JCTVC-D120, Daegu, January 2011.

[2] “CE3: Experimental results of DCTIF by Samsung”,

JCTVC-D344, Daegu, January 2011.

[3] “WD2: Working Draft 2 of High-Efficiency Video Coding”,

JCTVC-D503, Daegu, January 2011.

[4] “Common test conditions and software reference configurations”,

JCTVC-D600, Daegu, January 2011.

[5] “Improvements of the BD-PSNR model,” ITU-T SG16 Q.6 Document,

VCEG-AI11, Berlin, July 2008.

# Patent rights declaration(s)

JVC KENWOOD Holdings, Inc. and Victor Company of Japan, Limited may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).