| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E217 |
| --- | --- |

| Title: | Improved dQP calculation method |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao Pang, Oscar C. Au, Feng Zou, Jingjing Dai, Xing Wen, Xingyu Zhang and Lu Fang HKUST, Clear Water Bay, KLN, Hong Kong | Tel: Email: | +852-6747-3712 pcece@ust.hk |
| Source: | the Hong Kong University of Science and Technology |  |  |

_____________________________

# Abstract

The concept of Coding Units has been introduced in HEVC, and it contributes a lot to the coding efficiency improvement. In each LCU, all the CUs share the same QP value in current HM. However, since the LCU size is much larger than the MB size in AVC, this QP change might be too coarse for future rate control design. To deal with this problem, the dQP information can be sent at sub-LCU level. In this proposal, the QP prediction method in AVC is extended in two different ways, previous prediction and left prediction. It is reported in the experimental results that the left prediction method outperforms the previous prediction method in terms of the number of non-zero dQP values.

# Problem Statement

In current HM, the delta-QP can be specified only at LCU level. In last meeting, experts expressed that it is a bit too coarse for both subjective quality improvement and future rate control design. In last meeting, it is suggested that the dQP information could be specified for each CU in [1]. In order to make a better QP prediction, in [2], the QP information of the left CU is used for QP prediction in each LCU.

We consider that extension of the QP prediction method in AVC may be a simple and straightforward solution. So in this proposal, the QP prediction method in AVC is extended to the HM in two different ways, and the comparison is made between the two methods.

# Improved dQP calculation

(a) Method 1                   (b) Method 2

Fig. 1 Two QP prediction methods

In AVC, the size of basic coding unit, i.e. MB is fixed, and all MBs are processed in the raster scan order. Therefore, the QP prediction method in AVC can be considered as either the previous prediction or the left prediction.

Then, there are two ways of extending the QP prediction methods in AVC to HM:

- Method1: Use the QP value of the previous CU in the coding order as shown in Fig. 1(a);

- Method2: Use the QP value of the left CU as shown in Fig.1(b). Suppose the position of the upper-left pixel in current CU is (xc, yc), then the left CU is the CU that contains the pixel at the position of (xc-1, yc). With this specification of the left CU, the left prediction can be applied to the current HM where neighboring CUs may have different sizes. Note that the prediction can cross neighbor LCUs

To evaluate its performance, we compare the numbers of non-zero dQP values using these two QP prediction methods. 13 internal test sequences are used experiments, and MB-level subjective quality rate control method is employed. The detailed results are shown in Table 1. Using the QP of left CU as the QP predictor, on average, the number of non-zero dQP values is reduced by 42%.

|  | number of non-zero dQP values using Method 1 | number of non-zero dQP values using Method 2 | reduction of number of non-zero dQP values |
| --- | --- | --- | --- |
| Sequence 1 | 47884 | 19985 | 58.26% |
| Sequence 2 | 129832 | 65092 | 49.86% |
| Sequence 3 | 39401 | 14338 | 63.61% |
| Sequence 4 | 34268 | 26202 | 23.54% |
| Sequence 5 | 242069 | 183295 | 24.28% |
| Sequence 6 | 43892 | 31641 | 27.91% |
| Sequence 7 | 37144 | 24728 | 33.43% |
| Sequence 8 | 67174 | 34018 | 49.36% |
| Sequence 9 | 102506 | 61838 | 39.67% |
| Sequence 10 | 70733 | 42090 | 40.49% |
| Sequence 11 | 67572 | 39803 | 41.10% |
| Sequence 12 | 37571 | 23128 | 38.44% |
| Sequence 13 | 39363 | 14563 | 63.00% |
| Average |  |  | 42.54% |

# Conclusion

In this proposal, the QP prediction method in AVC is extended to HM. However, due to the fixed coding unit size, i.e. macroblock, this extension can have two possibilities: prediction with the left CU and prediction with the previous CU in the coding order. Experimental results show that the prediction with the left CU outperforms the prediction with the previous CU in terms of the number of non-zero dQP values.

# References

[1] JCT-VC, CU adaptive quantization syntax change for better decoder pipelining, JCTVC-D258, 4th JCTVC meeting, Daegu, Jan. 2011

[2] JCT-VC, Delta QP signaling at sub-LCU level, JCTVC-D038, 4th JCTVC meeting, Daegu, Jan. 2011

# Patent rights declaration(s)

The Hong Kong University of Science and Technology may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).