| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E218 |
| --- | --- |

| Title: | CE9: Cross verification of experiments G and H from DOCOMO (JCTVC-E155) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Patrice ONNO Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: 33 (0)2 99 87 68 92 Email: patrice.onno@crf.canon.fr |  |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution is about the cross check of the coding efficiency results regarding the case G and H of CE9 described in section 4.3 of the JCTVC-D609_r3_d2 document. These two experiments related to cases G and H mainly focus on the restriction of the number of top and left candidates to evaluate for AMVP. The results provided by these two experiments G and H are very close. This report confirms the results provided by DOCOMO in document JCTVC-E155 where a BDR gain of around 0.5 % is obtained for the LC RA and HE LD and a BDR loss of 0.8% for the LC LD case. For the HE RA configuration no gain (0%) is reported. Finally regarding the complexity figures, the report confirms that the experiments do not have impact on complexity ranging from 97% to 104%.

# Introduction

The contribution JCTVC-D055 advocates simplification of upper/left spatial motion vector predictor calculation by using up to two neighboring PUs instead of up to 16 PUs as currently defined in the test model, no coding loss has been reported by this simplification. It was recommended to evaluate the impact of this simplification on the coding efficiency and complexity. In experiment G and H of CE9  (Jung, Bross, Chen, & Han), the following two simplifications of upper/left spatial motion vector predictor calculation (only number of PU to be searched will be change) will be evaluated in addition to the modification of the number of candidates for the SKIP mode.

# Principle of the modification

## MVP candidate search in HM2.0

The first modification is about the motion candidate search as described in (Jung, Bross, Chen, & Han). In the HM2.0, as shown in the following figure, the spatial MVP candidates (left, upper) of the current prediction unit (PU) are calculated from the motion vectors of its neighboring PUs as shown in Figure 1.

Figure 1: PU organization in HM2.0

- MVP candidate search from left neighbors:

  - Scanning the left neighbors (PU) from bottom to top (A0 … Am,Am+1; maximum 17 PUs),

  - find first PU that meets the following conditions:

    - It is available

    - It is inter-coded

    - It has a MV for the same direction (L0 or L1)

    - It uses the same reference index

  - Otherwise, left predictor is set to (0,0) and marked unavailable.

- MVP candidate from upper neighbors :

  - Do the same procedure with one following additional condition (as that of left predictor) for the upper

neighbours, scanning from right to left (B-1,B0 … Bn, Bn+1, maximum 18 PUs)

    - It is not same to left predictor

Note that « m » and « n » can be 0 if the current PU has the minimum vector block size allowed by the HEVC standard.

### MVP candidate search in for case G and case H

Figure 2: Case G limitation                                  Figure 3: Case H limitation

#### Case G of CE9

As shown in Figure 2, for the search of left predictor, instead of scanning all the left neighboring PUs from bottom to top, only up to three neighboring PUs, Am+1, Am,A0 are used. Similarly, for the search of top predictor, only up to four neighboring PUs,Bn+1,Bn,B0,B-1 ,are used. Other steps to select MVP candidate are unchanged.

#### Case H of CE9

As shown in Figure 3, left/upper MVP candidate calculation is further simplified. For the search of left predictor, instead of scanning all the left neighboring PUs from bottom to top, only up to two neighboring PUs, Am+1,Am, are used. Similarly, For the search of top predictor, only up to three neighboring PUs, Bn+1,Bn ,B-1　are used. Other steps to select MVP candidate are unchanged.

## Modification of the skip candidates

In G and H cases, for the SKIP mode the same number of candidates is used based on the merge mode as presented in this table.

| Experiment | Inter | MERGE | SKIP |  |  |
| --- | --- | --- | --- | --- | --- |
|  | candidates | candidates | Candidates | ref_idx | inter_pred_idc |
| anchor | AMVP (HM2.0) | MERGE (HM2.0) | AMVP (HM2.0) | 0 (HM2.0) | slice dependent (HM2.0) |
| G | As described in section 2.1.1.1 | HM2.0 | MERGE | derived | derived |
| H | As described in section 2.1.1.2 | HM2.0 | MERGE | derived | derived |

# Results

This section present the results about the modification performed above the HM 2.0 for the two cases G and H considered based on the source code provided by DOCOMO.

## Experimental results for case G.

|  |  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
|  | Class A | 0,2 | -0,1 | -0,2 | -0,1 | -0,3 | -0,3 |
|  | Class B | -0,1 | -0,3 | -0,4 | -0,7 | -0,6 | -0,5 |
|  | Class C | -0,1 | 0,0 | 0,0 | -0,3 | -0,2 | -0,3 |
|  | Class D | -0,1 | 0,0 | 0,0 | -0,3 | -0,3 | 0,0 |
|  | Class E |  |  |  |  |  |  |
|  | All | 0,0 | -0,1 | -0,1 | -0,4 | -0,4 | -0,3 |
|  | Enc Time[%] | 99% |  |  | 97% |  |  |
|  | Dec Time[%] | 103% |  |  | 103% |  |  |
|  |  | Low delay |  |  | Low delay LoCo |  |  |
|  |  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
|  | Class A |  |  |  |  |  |  |
|  | Class B | -0,3 | 0,0 | -0,3 | 1,4 | 0,6 | 0,8 |
|  | Class C | -0,1 | 0,3 | 0,2 | 0,7 | 0,7 | 0,6 |
|  | Class D | -0,3 | 0,3 | 0,1 | 0,4 | 0,3 | -0,2 |
|  | Class E | -1,7 | -1,4 | -0,2 | 0,5 | -0,6 | 0,1 |
|  | All | -0,5 | -0,1 | 0,0 | 0,8 | 0,3 | 0,3 |
|  | Enc Time[%] | 98% |  |  | 99% |  |  |
|  | Dec Time[%] | 104% |  |  | 101% |  |  |

## Experimental results for case H.

|  |  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
|  | Class A | 0,1 | 0,0 | 0,3 | -0,2 | -0,3 | -0,1 |
|  | Class B | -0,1 | -0,2 | -0,3 | -0,6 | -0,6 | -0,6 |
|  | Class C | -0,1 | 0,0 | 0,0 | -0,3 | -0,2 | -0,2 |
|  | Class D | -0,1 | -0,1 | 0,2 | -0,3 | -0,3 | -0,1 |
|  | Class E |  |  |  |  |  |  |
|  | All | 0,0 | -0,1 | 0,0 | -0,4 | -0,4 | -0,3 |
|  | Enc Time[%] | 99% |  |  | 98% |  |  |
|  | Dec Time[%] | 102% |  |  | 103% |  |  |
|  |  | Low delay |  |  | Low delay LoCo |  |  |
|  |  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
|  | Class A |  |  |  |  |  |  |
|  | Class B | -0,3 | 0,1 | -0,1 | 1,4 | 0,8 | 0,8 |
|  | Class C | -0,2 | 0,5 | 0,3 | 0,7 | 0,6 | 0,6 |
|  | Class D | -0,4 | 0,5 | 0,1 | 0,3 | 0,1 | 0,2 |
|  | Class E | -1,5 | -1,1 | -0,4 | 0,6 | -0,6 | -0,1 |
|  | All | -0,5 | 0,1 | 0,0 | 0,8 | 0,3 | 0,4 |
|  | Enc Time[%] | 99% |  |  | 98% |  |  |
|  | Dec Time[%] | 103% |  |  | 101% |  |  |

# Conclusion

This report confirms the results provided by DOCOMO in document JCTVC-E155 where a BDR gain of around 0.5 % is obtained for the LC RA and HE LD and a BDR loss of 0.8% for the LC LD case. For the HE RA configuration no gain (0%) is reported. Finally regarding the complexity figures, the report confirms that the experiments do not have impact on complexity ranging from 97% to 104%.

# Reference

Jung, J., Bross, B., Chen, P., & Han, W.-J. (s.d.). Core Experiment 9: MV Coding and Skip/Merge operations. Joint Collaborative Team on Video Coding (JCT-VC). JCTVC-D609_r3d2.doc.