| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E219 |
| --- | --- |

| Title: | Robust solution for the AMVP parsing issue |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume LAROCHE Christophe GISQUET Patrice ONNO Edouard FRANCOIS Naël OUEDRAOGO Julien RICARD Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | guillaume.laroche@crf.canon.fr christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr edouard.francois@crf.canon.fr nael.ouedraogo@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution deals with the parsing issue related to the temporal motion prediction of AMVP. AMVP candidates consist not only of spatial MVP candidates but also of temporal MVP candidates from previous reference pictures. Before coding the index of the best MVP candidate, redundant MVPs are removed from the candidates set. This might cause parsing errors when previous reference pictures are lost. In this contribution, we propose two tools related to the AMVP process. The first tool consists in forcing the availability of a temporal predictor for the Inter, Skip and Merge modes whatever the existence of this predictor. The second tool consists in replacing the duplicate candidates by candidates with different (non-redundant) values. Thanks to these tools, the parsing issue is fully solved.

These two tools have been evaluated in three configurations. Configuration 1 only uses tool 1. Configurations 2 and 3 additionally use tool 2 and correspond to different modes combinations. Coding performance is respectively of 1.3%, 0.8% and 0.4% BD-rate loss.

# Introduction

## Parsing issue in AMVP predictor

The AMVP scheme for Inter and Skip modes and the Merge mode use several predictors which are competing using the RD criterion. The AMVP scheme uses 3 predictors and the Merge mode 5. These predictors are spatial or temporal. To limit the rate dedicated to the best predictor index signaling, the duplicate candidates are removed from the predictors set in order to produce a reduced predictors set. The index coding depends on the amount of predictors in this set. As shown in Table 1, if the predictor set contains only one predictor, the index of the predictor is inferred so no bit is transmitted to the decoder side.

The parsing issue comes from this suppression process which depends on data which come from another slice. Indeed, if the temporal slice is lost, the decoder cannot apply the suppression algorithm in the same context as the encoder. In that case, the decoder cannot determine if a missing motion vector predictor was equal to at least one of the other motion vector predictor or not.

|  | Amount of predictors N in the set |  |  |
| --- | --- | --- | --- |
| Index | N=1 | N=2 | N=3 |
| 0 | (inferred) | 0 | 0 |
| 1 |  | 1 | 10 |
| 2 |  |  | 11 |

Table 1: Code word for each predictors index when the amount of predictors in the set is N.

## Previous proposed solutions

Several solutions which solve or control this issue were already submitted to the group. In [1, 2], it was proposed to disable the temporal predictor for some frames in order to obtain re-synchronization on some frames in case of loss. This solution may have a lower impact on the coding efficiency than removing it for all frames. But these solutions don’t solve the parsing issue. In [3], 4 basic solutions were described; the best one was the ‘Full idx’ which corresponds to a unary code of the predictor index except when the index of the predictor is the maximum index. At least one bit needs to be transmitted in the bitstream. When combined with efficient derivation process of the temporal predictor [4] interesting results are obtained. But to fully solve the parsing issue, this proposal needs also to consider the Merge candidate selection.

## Our proposal

In this contribution, we propose two tools related to AMVP to solve the parsing issue:

- The first tool relates to the temporal predictor derivation, and consists in forcing the availability of a temporal predictor for the Inter, Skip and Merge modes irrespectively of the existence of this predictor. In case of non-availability of the collocated temporal predictor, neighboring corner temporal blocks are considered. Moreover, the predictor index coding depends on amount of spatial predictors before the suppression process.

- The second tool relates to the processing of redundant candidates. Instead of removing redundant candidates, we propose to replace them by new (non-redundant) candidates.

These two tools are evaluated using different combinations in three configurations:

- Configuration 1 uses tool 1 only, related to the forcing the systematic use of a temporal predictor.

- Configurations 2 and 3 additionally use tool 2, related to the replacement of the duplicate candidates by candidates with different values.

The document is organized as follows. Section 2 describes in more details the two tools that we are proposing. Next section describes the evaluation of these two tools using three different combinations. Parsing issues are commented in section 4. Finally conclusions are given in section 5.

# Proposed AMVP modifications

## Tool 1: Systematic use of temporal predictor

The first proposed tool relates to the temporal motion vector derivation. In our proposal, the temporal predictor is enforced to be available for the Inter, Skip and Merge modes whatever the existence of this predictor. The AMVP candidates set remains adaptive, but a temporal candidate is always inserted. To improve the efficiency when this temporal predictor is not available, the value of the temporal predictor can be changed by the value of another temporal motion vector. So if the collocated block is not available, the availability of its neighboring corner blocks is checked according to the following order:

- Bottom right corner

- Bottom left corner

- Top right corner

- Top left corner

The positions of these four motion vectors are depicted in Figure 1. The value of the first available corner motion vector is considered as the temporal predictor. If no corner is available, the value of the temporal predictor is set equal to zero.

Figure 1: Corner positions for temporal predictor derivation process.

The spatial derivation process is similar to HM2.0 (JCTVC-D231, [3]). The only difference is that the redundant vectors removal of these spatial predictors is not used. Indeed, the amount of predictors in the set cannot depend on the value of the predictors to solve the parsing issue.  As in the other configurations, at the end of this derivation process, the predictors set can contain 1, 2 or 3 predictors. The index coding of the best predictor is based on this amount of predictors. So we used exactly the same conversion table as in the current implementation, as shown in Table 1. The proposed derivation process for the temporal predictor is summarized by the following pseudo-code:

Identify colPu

if ( colPu is not intra )

mvCol = mv of colPu

else {

Identify colBottomRightPu

if ( colBottomRightPu is not intra )

mvCol = mv of colBottomRightPu

else {

Identify colBottomLeftPu

if ( colBottomLeftPu is not intra )

mvCol = mv of colBottomLeftPu

else {

Identify colTopRightPu

if ( colTopRightPu is not intra )

mvCol = mv of colTopRightPu

else {

Identify colTopLeftPu

if ( colTopLeftPu is not intra )

mvCol = mv of colTopLeftPu

else mvCol = (0,0)

}

}

}

}

## Tool 2: Replacement of redundant candidates

For the spatial predictors, a first spatial derivation process similar to the current implementation is used. This process is based on the availability of the neighboring motion vectors and on their reference frame and list indexes. This first derivation process is used to determine the amount of predictors in the set as Tool 1. However in this proposal, duplicate candidates are replaced by candidates with different values. This offers the possibility to efficiently use the signaling bits required to be transmitted to solve the parsing issue and which may cause coding efficiency loss.

The modification of the derivation process applies to the spatial predictors, as explained below.

For the top and the left spatial predictors, the derivation process starts with the same derivation as in HM2.0 (JCTVC-D231, [3]). If a spatial predictor ‘exists’ and has the same value as another predictor, its collocated motion vector is checked (it means that the value of the current spatial vector is changed by the value of the motion vector at the same position as this spatial block in the previous frame).

Figure 2 illustrates this derivation process. In this example, the predictor selected for respectively the top predictor and the left predictor are T2 and L1. If one or two of these predictors are redundant, their values are replaced respectively by the value CT2 and CL1. For a spatial predictor, if this collocated block does not exist or if its value (after scaling if needed), is equal to another predictor in the set, other spatial predictors are checked. The spatial neighboring motion vectors are tested in the same order as done in the current implementation. The main difference with the classical spatial derivation process is that their values are scaled if needed. If no considered spatial motion vector gives a non-duplicate predictor, an offset is added to the first predictor value in order to obtain a non-duplicate candidate.

Figure 2: replacement of duplicate motion vector candidates.

At the end of this derivation process, the predictors set can contain 1, 2 or 3 predictors which are not redundant. The index coding of the best predictor is based on this amount of predictors. So we used exactly the same conversion table as in the current implementation, as shown in Table 1.

The proposed derivation process for non redundant candidates is summarized in the following pseudo code for the above spatial predictor:

Identify ListOfAboveCand

mvAbove = FindAvailableMV ( ListOfAboveMV )

if ( Duplicate ( mvAbove ) )

{

Identify colAbovePu

if( colAbovePu is not intra )

mvAbove = mv of colAbovePu

if ( Duplicate ( mvAbove ) )

{

i=0

while ( Duplicate ( mvAbove ) or i > Card( ListOfAboveMV ) )

{

mvAbove = Find( ListOfAboveMV,i )

Scale( mvAbove )

i++

}

If ( Duplicate( mvAbove ) )

{

AddOffset ( mvAbove )

}

}

}

# Evaluated configurations

These two tools have been evaluated in three different combinations. For the three considered configurations, the coding of the predictors index depends only on data of current slice which can’t be modified by the data of other slices. In other words, the number of predictors only depends on the spatial predictors selected during the first derivation process. Please note that for these three configurations, we disabled the temporal predictor when the collocated frame is an Intra frame. This is signaled by a flag in the slice header (indeed, this can’t be inferred when this previous Intra frame is lost).

The three configurations depend on the usage or not of the two tools, and on their usage in Inter, Skip and Merge modes. The Table 2 summarizes the main differences between these three configurations. In all cases the tool 1 ‘systematic temporal predictor derivation’ is applied. Indeed this is required to solve the parsing issue.

| Configuration | Tool 1: Forced temporal | Tool 2: Redundant candidates replacement |  |
| --- | --- | --- | --- |
|  |  | Inter and Skip | Merge |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |

Table 2: overview of the three proposed configurations.

## Configuration 1: tool 1 only

In this first configuration, only the systematic temporal predictor derivation is applied.  As in the other configurations, at the end of the first spatial derivation process, the predictors set can contain 1, 2 or 3 predictors.

For the Merge mode, the spatial derivation process is similar to HM2.0 but the duplicate candidates are not removed from the set. However, at encoder side, the duplicate candidates of the Merge are not checked by the RD criterion. Thus only the candidate index coding differs from HM2.0.

The results of this first configuration, using the common test conditions [4], are provided in Table 3. In average for the four configurations the bitrate increase is of 1.3%. The impact on coding efficiency is lower for Random Access cases with 0.7% and 0.9% bitrate increase. For Low delay case the impact is mainly due to the Class E. Without class E, the average bitrate increase is 1%. The impact on complexity is low, with in average 101% of the anchor runtime at encoder side and 100% at decoder side.

|  |  |  | Random access |  |  |  |  |  | Random access LoCo |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y BD-rate |  | U BD-rate |  | V BD-rate |  | Y BD-rate |  | U BD-rate |  | V BD-rate |
| Class A | 0.5 |  | 0.4 |  | 0.2 |  | 0.6 |  | 0.1 |  | 0.0 |  |
| Class B | 0.7 |  | 0.5 |  | 0.4 |  | 1.1 |  | 0.6 |  | 0.6 |  |
| Class C | 0.7 |  | 0.5 |  | 0.6 |  | 0.9 |  | 0.6 |  | 0.6 |  |
| Class D | 0.8 |  | 0.7 |  | 0.6 |  | 1.0 |  | 0.6 |  | 0.7 |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |
| All | 0.7 |  | 0.5 |  | 0.4 |  | 0.9 |  | 0.5 |  | 0.5 |  |
| Enc Time[%] | 101% |  |  |  |  |  | 102% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  |  | 101% |  |  |  |  |  |
|  | Low delay |  |  |  |  |  | Low delay LoCo |  |  |  |  |  |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate |  | Y BD-rate |  | U BD-rate |  | V BD-rate |  |
| Class A |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 1.2 |  | 1.0 |  | 0.9 |  | 1.6 |  | 0.6 |  | 0.8 |  |
| Class C | 1.1 |  | 1.0 |  | 1.0 |  | 1.2 |  | 0.6 |  | 0.4 |  |
| Class D | 1.2 |  | 0.8 |  | 0.6 |  | 1.3 |  | 0.4 |  | 0.2 |  |
| Class E | 2.2 |  | 1.8 |  | 1.9 |  | 5.8 |  | 4.2 |  | 4.6 |  |
| All | 1.4 |  | 1.1 |  | 1.0 |  | 2.2 |  | 1.2 |  | 1.3 |  |
| All wo Class E | 1.1 |  | 1.0 |  | 0.9 |  | 1.4 |  | 0.7 |  | 0.7 |  |
| Enc Time[%] | 100% |  |  |  |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  | 99% |  |  |  |  |  |

Table 3: Results of configuration 1 (tool 1 only).

## Configuration 2: tool 1 and tool 2 for AMVP only

The second configuration combines the systematic temporal predictor derivation and applies the redundant candidates replacement for Inter and Skip modes only.

In configuration 2, the merge mode applies the same process as configuration 1.

The results of this second configuration are provided in Table 4. In average for the four configurations the bitrate increase is 0.8% (0.6% without class E). The impact on coding efficiency is lower for RA cases with 0.5% bitrate increase. The impact on complexity is in average 104% at encoder side and 102% at decoder side.

|  |  |  |  | Random access |  |  |  |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate |  | Y BD-rate |  | U BD-rate |  | V BD-rate |  |
| Class A |  | 0.3 |  | 0.2 |  | 0.0 |  | 0.0 |  | -0.1 |  | -0.3 |
| Class B |  | 0.5 |  | 0.3 |  | 0.3 |  | 0.7 |  | 0.4 |  | 0.4 |
| Class C |  | 0.5 |  | 0.4 |  | 0.4 |  | 0.5 |  | 0.3 |  | 0.3 |
| Class D |  | 0.7 |  | 0.4 |  | 0.4 |  | 0.7 |  | 0.2 |  | 0.3 |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |
| All |  | 0.5 |  | 0.3 |  | 0.2 |  | 0.5 |  | 0.2 |  | 0.2 |
| Enc Time[%] |  | 104% |  |  |  |  |  | 106% |  |  |  |  |
| Dec Time[%] |  | 102% |  |  |  |  |  | 103% |  |  |  |  |
|  |  | Low delay |  |  |  |  |  | Low delay LoCo |  |  |  |  |
|  |  | Y BD-rate |  | U BD-rate |  | V BD-rate |  | Y BD-rate |  | U BD-rate |  | V BD-rate |
| Class A |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  | 1.0 |  | 0.6 |  | 0.3 |  | 0.5 |  | 0.2 |  | 0.5 |
| Class C |  | 0.7 |  | 0.6 |  | 0.6 |  | 0.4 |  | 0.0 |  | 0.0 |
| Class D |  | 0.8 |  | 0.8 |  | 0.5 |  | 0.7 |  | 0.4 |  | 0.4 |
| Class E |  | 1.8 |  | 1.0 |  | 1.6 |  | 4.3 |  | 3.6 |  | 4.0 |
| All |  | 1.0 |  | 0.7 |  | 0.7 |  | 1.2 |  | 0.8 |  | 1.0 |
| All wo Class E |  | 0.8 |  | 0.6 |  | 0.9 |  | 0.5 |  | 0.2 |  | 0.3 |
| Enc Time[%] |  | 102% |  |  |  |  |  | 104% |  |  |  |  |
| Dec Time[%] |  | 101% |  |  |  |  |  | 103% |  |  |  |  |

Table 4: Results of configuration 2 (tool 1 + tool 2 for Inter and Skip).

## Configuration 3: tool 1 and tool 2 for AMVP and Merge mode

The third configuration combines the systematic temporal predictor derivation and applies the redundant candidates replacement for Inter, Skip modes and also to the Merge mode.

In the merge mode, a predictor is considered as duplicate if it has the same direction, the same reference indexes and the same values of its motion vectors than another predictor. So, to avoid parsing issue, only the values of the motion vectors of the duplicated candidates are changed. Indeed, if the reference frame indexes and the direction of a candidate are modified, the merge mode could propagate wrong reference frame indexes when a slice loss occurs because a wrong duplicate candidate can be found. These indexes are used to compute the amount of predictors in the spatial set for Skip and Inter modes; consequently a wrong amount of predictors could be found and cause a de-synchronization of the bitstream parsing.

The results on this third configuration are provided in Table 5. In average for the four configurations the bitrate increase is of 0.4% (0.2% without Class E). The impact on complexity is in average 109% at encoder side and 105% at decoder side.

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.1 | -0.1 | -0.3 | -0.4 | -0.3 |
| Class B | 0.2 | 0.1 | 0.1 | 0.3 | 0.2 | 0.2 |
| Class C | 0.2 | 0.0 | -0.1 | 0.3 | 0.0 | 0.0 |
| Class D | 0.4 | 0.2 | 0.2 | 0.5 | 0.2 | 0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.2 | 0.1 | 0.0 | 0.2 | 0.0 | 0.0 |
| Enc Time[%] | 110% |  |  | 110% |  |  |
| Dec Time[%] | 104% |  |  | 107% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.4 | 0.2 | 0.0 | -0.1 | -0.3 | 0.1 |
| Class C | 0.4 | 0.3 | -0.1 | 0.0 | -0.2 | -0.4 |
| Class D | 0.5 | 0.4 | 0.3 | 0.3 | -0.3 | -0.3 |
| Class E | 1.3 | 1.0 | 1.8 | 3.8 | 3.2 | 4.2 |
| All | 0.6 | 0.4 | 0.4 | 0.8 | 0.4 | 0.7 |
| All wo Class E | 0.3 | 0.2 | -0.1 | 0.0 | -0.3 | -0.1 |
| Enc Time[%] | 107% |  |  | 107% |  |  |
| Dec Time[%] | 104% |  |  | 105% |  |  |

Table 5: Results of configuration 3 (tool 1 + tool 2 for Inter, Skip and Merge).

## Summary and comments

The results are summarized in Table 6. Regarding the coding performance, the following comments can be derived from these results:

- The proposed configurations generate a BD-rate loss from 1.3% to 0.4% compared to the anchor.   Configuration 1 has similar complexity as the anchor. Configurations 2 and 3 require slightly added complexity at the encoder and decoder sides. At the encoder side, the added complexity comes from the testing of all the candidate motion vectors, for Skip and Inter modes in configuration 2, for Skip, Inter and Merge modes in configuration 3. Regarding the decoder, the current implementation is definitely not optimized because it uses the same common implementation as the encoder. However at the decoder side, the derivation process can be much more straightforward. For instance, if candidate 0 is selected, it is not needed to check any spatial candidate. In the current decoder implementation, this checking is nevertheless performed.

- Even if the current settings regarding the number of predictors have been made to reach a good trade-off coding efficiency/complexity, possible improvements can be considered. First the number of predictors can be dependent on the mode. Secondly the derivation process of the non-duplicate candidates can possibly be improved, by testing other candidates, or in a different order.

The proposed solutions are all fully solving the parsing issue of temporal predictor in the AMVP process, for both AMVP (Skip and Inter modes) and Merge mode.

It can be also noticed that the proposed solutions reduce the parsing complexity of the AMVP process. Indeed the index coding depends only on testing the motion vector availability, the reference frame indexes and the direction of the spatial predictors, instead of depending on the value of the predictors. Motion vectors equality tests are no more required to parse the motion vector prediction data.

| Configuration | Tool 1 Forced temporal | Tool 2 Redundant cands replacement |  | Average BD-rate | Average BD-rate-{Class E} | Enc time variation | Dec time variation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Inter + Skip | Merge |  |  |  |  |
| 1 | X |  |  | 1.3% | 1.0% | 101% | 100% |
| 2 | X | X |  | 0.8% | 0.6% | 104% | 102% |
| 3 | X | X | X | 0.4% | 0.2% | 109% | 105% |

Table 6: configurations description and performance summarizing.

The results related to the configuration 1 and 3 presented in Table 6 have been cross-checked by INRIA in document JCTVC-E261.

# Additional results with Motion Memory Compression

This section report the results of the configuration 1 and 3 combined with the improvements on motion vector memory compression described in JCTVC-E221. For these test, we used a motion vector memory compression ratio equal to 52x which gives an average coding gain of 0.5%.

The results are summarized in Error: Reference source not found. For the configuration 1, the average bitrate increase is of 0.9%, 0.6% without Class E. And for the configuration 3, in average, this combination gives no coding efficiency loss, and even a slight gain without class E, compared to the HM2.0 Anchors which can’t be parsed in case of loss.

| Configuration | Tool 1 Forced temporal | Tool 2 Redundant cands replacement |  | Average BD-rate | Average BD-rate-{Class E} | Enc time variation | Dec time variation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Inter + Skip | Merge |  |  |  |  |
| 1 | X |  |  | 0.9% | 0.6% | 99% | 98% |
| - | - | - | - | - | - | - | - |
| 3 | X | X | X | 0.0% | -0.2% | 109% | 105% |

Table 7: Configurations description and performance summarizing.

# Parsing Check

In order to be sure that the proposed configurations solve entirely the parsing issue, we have simulated losses at decoder side. In this implementation, for the decoding of each frame of the sequence, all its reference frames are re-initialized. It means that all data for all CU of the previous frame are initialized (CU partitioning, mode, reference frames indexes, directions, motion vectors, etc.). Please note that the slice header is also lost except the POC number. Indeed, the same parsing issue when the POC of the reference frame is loss occurs in HEVC as H.264/AVC. In an error resilient implementation, this issue can be easy solving.

For the three tested configurations and each test case (RAHE, RALC, LDHE, LDLC), the decoder did not crash which demonstrates that our proposed solutions fully solve the parsing issue of AMVP and Merge mode.

The Parsing Check has been also cross-checked by INRIA in document JCTVC-E261.

# Conclusion

The proposed solutions are all fully solving the parsing issue of temporal predictor for both AMVP and Merge modes. Two tools are considered: tool 1 achieves the systematic usage of a temporal predictor in the candidates set, tool 2 performs the replacement of redundant candidates by alternate and non-redundant candidates.

They have been evaluated using different combinations in three configurations. The three configurations show a limited coding efficiency impact (from 1.3% to 0.4%) compared to anchor, while solving the parsing issue. When the proposed tool 1 and 2 are combined with the improvement on motion vectors memory compression presented in JCTVC-E221, same coding efficiency than the current HM2.0 Anchor is reported.

The first solution allows the usage of temporal predictor without added complexity. Therefore we suggest adopting this tool in the HM design.

The second solution brings additional coding performance, with different complexity levels. We propose to further study this tool in the AhG on Parsing robustness or in a related core experiment in order to find the best tradeoff between coding efficiency and complexity.

# References

[1]	Bin Li, Jizheng Xu, Feng Wu, Houqiang Li, “Constrained temporal motion vector prediction for error resilience”, JCTVC-D139, Daegu, KR, 20-28 January, 2011.

[2]	Jian-Liang Lin, Yu-Wen Huang, Chih-Ming Fu, Ching-Yeh Chen, Yu-Pao Tsai, Shawmin Lei, “Syntax for AMVP Parsing Error Control”, JCTVC-D126, Daegu, KR, 20-28 January, 2011.

[3] J. Jung, G. Clare, “Proposition for robust parsing with temporal predictor”, JCTVC-D197, Daegu, KR, 20-28 January, 2011.

[4] J. Jung, G. Clare, “Temporal MV predictor modification for MV-Comp, Skip, Direct and Merge schemes”, JCTVC-D164, Daegu, KR, 20-28 January, 2011.

[5] Akira Fujibayashi, Frank Bossen, " CE9 3.2d Simplified Motion vector prediction," JCTVC-D231, 4th Meeting: Daegu, KR, 20-28 January, 2011

[6] Frank Bossen, "Common test conditions and software reference configurations," JCTVC-D600 / m19497, 4th Meeting: Daegu, KR, 20-28 January, 2011.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).