| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E220r2 |
| --- | --- |

| Title: | Preliminary Implementation on Sub-LCU-Level DeltaQP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazushi Sato W12F 1-11-1 GCO Osaki, Shinagawa, 141-0032 Tokyo, Japan Jun Xu 1730 N 1st Street San Jose, CA 95112 | Tel: Email: | +81-3-5435-3308 Kazushi.Sato@jp.sony.com +1-408- 352-4710 Jun.Xu@am.sony.com |
| Source: | Sony Corp. / Sony Electronics Inc. |  |  |

_____________________________

# Abstract

At JCTVC-4th mtg, we proposed the chance in dQP syntax for decoder pipelining with D-258, that was adopted and software has already been provided to the HM software coordinator.

In addition, we proposed Sub-LCU-level dQP coding with D-308, which is one of the topics for discussion in Quantization AHG.

This contribution reports preliminary implementation of Sub-LCU-Level dQP with HM2.0 + D-258 implementation. Source code is attached with this contribution.

# Introduction

At JCTVC-4th mtg, we proposed the chance in dQP syntax for decoder pipelining [1]. This proposal has been adopted. Software based on HM2.0 has been implemented by the second author of this contribution and it has been already provided to the HM software coordinator for HM2.1 implementation.

Furthermore we proposed Sub-LCU-Level dQP coding for subjective quality improvement [2]. TI also had same proposal [3] and this functionality is supported by as many as 6 companies / organizations [3].

This document is description on the preliminary implementation of Sub-LCU-Level dQP coding based on the software of D-258.

# Software Description

Implementation of D-258 is still LCU-level. Software attached with this contribution is its modification so that information on dQP can be transmitted with sub-LCU level.

In each of the configuration file the following options can be found:

MaxDeltaQP                    : 0           # CU-based multi-QP optimization

To test dQP coding either with LCU-Level or Sub-LCU-Level, the value of “MaxDeltaQP” should be >=1. With this setting the following definitions will be enabled.

In “TLibCommon/TypeDef.h” the following portions can be found:

#define SNY_DQP                          1           ///< SONY's proposal on syntax change of dQP (JCT-VC D258)

#define SNY_SLL_DQP                      1           ///< Sub-LCU-Levl dQP

#define SNY_SLL_DQP_TEST              1           ///< Test : Set all dQP=0

#define ACTBLOCKSIZE                    16           ///< BlockSize for Activity Calc.

The first definition “SNY_DQP” is a switch to turn on/off modification of D-258.

The second definition “SNY_SLL_DQP” is a switch to turn on/off modification of Sub-LCU-Level dQP.

The third definition “SNY_SLL_DQP_TEST” is the one for the experiment described in the next section of this contribution. If it is turned on, all dQP is set as 0.

The fourth definition “ACTBLOCKSIZE” is the unit for calculation of activity and for dQP coding.

Although there might be possible ways for dQP coding method it is implemented as

dQP =CurrQP – PrevQP

in the current implementation. This is the consensus among the members of Quantization AHG members interested in this topic. Methods for improvement like the following will be investigated:

dQP=CurrQP – Median (QPa, QPb, QPc)

where QPa, QPb, QPc are the QP values for left, upper and upper-left blocks. Alternative method is to send a flag to indicate which QP of a neighboring block is used for QP prediction.

TM-5 adaptive quantization for subjective quality improvement, already mentioned in [2] and has been implemented on TMuC0.9 by the first author of this document, is now being ported to the software of this contribution.

# Simulation

## Simulation Condition

When Sub-LCU-Level dQP was first proposed [5], there has been comments as follows:

- How much is the impact on coding efficiency with all dQP=0, compared with the one coded only at LCU-level?

- How does it work if combined with adaptive quantization?

In this section simulation results are shown to provide information on the question 1.

Both with “Anchor” and “Tested”, the value of  “MaxDeltaQP” is set as 1.

Settings in “TypeDef.h” are

#define SNY_DQP                          1

#define SNY_SLL_DQP                      0

#define SNY_SLL_DQP_TEST              1

for the anchor [LCU-Level dQP] and

#define SNY_DQP                          1

#define SNY_SLL_DQP                      1

#define SNY_SLL_DQP_TEST              1

#define ACTBLOCKSIZE                    16

for the tested [Sub-LCU-Level dQP].

We believe practically that it is OK that dQP should be transmitted with CUs larger than or equal to 16x16 and need not to be transmitted with 8x8 CUs.

Other settings are same as the one described in [6]. Only one frame with each sequence and each QP has been tested with High Efficiency condition.

## Simulation Result

Summary of results are shown in Table 3.2.1. Complete results are shown in “JCTVC-E220r1.xls”.

This result has been verified by Canon [7].

As can be seen no loss, sometimes even gain, can be observed with High Efficiency cases. This is because in CABAC case, coding efficiency will be improved if same symbol continues.

Table 3.2.1

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.6 | 0.1 |
| Class B | 0.0 | 0.0 | -0.2 |
| Class C | -0.1 | 0.5 | 0.0 |
| Class D | 0.0 | -0.2 | 0.0 |
| Class E | 0.0 | -1.0 | -1.3 |
| All | 0.0 | -0.2 | -0.2 |

# Conclusion

This contribution is introduction on the preliminary implementation on Sub-LCU-Level based on D-258 implementation, which has already been adopted and will be integrated into HM2.1.

Simulation result shows that no loss in coding efficiency is observed with Sub-LCU-Level dQP compared with LCU-Level dQP with high-efficiency-intra conditions.

We recommend this software be the basis for Sub-LCU-Level dQP investigation for HEVC.

In Common test condition [6], the value of QP remains same within a frame, but with this setting methods related to dQP coding cannot be evaluated. There should be consensus for evaluation of dQP-coding methods like applying adaptive quantization in TM-5, as mentioned in [2]. We will be able to provide software with TM-5 step3 functionality.

# References

- “CU Adaptive Quantization Syntax Change for Better Decoder pipelining”, JCTVC-D258, Jan 2011

- “On LBS and Quantization”, JCTVC-D308, Jan 2011

- “Delta QP signaling at sub-LCU level”, JCTVC-D038, Jan 2011

- “Support for Sub-LCU-Level QP in HEVC”, JCTVC-E202, Mar 2011

- “Proposal on Large Block Structure and Quantization”, JCTVC-C167, Oct 2010

- ” Common test conditions and software reference configurations”, JCTVC-D600, Jan 2011

- “Verification result of Sony's Preliminary Implementation on Sub-LCU-Level DeltaQP (JCTVC-E220)”, JCTVC-E486, Mar 2011

# Patent rights declaration(s)

Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).