| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E221 |
| --- | --- |

| Title: | On memory compression for motion vector prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard FRANCOIS Christophe GISQUET Guillaume LAROCHE Patrice ONNO Naël OUEDRAOGO Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | edouard.francois@crf.canon.fr christophe.gisquet@crf.canon.fr guillaume.laroche@crf.canon.fr patrice.onno@crf.canon.fr nael.ouedraogo@crf.canon.fr |  |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution presents some modifications of the motion vector memory compression algorithm proposed in JCTVC-D072 which was adopted during the last meeting. The motion vector memory compression reduces the size of the motion vector buffer to store temporal collocated vectors. The current implementation stores the motion vector data from previous frames at lower resolution by summarizing a block of 16 motion vectors by the top left motion vectors of this block. The first proposed modification consists in using another motion vector for the summarization. The second proposed modification reduces the amount of bits needed to store the different motion data related to the collocated temporal predictor. The proposed modifications give 0.5% bitrate savings in average for the four configurations with 52x motion memory compression ratio compared to storing the entire motion vectors field.

# Introduction

The motion vector memory compression was introduced in [1] and [2] in order to reduce the significant size of the temporal motion vectors buffer and the associated memory access bandwidth. The current implementation stores the motion vector data from previous frames at lower resolution by summarizing a block of 16 motion vectors by the top left motion vectors of this block as depicted in Figure 1Error: Reference source not found.

Figure 1: HM2.0 motion vector summarization.

However, the derivation process of the collocated motion vector also depends on other data such as the reference frame index, the list index and the intra/inter mode. Consequently, if we consider that 12 bits are used for each motion vector component (as used in level 3x, 4x and 5x of H.264/AVC) and that a maximum of 4 reference frames could be used in each list, 55 bits need to be stored for each block of a B Slice as depicted in Table 1. Without motion vector memory compression, the buffer needed for the derivation of the temporal predictor is approximately 30Mbits/frame for 4Kx2K video sequence (30Mbits/frame ≈ ((4096x2048)/(4x4))*55).

| L0 |  | L1 |  |
| --- | --- | --- | --- |
| Mvx | 12 bits | Mvx | 12 bits |
| Mvy | 12 bits | Mvy | 12 bits |
| Ref idx | 2 bits | Ref idx | 2 bits |
| List idx | 1 bit | List idx | 1 bit |
| Inter/Intra mode |  |  |  |
| 1 bit |  |  |  |
| Total |  |  |  |
| 55 bits |  |  |  |

Table 1: Number of bits for storing each collocated vector and the associated coding information.

In this proposal, we have identified an issue on current implementation, and we propose three improvements of the current motion vector memory compression in order to increase the motion vectors compression ratio while increasing the global coding efficiency.

- The first improvement consists in changing the block used for the summarization process.

- The second improvement consists in reducing the number of bits to store the motion vector components. Our proposal suggests using 8 bits instead of 12 bits.

- Finally, the third improvement concerns the removal of the reference frames indexes, the list indexes buffer and one list of motion vectors in order to increase the compression ratio.

# Issue related to reference/list indexes and mode in motion memory compression

In the current HM2.0 implementation of the motion vector memory compression, only the motion vectors are summarized. As illustrated in Figure 2, the reference/list indexes or the mode index are still stored at the ‘full’ 4x4 blocks level. In the temporal motion vectors derivation, some summarized motion vectors can be assigned to blocks that have actually a different reference or a different mode or a different list. Consequently, some temporal motion vectors are not correctly derived. Moreover, if we consider all data used for the derivation process of the temporal predictor which needs to be compressed, the compression ratio of the HM2.0 anchor is 5.5x as depicted in Figure 2. Indeed, for 16 blocks, 96 bits need to be stored for reference frames and list indexes, and 16 for the mode. So, with the motion vector information 160 bits are stored.

To solve this issue, it is proposed to apply the summarization process to all data needed for the derivation process of the temporal predictor, as illustrated in Figure 3. In that case, the compression ratio of all data used in the derivation process is 16x

Figure 2: HM2.0 motion vector, reference/list index and mode memory.

Figure 3: Full summarization on motion vector, reference/list index and mode memory.

.

## The results of this modification (using test conditions of JCTVC-D600 [3]) are reported in Table 2, using the Top left block and a compression ratio of 16x. This modification has a slightly positive impact on coding efficiency for the Low Delay configuration, and no difference is noticed for Random Access configuration.

|  | Compression ratio | RAHE | RALC | LDHE | LDLC |
| --- | --- | --- | --- | --- | --- |
| H2.0 anchors | 5.5x | 0.0 | 0.0 | 0.0 | 0.0 |
| Full summarization | 16x | 0.0 | 0.0 | -0.1 | -0.1 |

Table 1: Coding performance results when integrating the summarization process for all data.

# Block summarization candidates

For this modification, we propose to use the bottom right block for the summarization instead of using the top left block as depicted in Figure 4Error: Reference source not found.

Figure 4: Motion vector summarization based on bottom right block.

The bottom right block offers two advantages regarding the temporal motion predictor derivation:

- For CUs of small size, the bottom right vector candidate offers a larger diversity compared to the spatial motion vector candidates, because it is more distant from these spatial vectors.

- For larger CUs, the bottom right vector is actually closer to the ‘real’ collocated centered block (that is, when no mv compression is applied) used as temporal motion predictor, as illustrated in Figure 5 for a 32x32 CU (8x8 MV data blocks) and a MV memory compression ratio of 16.

Figure 5: Derivation of temporal MV predictor for different memory compression algorithms.

Table 2 shows the results of this modification for compression ratios of 4x and 16x (16x is the current HM2.0 compression ratio). For all configurations, the bottom right block summarization outperforms the top left block summarization. For the 16x compression, the average bitrate savings is 0.6%. For 4x compression ratio the bottom right block summarization has the same bitrate savings as when the motion vector memory compression isn’t used (cf. Section 9)

| Compression ratio | 4x |  |  |  |  | 16x |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RAHE | RALC | LDHE | LDLC | AVG | RAHE | RALC | LDHE | LDLC | AVG |
| Top Left | -0.6 | -0.5 | -0.8 | -0.8 | -0.7 | 0.0 | 0.0 | -0.1 | -0.1 | 0.0 |
| Bottom Right | -0.6 | -0.7 | -0.9 | -0.9 | -0.8 | -0.5 | -0.5 | -0.6 | -0.6 | -0.6 |

Table 2: Coding performance results between the Top Left block and the Bottom Right block.

# Increase of the motion vector memory compression ratio

Our proposal is based on the following improvements:

## Motion vectors clipping

To further compress the collocated motion vectors in memory, we propose to reduce the number of bits needed to store each component of each motion vector. In our proposal each collocated motion vector component is clipped to 8 bits. This clipping process is depicted in Figure 9 in case of 12 bits storage per motion component. Please note that we chose a particular representation of integers that may not correspond to the physical one (known as 2-complement arithmetic), but that should ease understanding without invalidating an actual implementation. In any case, the clipping corresponds to the Clip3 process described in H.264/AVC.

Figure 6: MV clipping by removing the 4 most significant bits.

With this simple clipping, the maximum range of a motion vector without sub-pel accuracy is 64. When the temporal distance between the current frame and its nearest reference frame is small, this clipping has no impact on coding efficiency.

However, when the temporal distance between the current frame and its nearest reference frame increases, especially for the hierarchical coding structure used in the Random Access configuration, the absolute motion vector value could overrun 32. So, for higher temporal distance value between frames, we defined a particular clipping where the ¼-pel or the sub-pel accuracy is removed as depicted in Figure 10. For example, if the ¼-pel accuracy is removed, the maximum range is 128. The number of bits removed from the least significant bits can be specified in the picture parameter set or inferred based on the POC difference between the current frame and its nearest reference frame. In our current implementation, only the ¼-pel accuracy is removed for each hierarchical B frame. We expect that the sub-pel accuracy needs to be removed when the temporal distance increases.

It is important to note that this modification has been performed only for the motion vector temporal predictor storage. Motion vectors are still encoded with the sub-pel accuracy as done in the current HM 2.0.

Figure 7: MV clipping by removing the 3 most significant bits and the least significant bit.

The storage of the motion vector component on 8 bits instead of 12 bits reduces the number of bits needed for one block. In that case, only 39 bits are stored instead of 55 bits compared to the full summarization presented in section 2. It offers a compression ratio equal to 22.6x instead of 16x with the current block summarization of 16 blocks in 1 block.

The results of this motion vector clipping are summarized in the following in Table 3. Compared to the results presented in Table 2, the same coding gain is obtained for the Low delay case and additional bitrates savings are reported for Random Access cases. These additional coding gains come from the removal of the ¼-pel accuracy.

| Compression ratio | 22.6x |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | RAHE | RALC | LDHE | LDLC | AVG |
| Bottom right + MV clipping | -0.7 | -0.6 | -0.7 | -0.6 | -0.7 |

Table 3: Coding performance results of the motion vector clipping combined with other modifications described in the previous sections.

## Motion vectors scaling in one single list

To avoid the storage of the reference frame index, among 4 possible reference frames in current test conditions, the motion vectors field is scaled to the nearest reference frame. In that case, all the motion vectors point to one unique reference frame.

Moreover, to avoid the storage of the list index and to remove one motion vector per block, only the list L0 is stored. When the motion vector of the list L0 is not available, its value is changed by the scaling of the motion vector at the same position in list L1. This process is illustrated in Figure 8Error: Reference source not found. For instance, the 1st top block has two motion vectors. Only the L0 MV is kept, after rescaling. The 2nd top block uses L0 and ref 0, so its MV is stored as is. The 3rd top block uses L1, so its MV is rescaled to refer to ref 0 of L0.

Figure 8: Schematic representation of the scaling process into one single list.

These modifications simplify the motion vector temporal derivation process. Indeed, only the mode needs to be checked for the derivation process instead of checking the availability of each list and comparing the direction, the reference frame index of 2 motion vectors and selecting the one which cross the current frame and/or the one which has the lower temporal distance.

Consequently, the maximum number of operations needed for the proposed scheme is much lower than the maximum number needed for the current derivation process of the collocated in HM2.0. Eventually, for Skip mode and Merge mode no scaling is needed for the temporal predictor.

## This modification, combined with the previous one, significantly reduces the amount of bits needed for the storage of the motion information of one block. As depicted in Table 4, our proposal removes the information regarding the components of one motion vector, the reference frame indexes buffer and the list indexes buffer. Consequently, no data access is needed for these data. Moreover, each motion vector component is stored on 8 bits instead of 12 bits. Consequently, 17 bits for each block need to be stored instead the initial 55 bits for the current implementation of the motion vector memory compression in HM2.0.

| L0 |  | L1 |  |
| --- | --- | --- | --- |
| Mvx | 8 bits | Mvx | 8 bits |
| Mvy | 8 bits | Mvy | 8 bits |
| Ref idx | 2 bits | Ref idx | 2 bits |
| List idx | 1 bit | List idx | 1 bit |
| mode |  |  |  |
| 1 bit |  |  |  |
| Total |  |  |  |
| 17 bits |  |  |  |

Table 4: Reduced number of bits for storing each collocated vector and the associated coding information.

The performance results of the combination of this modification and of the previous modification are reported in Table 5 for a summarization of 4 blocks and a summarization of 16 blocks. With our proposals, this corresponds respectively to a compression ratio equal to 13x and 52x. In average for the 4 configurations, 0.7% bitrate savings is obtained for 13x compression ratio and 0.5% for 52x compression ratio.

| Compression ratio | 13x |  |  |  |  | 52x |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RAHE | RALC | LDHE | LDLC | AVG | RAHE | RALC | LDHE | LDLC | AVG |
| Bottom Right + MV Clipping +MV scaling | -0.6 | -0.5 | -0.8 | -0.8 | -0.7 | -0.5 | -0.4 | -0.5 | -0.6 | -0.5 |

Table 5: Coding performance results of the motion vector scaling combined with other modifications described in the previous sections.

# Experimental results

All results were generated using the test conditions specified in JCTVC-D600 [3]. Figure 11Error: Reference source not found and  show the average bitrate savings for several compression ratios with our proposed methods. The results presented in that section have been performed with the HM2.0 after introducing the three following modifications:

- the bottom right candidates presented in section 3,

- the motion vectors scaling in one single list 4.1

- the motion vector clipping method described in section 4.3

The depicted results are the average of the four configurations (RAHE, RALC, LDHE, LDLC) compared to the HM2.0 anchors, which use a motion memory compression ratio equal to 5.5 for all data needed for the temporal predictor derivation.

Figure 11: Performance in terms of coding efficiency of the different methods for the motion vector compression.

| Configuration |  |  | MV Block summarization | Compression ratio | Mbits/frame 4kx2k | Average BDRate |
| --- | --- | --- | --- | --- | --- | --- |
| Bottom Right | Clipping | Scaling |  |  |  |  |
|  |  |  | 4 | 4x | 7.6 | -0.8 |
|  |  |  | 4 | 13x | 2.4 | -0.7 |
|  |  |  | 16 | 16x | 1.9 | -0.6 |
|  |  |  | 16 | 22.6x | 1.3 | -0.7 |
|  |  |  | 16 | 52x | 0.6 | -0.5 |
| No MV compression |  |  | - | 1x | 30.4 | -0.8 |
| Anchor |  |  | 16 | 5.5x | 5.5 | 0.0 |

Table 6: Results of our proposed method for memory compression.

All tested configurations show a coding gain against the HM2.0 anchors. According to these results, the configuration with 52x compression ratio and 0.5% bitrate savings seems to be the best compromise. Compared to the configuration without any MV compression, this configuration gives 0.3% loss. The entire summary table for this configuration is reported in Table 7Error: Reference source not found.

Please note that full results of all configurations presented in this contribution are available in the attached Excel sheets.

The results related to compression ratio 13x and 52x presented in Table 6 have been cross-checked by Sharp in document JCTVC-E410.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | -0.2 | -0.3 | -0.1 | 0.0 | 0.1 |
| Class B | -0.4 | -0.3 | -0.3 | -0.3 | -0.1 | -0.1 |
| Class C | -0.7 | -0.6 | -0.7 | -0.6 | -0.4 | -0.4 |
| Class D | -0.8 | -0.8 | -0.8 | -0.6 | -0.6 | -0.7 |
| Class E |  |  |  |  |  |  |
| All | -0.5 | -0.5 | -0.5 | -0.4 | -0.3 | -0.2 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.4 | -0.2 | -0.4 | -0.3 | -0.2 | -0.2 |
| Class C | -0.6 | -0.6 | -0.5 | -0.7 | -0.7 | -0.8 |
| Class D | -0.7 | -1.1 | -0.9 | -0.6 | -0.4 | -0.3 |
| Class E | -0.6 | -0.7 | -0.3 | -0.7 | -0.9 | -0.8 |
| All | -0.5 | -0.6 | -0.5 | -0.6 | -0.5 | -0.5 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

Table 7: Summary results of the proposed scheme with 52x compression ratio.

# Conclusion

This contribution presents four modifications of the memory compression of the motion vectors. The first one is about a bug fix about the selection of the right reference index. The second one is about the change of the Top Left candidate by the Bottom Right candidate which brings a non-negligible gain.

In addition to these two small changes, the current contribution described two methods (motion vectors scaling in one single list and motion vector clipping) to further compress the motion vector data while improving the coding efficiency compared to HM2.0.

The best compromise of this proposal is the memory reduction of 52x with an average bitrate savings equal to 0.5% compared to the current HM2.0 anchors.

With these proposed modifications, the memory bandwidth represents only 600 Kbits/frame for a 4Kx2K video sequence and includes all the related motion data needed for the derivation process of the temporal predictor. Moreover, the buffer of the reference frame indexes and the list index are removed, which avoids memory access to these data. We can note that the increase of compression ratio also reduces the memory access of the motion vector data.

Taking into account the memory compression increase proposed by this method and the average BDR gain of 0.5%, we recommend to adopt these modifications in the HM design.

- References

[1]	Yeping Su and Andrew Segall, “On motion vector competition”, JCTVC-C257, Guangzhou, CN, 7-15 October, 2010.

[2] Yeping Su and Andrew Segall, “CE9: Reduced resolution storage of motion vector data”, JCTVC-D072, Daegu, KR, 20-28 January, 2011.

[3] Frank Bossen, "Common test conditions and software reference configurations," JCTVC-D600 / m19497, 4th Meeting: Daegu, KR, 20-28 January, 2011.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).