| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, 16-23 March, 2011 | Document: JCTVC-E222 |
| --- | --- |

| Title: | Slices modifications for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Naël OUEDRAOGO Patrice ONNO Guillaume LAROCHE Edouard FRANCOIS Christophe GISQUET Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | nael.ouedraogo@crf.canon.fr patrice.onno@crf.canon.fr guillaume.laroche@crf.canon.fr edouard.francois@crf.canon.fr christophe.gisquet@crf.canon.fr |  |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution proposes a modification of the Slice partitioning in HEVC. Entropy slices principle presented in JCTVC-D070 and adopted in HM2.0 is slightly modified. Slice headers that were located at the beginning of the Entropy slices are moved to a single new Slice Parameter Sets NAL unit located at the beginning of the frame. Indeed in HM2.0, the current Slice headers contain several coding parameters like POC, frame_num or slice_type related to the frame itself but are duplicated for all slices. The main goal of having several Slice headers was just to indicate the coding dependencies boundaries.

Furthermore, this contribution proposes to add information in the Slice Parameter Set such as the coding dependencies signalization to indicate slice boundaries previously indicated with slice headers.  Additional information related to the signaling of processing boundaries for loop filters is also in traduced in this Slice Parameter Sets NAL.

The intent of this contribution is to provide solutions to ease the streaming of HEVC video and parallel decoding.

# Introduction

For video streaming typically with RTP over UDP communication protocol, NAL units are packetized into network packets according to three packetization methods which depend on the Maximum Transmission Unit size (MTU) of the network:

- a single NAL unit is embedded in one packet

- multiple NAL units are embedded in one packet

- one NAL unit is fragmented among several consecutive packets

It is quite obvious that the third strategy is not the optimal in case of transmission errors. Indeed, streaming clients generally prefer to discard the entire NAL unit when one of the fragmented NAL units is lost since the original NAL unit is not decodable. The coded data loss rate observed at the decoder is thus very important in comparison to the packet loss rate. For this reason, it is generally preferred to have slices which size is comparable to MTU size. Using numerous slices is a classical encoding strategy for streaming however the main drawback is the increase of the bit rate of the encoded video.

One requirement for streaming is to limit error propagation to predetermined areas within a frame in order to guarantee the decoded quality of at least a part of the image in case of packet loss.

Consequently, it is a key point for HEVC to provide error propagation limitation functionality and possibilities of defining multiple slices in frame with a reasonable impact on coding efficiency.

Two frame partitioning methods are currently under consideration in HEVC. First, slice partitioning divides each frame in independent subsets of decodable coding units by breaking all coding dependencies between these subsets. This feature fulfills the error propagation limitation functionality.  Second, lightweight slices (entropy slices) introduced in [1], guarantee independent entropy decoding of each lightweight slices by resetting context model initialization of entropy coder and modifying neighborhood definition. One advantage of lightweight slices against slice is that a coding gain is obtained by avoiding sending a slice header for each entropy slice and enabling intra prediction between two entropy slices. Figure 1 illustrates one frame partition of the current HM 2.0 in entropy slices with three error propagation boundaries defined on entropy slice boundaries.

Figure 1: Example of lightweight slice for one frame in HM2.0

HEVC decoding process of one frame involves several decoding stages which are syntax or entropy decoding, pixel sample reconstruction (dequantization, inverse transform and prediction) and in loop filtering processes. In some decoder implementation, these three processes are parallelized to reach high speed decoding performance.

Pixel sample reconstruction process uses neighbouring coding unit information to decode pixels value. Breaking dependencies with this neighbourhood define boundaries which limit error propagation.

Loop filter process (as entropy process) is adaptively performed in the frame and is known as a significant part of the decoding process complexity.

These three coding processes use neighbouring information (or context model) to code the information. Defining coding boundaries for each of these process answers to both the streaming and parallel decoding requirements for HEVC.

# Slice modifications

Entropy slices concept presented in [1] are used as a basis of this contribution. We propose some modifications in order to respond to streaming requirements and parallel decoding.

## Definition of a Slice Parameter Sets NAL unit

First modification consists in moving the syntax of coding parameters defined in slice headers of usual AVC slices to a Parameter Sets referred as Slice Parameter Sets NAL unit. These parameters are generally similar within a picture and thus are fixed for the whole frame. Typically such parameters are the slice type, Picture Order Count or parameters related to reference pictures. Coding syntax of the Slice Parameter Sets is presented in section 5 of this contribution. By removing these redundant information bitrate saving is expected as some contributions [2] have shown in simulation of slice headers removal in AVC slice.

## Addition of reconstruction and loop filter dependencies

Second modification consists in adding other information related to the coding dependencies between coding units in Slice Parameter Sets NAL unit. Such information indicates the location of coding boundaries in the picture for the intra and inter prediction processes and are referred as reconstruction process boundaries. For instance, two blocks separated by such a boundary cannot use pixels of the other block for intra prediction. Please note that these boundaries are not breaking dependencies for entropy process. It means that for entropy process two blocks separated by one reconstruction process boundaries are considered as available in the neighborhood process and context models are not reset.

As a third modification, we propose to add information to signal further processing boundaries for the loop filter which define independent processing region for loop filters. Consequently, partition for intra and inter prediction and loop filter boundaries can be independently defined and moreover are also independent to the entropy slices partitioning of the frame.

## New slice types definition

The proposal finally comes to the definition of a slice partition for each of the three decoding processes which are the entropy decoding, pixel sample reconstruction and loop filter processes. This contribution creates new independent slices entities representing each of these decoding processes. Three slices types are considered:

- Slices (with same coding and decoding properties as entropy slices). Each slice contains in a NAL unit a set of CU which can be parsed independently of other ones. The syntax included in the Slice Parameter Sets NAL unit indicates the address of the first LCU.

- Reconstruction slices (break all inter and intra prediction coding dependencies) which are signaled by LCU addresses in the Slice Parameter Sets NAL unit. Pixels values of each reconstruction slice are decoded independently of the others. Reconstruction slices can be seen as virtual slices since they are not a physical NAL unit but they enable to break the reconstruction dependencies.

- Loop filters slices which break loop filter dependencies are also signaled by LCU addresses in the Parameter Sets NAL unit. Deblocking and loop filter can be applied independently on each loop filter slice. As for reconstruction slices, loop filter slices are not physical NAL unit can be considered as virtual slices.

Coding parameters of pixel sample reconstruction and loop filter processes are defined for each of reconstruction and loop filter virtual slices in Slice Parameter Sets NAL unit. Typically, quantization parameter is defined for each reconstruction slice. Deblocking filter activation flag and ALF parameters are defined for each loop filter slice.

# Example

The figure 2 represents an example of slice partitioning for one frame according to our proposal. The frame is partitioned in several slice NAL units to respond to streaming requirement (dark lines). Two reconstruction “virtual” slices split the frame in two independent parts that are reconstructed independently and thus error propagation is limited to half part of the image (red dot lines). Two loop filter “virtual” slices are defined within the frame to allow independent loop filter decoding (blue dot lines). In addition the slice header previously placed at the beginning of each slice NAL unit has been moved into the Slice Parameter Sets. A reduced header is still kept in each slice NAL unit, as it is specified with entropy slices proposed in [1].  This reduced header contains first_tb_in_slice and cabac_init_idc syntax elements as described in section 5.

Figure 2: Example of our slice partitioning method for one frame

The decoder receives SlPS NAL unit prior to slice NAL units. It decodes the different coding parameters of SlPS and identifies reconstruction and loop filter slice boundaries. For parallelization process, one thread can be used for each of these slices. Slice NAL units are then decoded in parallel with same process as described in [1] for entropy slices. Once parsing CUs of one slice NAL unit corresponding to one reconstruction slice is done, the decoder decodes intra or inter coding units of corresponding reconstruction slice with its quantization parameter as defined in SlPS. Once coding units of one loop filter slices have been decoded, deblocking filter and ALF filter parameters of the corresponding loop filter are used to filter the decoded pixels of CUs.

# Advantages

The proposed method enables to define independent partitions for each of the three different decoding processes. Thus slice behavior for packetization and coding is separately defined. Indeed, we define three types of slices each of them being employed for different roles: for instance, slices are dedicated to network transport and thus can be adapted to network characteristics. Reconstruction slices are used to limit error propagation. Loop filter slice provide a partition for loop filter process and thus accurate post processing can be envisaged in the frame. Proposed method allows definition of a frame partitioning that match either current slice method considered in the slice AHG or entropy slice method proposed in [1].

One first advantage of the proposal is that the partitioning of the frame is flexible and can be adapted to the application requirements.

Secondly, our proposal shares most of the characteristics of the entropy slices. Thus, same entropy decoding parallelizing characteristics are enabled. Moreover, further parallel processing is provided by reconstruction slices and loop filter slices.

Third advantage of the proposal is that parallel processing is permitted for each processing stage of codecs and flexibility of the slice definition enables accentuating the parallelization possibilities for complex processing stages independently of others. Since Slice Parameter Sets NAL unit is transmitted prior to slice, the decoder is able to initialize its decoding resources prior to slice decoding by parsing the parameters of SlPS.

In comparison to entropy slice or AHG current slice partitioning strategy, coding gain is expected since redundant definition of slice’s parameters comprised in slice header are moved in the Slice Parameter Sets NAL unit and is thus defined once for each frame. Also our proposal gives more flexibility regarding the usage of inter-slice dependencies for the reconstruction and loop filter processes, which can be exploited to improve the coding efficiency.

Other advantages are provided for HEVC streaming. For instance, in case of slice loss, the decoder is able to detect the reconstruction slice boundaries. For entropy slices, it is not true when the entropy slice that includes AVC slice headers parameters is lost. This facilitates the error concealment strategy and video quality estimation process at decoder side.

Forward Error Correction (FEC) is often used to protect the transmission of one video stream. A classical method consists in defining different protection level in function of the importance of the information to be protected. Thus, including the coding parameters of slices in Slice Parameter Set NAL unit enables to adapt FEC protection level independently of slice data. This is not the case for lightweight slices or AHG slices because coding parameters and slice data are embedded in the same NAL unit.

# Coding Syntax

We propose the following coding syntax for our contribution: Syntax of slices has been modified as following:

| slice_layer_rbsp( ) { | Descriptor |
| --- | --- |
| first_tb_in_slice | ue(v) |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| slice_data( ) |  |
| rbsp_slice_trailing_bits( ) |  |
| } |  |

Below is the syntax of the proposed Slice Parameter Set NAL unit that defines reconstruction slices and loop filters slices boundaries. It includes several syntax elements previously defined in slice header syntax.

| slice_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| frame_type | ue(v) |
| frame_num | u(v) |
| if( IdrPicFlag ) |  |
| idr_pic_id | ue(v) |
| pic_order_cnt_lsb | u(v) |
| if( frame_type = = P ¦ ¦ frame_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( nal_ref_idc != 0 ) |  |
| dec_ref_pic_marking( ) |  |
| if( frame_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| num_recons_slices_minus1 | ue(v) |
| for ( i = 0; i < num_recons_slices_minus1; i++) |  |
| recons_slice_ index_offset _in_tb[i] | ue(v) |
| for ( i = 0; i <= num_recons_slices_minus1; i++) |  |
| recons_slice_qp_delta[i] | se(v) |
| num_loop_filter_slices_minus1 | ue(v) |
| for ( i = 0; i < num_loop_filter_slices_minus1; i++) |  |
| lf_slice_index_offset_in_tb[i] | ue(v) |
| for ( i = 0; i <= num_loop_filter_slices_minus1; i++) { |  |
| alf_param() |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |
| } |  |
| rbsp_trailing_bits() |  |
| } |  |

The semantics of new syntax elements which are different than in previous slice header definition is described below:

frame_type specifies the coding type of the frame. For each slice, the value of slice_type is equal to frame_type. Coding type of slice_type is specified in Table 7.3 of [3].

num_recons_slices_minus1 specifies the number of reconstruction slices for a picture. When num_recons_slices_minus1is equal to 0, a single reconstruction slice is used per frame.

recons_slice_index_offset _in_tb[i] is used to specify the index of the first tree block in the ith reconstruction slice of the picture. The index of the first tree block first_tb_in_recons_slice of the ith reconstruction slice is equal to the sum of i first recons_slice_index_offset _in_tb[i] values. If num_recons_slices_minus1 value is equal to 0, first_tb_in_recons_slice of the single reconstruction slice is equal to 0.

recons_slice_qp_delta[i]  specifies the QP values used for the ith reconstruction slice. Each value of recons_slice_qp_delta is coded relatively to the value of the previous reconstruction slice’s QP value.

num_loop_filter_slices_minus1 specifies the number of loop filter slices for a picture. When num_recons_slices_minus1is equal to 0, a single reconstruction slice is used per frame.

lf_slice_index_offset_in_tb[i] is used to specify the index of the first tree block in the ith loop filter slice of the picture. The index of the first tree block first_tb_in_lf_slice of the ith loop filter slice is equal to the sum of i first lf_slice_index_offset_in_tb [i] values. If num_loop_filter_slices_minus1 value is equal to 0, first_tb_in_lf_slice of the single loop filter slice is equal to 0.

# Conclusion

Slices modification proposed in this contribution enables a flexible and independent partitioning for the main decoding processes of HEVC codec. The principle of defining coding dependencies and processing boundaries in the proposed Slice Parameter Sets presents advantage both for streaming application and for parallel decoding of HEVC videos. Furthermore, the grouping of slice header parameters in one single Slice Parameter Sets improves the coding efficiency when numerous coding dependencies boundaries are defined per frame. Forward Error Correction can also take advantage of this single Slice Parameter Sets NAL unit for reliable video streaming.

Considering the numerous advantages of the proposal for streaming, parallel processing and coding efficiency, we propose to further study this proposition in Slice Ad-Hoc Group or in a CE to further evaluate this technology.

# Reference

[1] Kiran Misra and Andrew Segall, “Lightweight slicing for entropy coding”, JCTVC-D070, Daegu, KR, 20-28 January, 2011

[2] A. Fuldseth, “Replacing slices with tiles for high level parallelism”, JCTVC-D227, Daegu, KR, 20-28 January, 2011

[3] Thomas Wiegand, Woo-Jin Han, Benjamin Bross, Jens-Rainer Ohm, Gary J. Sullivan, “Microsoft WD2: Working Draft 2 of High-Efficiency Video Coding”, JCTVC-D503, Daegu, KR, 20-28 January, 2011

# Patent rights declaration(s)

Canon Research Centre France may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).