| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E224 WG11 Number: m19747 |
| --- | --- |

| Title: | CE12.2: Results for parallel deblocking filter decisions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Matthias Narroschke Thomas Wedi Panasonic R&D Center Germany Monzastr. 4c, 63225 Langen, Germany | Tel: Email: | +49 6103 766 204 Matthias.Narroschke@eu.panasonic.com +49 6103 766 1194 Thomas.Wedi@eu.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution is a part of the CE12, Subset 2: It presents the results for the Parallel deblocking filter decisions which was presented first in JCTVC-D214. For the deblocking of a current coding unit, it is proposed to perform all required decisions based on the unfiltered signal of the current coding unit. This dissolves the dependencies which are present in the current HM2.0 deblocking filter design. The dissolved dependencies increase the parallel processing capabilities of the deblocking filter. As a consequence, the sequential operations in the critical path required for the decision and filtering operations of the deblocking filter are reduced by 30%. Experiments following the common conditions show that the BD-bit rate stays unchanged for the luminance signal and almost unchanged for the two chrominance signals. An increased subjective quality is noticeable at the same bit rate. It is proposed to adopt this technique into the next version of the HM.

# Introduction

The Parallel deblocking filter decisions was first presented in the proposal JCTVC-D214 [6] at the last meeting in Daegu. At that meeting, an expert viewing test coordinated by Dr. Vittorio Baroncini was performed, which had the result that difference in subjective quality between the proposal and the reference was imperceptible [8]. It has been decided to evaluate the Parallel deblocking filter decisions within the core experiment CE12 [4].

In the following section 2, a brief technical review is given. All further details can be found in JCTVC-D214 [6]. In section 3, memory requirements are analyzed. Section 4 explains the performed experiments and shows the results. Section 5 concludes this contribution.

# Parallel filter decisions

The deblocking of images is performed based on coding units (CU), which may have various sizes, e.g. 8x8 samples, 16x16 samples. Vertical and horizontal edges of prediction and transform blocks are deblocked. Each edge consists of one or several segments, whereas a segment consists of 8 consecutive lines or columns. The segments vi of the vertical edges are deblocked before the segments hi of the horizontal edges. Figure 1 shows an example coding unit of the size 16x16 samples and the positions of the corresponding 4 segments v1,…,v4 and four segments h1,…,h4. The order of deblocking the vertical edges is from top to bottom and from left to right. The order of deblocking the horizontal edges is from left to right and from top to bottom.

Figure 1: Illustration of the vertical edges and the horizontal edges of an example coding unit (CU) of the size 16x16 samples.

In the following, the samples on the respective sides of the segments of the edges are denoted as  A and B, see Figure 2. The segment A corresponds to the left neighboring partition to B for vertical edges and to the above neighboring partition to B for horizontal edges.

Figure 2: Notation of a part of a vertical edge for deblocking (from [3]).

For each segment, the decisions and filtering operations are performed as explained in the following. The first performed decision is:

p22−2⋅p12+p02+q22−2⋅q12+q02+p25−2⋅p15+p05+q25−2⋅q15+q05<β.

The first part of the above equation, which isp22−2⋅p12+p02+q22−2⋅q12+q02, is denoted as d1,vin the case of vertical edges and as d1,h in the case of horizontal edges. The second part of the above equation, which isp25−2⋅p15+p05+q25−2⋅q15+q05, is denoted as d2,vin the case of vertical edges and as d2,h in the case of horizontal edges.

If the above decision is true, filtering is applied to the samples of the corresponding segment. By the use of further decisions for each of the eight lines/columns, i=0,…,7, a decision is performed to decide between a so called strong and a so called weak filter. This decision is of the form:

d<β>>2andp3i−p0i+q0i−q3i<β>>3andp0i−q0i<5⋅tc+1>>1

If this decision is true, the strong filter is applied, otherwise the weak filter. This decision is denoted as Di,v in the case of vertical edges and as Di,h in the case of horizontal edges.

Among the filter operations of the weak filter and the strong filter, the following one of the weak filter requires most sequential operations:

Δ=Clip−tc,tc,13⋅q0i−p0i+4⋅q1i−p1i−5⋅q2i−p2i+16>>5

p1i=Clip0,255,p1i+Δ2.

The filter operations are denoted as Fi,v in the case of vertical edges and as Fi,h in the case of horizontal edges.

The basic idea of this proposal is to perform all decisionsd1,v, d1,h, d2,v, d2,h, Di,v, Di,h based on the non-deblocked signal of the current coding unit. This dissolves dependencies between the horizontal and vertical filtering which are currently limiting the degree of parallelization.

In order to deblock a coding unit of the size 8x8 samples according to the proposal, the following 4 sequential steps are necessary:

- Performing the decisions d1,v, d2,v,d1,h, and d2,h

- Performing the decisions Di,v, with i=1,…,8 and Di,h, with i=1,…,8

- Performing the filtering operations Fi,v, with i=1,…,8 and Fi,h, with i=1,…,4

- Performing the filtering operations Fi,h, with i=5,…,8

Figure 3 to Figure 6 illustrate these 4 sequential steps. Furthermore they show the samples used for the individual operations.

Figure 3: Illustration of samples required to perform the decisionsd1,v, d1,h, d2,v, d2,h according to the proposal.

Figure 4: Illustration of samples required to perform the decisionsDi,v, Di,haccording to the proposal.

Figure 5: Illustration of samples required to perform the filtering operations F1,v−F8,vand F5,h−F8,h according to the proposal.

Figure 6: Illustration of samples required to perform the decisions according to the proposal.

As the performance of all decisionsd1,v, d1,h, d2,v, d2,h, Di,v, Di,h based on the non-deblocked signal of the current coding unit dissolves the dependencies, the parallel processing capabilities are increased. As a consequence, the number of sequential operations in the critical path is reduced from 44 of the HM2.0 to only 32. This is a reduction of around 30%. For details, see JCTVC-D214 [6].

# Memory requirements

In the separate presentation “JCTVC-E224_Mem_analysis_r1.pdf”, an analysis of the memory is performed for CU of the size 8x8 to 64x64, which are currently applied according to the commonly defined conditions [5]. The results are summarized in the following table.

| CU Size [samples x samples] | Additional memory to store decisions [bit] |
| --- | --- |
| 8x8 | 5 |
| 16x16 | 8 |
| 32x32 | 10 |
| 64x64 | 14 |

At maximum, 14 additional bits are required for coding units of the size 64x64.

# Experiments and results

The proposed changes to the deblocking filter have been implemented into the HM2.0 [1]. The required software modifications are minor. However, the implementation does not exploit the introduced increased parallel processing capabilities.

## BD-bit rate and run time ratios

Following the test conditions of the CE12 [4] which match to the commonly defined conditions [5], the performance of all six test cases is evaluated, which is Intra, Random access, and Low delay, each in high efficiency and low complexity operation mode.

For all run time measurements, computers of the same configuration are used for both, reference and proposal.

The results with respect to BD-bit rate as well as to the encoder-/decoder run time ratios compared to the reference are shown in Figure 7. Negative BD-rate numbers show a gain of the proposal compared to the reference. Run-time ratios less than 100% show reflect that the run time of the proposal is faster than the one of the reference.

Further details can be found in the document “JCTVC-E224_r1.xls”

The following results can be observed:

- Approximately no change with respect to BD-bit rate reduction

- Approximately same run time since the implementation does not exploit the benefit of increased parallel processing capabilities

Figure 7: BD-bit rates and run time ratios of the proposal compared to the reference [1].

For the decoder, additional run time measurements have been performed. For each of the six coding configurations, the whole test set has been decoded 80 times. Each time, the decoding time for the whole test set has been measured. Using the 80 measured decoding times, an average decoding time T and the corresponding standard deviation σT is calculated. The following table shows the mean decoding times and the standard deviations for the proposal and for the reference.

| Coding configuration | Reference |  | Proposal |  | Decoder run time increase [%] |
| --- | --- | --- | --- | --- | --- |
|  | T[s] | σT[s] | T[s] | σT[s] |  |
| Intra, High efficiency | 4735.35 | 40.43 | 4750.74 | 39.79 | 0.32 |
| Intra, Low complexity | 2157.63 | 23,30 | 2172.35 | 21.38 | 0.68 |
| Random access, High efficiency | 2474.28 | 34.87 | 2479.38 | 33.10 | 0.21 |
| Random access, Low complexity | 1380.30 | 18.32 | 1389.45 | 22.43 | 0.66 |
| Low delay, High efficiency | 1758.84 | 24.60 | 1763.43 | 27.12 | 0.26 |
| Low delay, Low complexity | 1000.76 | 14.43 | 1004.77 | 13.71 | 0.40 |

## Subjective evaluation

In CE12, various test sequences have been selected for subjective evaluations. For these test sequences, the subjective quality of the proposal compared to the reference has been performed with the results shown in the following table.

| Test sequence | Coding configuration | QP | Subjective quality |
| --- | --- | --- | --- |
| Kimono | Low delay, low complexity | 32 | No difference noticeable |
| Kimono | Random access, high efficiency | 37 | No difference noticeable |
| BQ_Mall | Low delay, low complexity | 32 | No difference noticeable |
| BQ_Mall | Random access, high efficiency | 37 | No difference noticeable |
| Vidyo3 | Low delay, low complexity | 32 | No difference noticeable |
| Vidyo3 | Low delay, high efficiency | 37 | Proposal sharper than reference without increase of blocking. Proposal has less color artifacts than reference. |

For five out of the six test sequences, no difference in subjective quality is noticeable. For one out of the six test sequences, the proposal is clearly sharper than the reference without increased blocking. In addition, the proposal shows less color artifacts.

The increase of the sharpness is illustrated in Figure 8 and Figure 9. In Figure 8, a cropped part of a deblocked frame of the test sequence Vidyo3 is shown for the case of the reference HM2.0 [1], low delay, high efficiency, QP37. Figure 9 shows the same cropped part for the case of the proposed deblocking.

The reduction of color artifacts is illustrated in Figure 10 and Figure 11. In Figure 10, a cropped part of a deblocked frame of the test sequence Vidyo3 is shown for the case of the reference HM2.0 [1], low delay, high efficiency, QP37. Figure 11 shows the same cropped part for the case of the proposed deblocking.

Figure 8: Cropped part of a deblocked frame of the test sequence Vidyo3 in the case of the reference [1]. Test case: Low delay, High Efficiency, QP37.

Figure 9: Cropped part of a deblocked frame of the test sequence Vidyo3 in the case of the proposal. Test case: Low delay, High Efficiency, QP37.

Figure 10: Cropped part of a deblocked frame of the test sequence Vidyo3 in the case of the reference [1]. Test case: Low delay, High Efficiency, QP37.

Figure 11: Cropped part of a deblocked frame of the test sequence Vidyo3 in the case of the proposal. Test case: Low delay, High Efficiency, QP37.

# Conclusion

For the deblocking of a current coding unit, it is proposed to perform all required decisions based on the unfiltered signal of the current coding unit. This dissolves the dependencies which are present in the current HM2.0 deblocking filter design. The dissolved dependencies increase the parallel processing capabilities of the deblocking filter. As a consequence, the sequential operations in the critical path required for the decision and filtering operations of the deblocking filter are reduced by 30%. An analysis has the result that sequential implementations need only 14 bits to store intermediate decisions. Experiments following the commonly defined conditions show that the BD-bit rate stays unchanged for the luminance signal and approximately unchanged for the two chrominance signals. An increased subjective quality is noticeable at the same bit rate. Therefore, it is proposed to adopt this technique into the next version of the HM.

# References

- HM2.0 software: http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-2.0.

- T. Wiegand, W.-J. Han, J.-R. Ohm, G. J. Sullivan, High Efficiency Video Coding (HEVC) text specification Working Draft 1, JCTVC-C403, Guangzou, China, October 2010.

- JCT-VC, Test Model under Consideration, JCTVC-B205_draft007, Geneva, Switzerland, 21-28 July 2010.

- A. Norkin, B. Jeon, M. Narroschke, Description of Core Experiment 12 (CE12): Deblocking filtering, JCTVC-D612, Daegu, Korea, January, 2011.

- F. Bossen, Common test conditions and software reference configurations, JCTVC-D500, Daegu, Korea, January, 2011.

- M. Narroschke, H. Sasai, T. Wedi, Reduction of operations in the critical path of the deblocking filter, JCTVC-D214, Daegu, Korea, January, 2011.

- Rickard Sjoberg, David Flynn, BoG report on deblocking filter - subjective viewing and decoder run-time, JCTVC-D457, Daegu, Korea, January, 2011.

- G. Sullivan, J.-R. Ohm, Meeting report of the fourth meeting of the Joint Collaborative Team on Video Coding (JCT-VC), Daegu, KR, 20-28 January, 2011, JCTVC-D500 Daegu, Korea, January, 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).