| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E226 WG11 Number: m19749 |
| --- | --- |

| Title: | Parallel Context Processing for Significance map using block-based context updates |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hisao Sasai Takahiro Nishi 1006 Kadoma, Kadoma Osaka, Japan | Tel: Email: | +81 6 6900 9689 sasai.hisao@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This proposal presents a technique for the parallelization of context processing to improve the throughput of the entropy coder for CABAC. In this contribution, it is proposed that block-based context updates are used for significance map. The context updates make difficult to increase through put due to their serial dependencies. The same probabilities are used for encoding and decoding within the coefficient block loop. The proposed solution were implemented in HMv2 and their coding efficiencies were evaluated. The parallelization improvements of the proposal comes at a cost of 0.2% performance loss.

# Introduction

The CABAC has high serial dependencies from multiple feedback loop, which poses practical problems for real-time processing of high resolution video sequences. It is difficult to increase throughput due to the dependencies as discussed in [1]. At both of encoder and decoder, "range update" and "context update" are performed on feedback loop. In the current HEVC[2], same context index is called continuously (bin by bin), e.g. "significant_coeff_flag" in high frequency. It might be the bottleneck limiting the bin throughput of the whole process . To increase the bin through put, 4-bin cycle based updates were also proposed and discussed at the previous JCTVC meeting [3].

In terms of bin throughput, the number of bin counts for significance map ("significant_coeff_flag" and "last_flag") accounts for 36% at random access and low delay coding test set and 43%  at intra only coding test set, of total bin counts.

In this contribution, we propose block-based context updates for significance map, which is represented as "significant_coeff_flag" and "last_flag".

# Proposed Solution

## Block based context updates for Significance map

For significance map operation, "significant_coeff_flag" and "last_flag" are encoded and decoded in same block loop. Figure 1 shows the operation flow of the significance map coding on current HEVC CABAC. Within the block loop, first of all context selection is conducted. Then, the probability ( state of context ) corresponding the selected context is loaded. At the encoder, a target bin for significance map is encoded by using the probability. At the decoder, a target bin for significance map is parsed by using the probability. In this stage, the range in arithmetic coder is updated bin by bin. Therefore, the operation  "Bin Code" has serial dependency on following bins. Finally, the probability is updated by encoded or parsed bin. The updated probability is used for following bin operation. Accordingly, the operation "Context Update on Current States" has also serial dependency on following probability derivation.

In proposed solution, temporal storing process and set process are added as shown in Figure 2. At the beginning of the operation of significance map coding per block, current state of context are stored to temporal memory. Then the same operation as CABAC is conducted within the block loop except for the operation "Context Update on Stored States". The difference is which of stored or current probability is updated at this stage. In our solution, stored probability is updated, but current probability which is used for encoding or decoding is no changed. Therefore, there is no dependency on the probability within the block loop. Finally, stored probability (state of context ) which is updated within the block loop is set to current probability (state of context) as following block operation.

Since the additional operation is out of block loop, the additional operation is limited. The additional memory size is equal to the context size of significance map.

Figure 1: Operation flow for significance map on HEVC CABAC

Figure 2:Operation flow for significance map on proposal solution

## The benefit of proposed solution

The benefit of this approach is that each process can be processed in parallel without waiting, since no dependencies on the probability. Figure 3 shows an example of the timing chart on hardware implementation on HEVC CABAC, in case that there is one module per each operations. If same context index is used continuously, "Context Load" module has to wait until previous probability is updated at "Context Update" module. Therefore, some waiting time is needed on current HEVC CABAC.

On the other hand, if we can remove the dependency from the probability, no waiting time is needed as shown in Figure 4.

Figure 3: An example of the timing chart on hardware implementation on HEVC CABAC

Figure 4: An example of the timing chart on hardware implementation on proposed solution.

# Results

The proposed method performance relative to the HMv2 software was checked under the common test condition described in JCTVC-D600[4]. The computing platform is Linux 64bits and the executables were compiled on 64-bit Linux with gcc 4.4.3. The cross verification is performed by TI.

The decoding time measurement of the simulation is still on going.

Table 1: BD BR for proposed solution (high efficiency)

The proposed method performance relative to another coding method for "significance map" which was proposed at JCTVC-D262[5], was also checked under the common test condition as shown in Table 2. The results is compared with the results of JCTVC-D262.

Table 2: BD BR for proposed solution with JCTVC-D262 significance map coding  (vs. JCTVC-D262)

# Conclusion

In this contribution, parallel context processing for significant map by block-based context updates is proposed. The proposed solution makes it easier to parallelize with negligible cost to the coding efficiency. An experimental result shows 0.2% performance drop in average BD BR. In addition to current HMv2, combination of JCTVC-D262 is also tested. A result shows 0.0%-0.2% performance drop in average BD BR. It shows consistency for significance map coding method. It is suggested to consider the inclusion of this proposal in the HM/WD of HEVC in order to introduce additional possibility of parallel processing for the future deployment of the standard.

# Reference

- Vivienne Sze et. al., “Context selection complexity in HEVC CABAC”, JCTVC-D244, Daegu, January 2011.

- JCT-VC, “WD2: Working Draft 2 of High-Efficiency Video Coding”, JCTVC-D503, JCT-VC 4th  Meeting, Daegu, January, 2011.

- Dake He et. al., “Improving throughput for V2V coding in HEVC”, JCTVC-B036, Geneva, July 2010.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-D600, JCT-VC 4th Meeting, Daegu, January 2011.

- J. Sole et. al, "Parallel Context Processing for the significance map in high coding efficiency", JCTVC-D262, Daegu, January 2011.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).