| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E227 WG11 Number: m19750 |
| --- | --- |

| Title: | CE11: Context size reduction for the significance map (JCTVC-D185) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hisao Sasai Takahiro Nishi 1006 Kadoma, Kadoma Osaka, Japan | Tel: Email: | +81 6 6900 9689 sasai.hisao@jp.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution is a test report for JCTVC-D185 listed in CE11. Proposed technique is aimed to reduce the number of contexts for significance map. The proposed modifications have been implemented in HMv2 and their coding efficiencies were evaluated. The reduction of the used context size is reported as -22.4% (107 to 83) for the "significant_coeff_flag" and -73.1% (104 to 28) for the "last_flag". The BD BR for the high efficiency intra, random access, and low-delay configurations is reported as 0.03%, 0.00% and -0.14% by reducing "significant_coeff_flag" and "last_flag", -0.04%, -0.08% and -0.22% by reducing only "significant_coeff_flag", 0.08%, 0.08% and 0.02% by reducing only "last_flag", respectively.

# Introduction

For improved coding efficiency, current HEVC specification[1] is taking a highly adaptive context selection approach for significance map, while increased number of context size is required in comparison with the AVC specification. Table 1 shows results of used context size in HMv2 under the common test conditions of high efficiency set where "sig" means "significant_coeff_flag", "last" means "last_flag", "coeff" means the contexts related to "coefficient level" and "total" means whole contexts. Both of the "significant_coeff_flag" and the "last_flag" takes more than 25% of the whole contexts. As the number of context size affects on the amount of memory size especially for hardware implementation and on the computational cost for the initialization of each context, this proposal is focusing on the reduction of "significant_coeff_flag" and "last_flag".

Table 1 : Number of used context size for HMv2 anchor streams (high efficiency)

# Proposed Solution

To reduce the context size of significance map, we modify the context model derivation process for "significant_coeff_flag" and "last_flag" as follows:

significant_coeff_flag

For greater block sizes than 8x8, HHI_TRANSFORM_CODING refers significant_coeff_flag of neighboring frequency components for the higher frequency components. In the proposed solution, they are also referred for 8x8 blocks, i.e. done for 8x8, 16x16 and 32x32 blocks. For the higher frequency components, the context index is derived from the count of surrounding significant_coeff_flag. For greater block size than 8x8, those context indexes can be grouped since those components have similar characteristics on each condition of the index. On the other hand, for low frequency components (2x2 coefficients), the context index is derived from the position of the coefficients since those components has different characteristics.

As the results of above modification, the total number of used context size for "significant_coeff_flag" changes from "107" to "83" (77.6% of HMv2) as shown in Table 2.

Table 2: Number of used context size of the reduction of context size for "significant_coeff_flag" (N1)

last flag

In current HEVC, the context index is derived from 16 partitioning of the last flag position at even intervals. However, it is not necessary to keep small segmentation on especially higher frequency part since non-zero coefficients are occasionally presented in higher frequency component. In proposed solution, the context index is derived from 4 partitioning of the last flag position instead of 16 as shown in Figure 1. As the results of this modification, the total number of used context size for "last_flag" is reduced from "104" to "28" (26.9% of HMv2) as shown in Table 2.

Figure 1: Context Index separation for Last Flag in proposed method.

Table 3: Number of used context size of the reduction of context size for "last_flag" (N2)

As the results of those modification for significance map, the total number of used context size is reduced from "409" to "309" (75.6% of HMv2) for random access and low delay coding set and from "366" to "266" (72.7% of HMv2) as shown in Table 4.

Table 4: Number of used context size of the reduction of context size for significance map (N1+N2)

# Results

The proposed method’s performance relative to the HMv2 software was checked under the common test condition described in JCTVC-D600[2] .The computing platform is Linux 64bits and the executables were compiled on 64-bit Linux with gcc 4.4.3.

The following statistics are required in CE11[3] in addition to BD BR.

a) the total number of bins and bin reduction/increase due to proposed technique.
	b) a breakdown of the number of bins for sig flag, last flag and absolute coefficient level.
	c) a breakdown of the number of bins coded in regular and bypass mode for CABAC.
	d) the size of the context memory (i.e. number of contexts used) should be reported.

Our proposals are listed as follows:

N1: Reduction of "significant_coeff_flag".
	N2: Reduction of "last_flag".
	N1+N2: Reduction of both of "significant_coeff_flag" and "last_flag".

Table 2, 3 and 4 show the results in terms of d). BD BR for N1, N2 and N1+N2 are shown in Table 5, 6 and 7. Table 8, 9 and 10 show the results in terms of a), b) and c).

Table 5 : BD BR for context size reduction for significant_coeff_flag (N1)

Table 6 : BD BR for context size reduction for last_flag (N2)

Table 7 : BD BR for context size reduction for significance map (N1+N2)

Table 8: Statistics for bin counts for significant_coeff_flag (N1)

Table 9: Statistics for bin counts for last_flag (N2)

Table 10: Statistics for bin counts for significance map (N1+N2)

# Conclusion

This contribution provides the results of CE11. An experimental result shows that the reduction of context size comes at no cost in performance. The reduction of the used context size is measured as -22.4% (107 to 83) for the "significant_coeff_flag" and -73.1% (104 to 28) for the "last_flag". The BD BRs for the high efficiency intra, random access, and low-delay configurations are reported as 0.03%, 0.00% and -0.14% by reducing "significant_coeff_flag" and "last_flag", -0.04%, -0.08% and -0.22% by reducing only "significant_coeff_flag", 0.08%, 0.08% and 0.02% by reducing only "last_flag", respectively. We recommend our proposal to be adopted in the HM/WD of HEVC.

# Reference

- JCT-VC, “WD1: Working Draft 1 of High-Efficiency Video Coding”, JCTVC-C403, JCT-VC 3rd  Meeting, Guangzhou, October, 2010.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-D600, JCT-VC 4th Meeting, Daegu, January 2011.

- Vivienne Sze et. al., “CE11: Coefficient Scanning and Coding”, JCTVC-D611, Daegu, January 2011.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).