| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E229 WG11 Number: m19752 |
| --- | --- |

| Title: | CE9: Cross-check report of experiment M22-M26 by Panasonic |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |
| Purpose: | Report |  |  |  |
| Author(s) or Contact(s): | Toshiyasu Sugio Takahiro Nishi 1006 Kadoma, Kadoma Osaka, Japan | Tel: Email: | +81 6 6900 9689 sugio.toshiyasu@jp.panasonic.com |  |
| Source: | Panasonic Corporation |  |  |  |

_____________________________

# Abstract

This contribution is a report of cross-check result for CE9 experiment M22 to M26 in JCTVC-D609 that were proposed by MediaTek to evaluate derivation process for spatial/temporal motion vector candidates. The experimental results perfectly match with the one provided by MediaTek in the rate-distortion performance.

- Experimental Condition

The software to be tested was provided by MediaTek and its performance relative to the HM2.0 software was checked under the common test condition described in JCTVC-D600 [1] and table 1 macro setting described in JCTVC-D609 [2]. The computing platform is Linux 64bits and the executables were compiled on 64-bit Linux with gcc 4.4.3.

Table 1 : Verified software version and configuration

| Software version | Experiment | Configuration |
| --- | --- | --- |
| source_MTKr10.rar | M22 | #define MTK_AMVP_TEMPORAL_MVP 1 #define MTK_AMVP_SMVP_DERIVATION 0 #define MTK_MERG_TEMPORAL_MVP 0 #define MTK_TMVP_DERIVATION 1 #define MTK_MERG_TCTR_DERIVATION 0 #define MTK_AMVP_TCTR_DERIVATION 0 |
| source_MTKr10.rar | M23 | #define MTK_AMVP_TEMPORAL_MVP 1 #define MTK_AMVP_SMVP_DERIVATION 0 #define MTK_MERG_TEMPORAL_MVP 0 #define MTK_TMVP_DERIVATION 1 #define MTK_MERG_TCTR_DERIVATION 1 #define MTK_AMVP_TCTR_DERIVATION 0 |
| source_MTKr10.rar | M25 | #define MTK_AMVP_TEMPORAL_MVP 1 #define MTK_AMVP_SMVP_DERIVATION 0 #define MTK_MERG_TEMPORAL_MVP 1 #define MTK_TMVP_DERIVATION 1 #define MTK_MERG_TCTR_DERIVATION 0 #define MTK_AMVP_TCTR_DERIVATION 0 |
| source_MTKr10.rar | M26 | #define MTK_AMVP_TEMPORAL_MVP 0 #define MTK_AMVP_SMVP_DERIVATION 1 #define MTK_MERG_TEMPORAL_MVP 1 #define MTK_TMVP_DERIVATION 1 #define MTK_MERG_TCTR_DERIVATION 0 #define MTK_AMVP_TCTR_DERIVATION 1 |

- Results

R-D performance and execution time of the software are summarized in Table 2, and each experimental result is shown in Table 3 to 6. Detailed results are included in the attached excel sheet. It was confirmed that these results perfectly match with the one provided by MediaTek.

Table 2 : Summarized experimental results

|  | Random access |  | Random access LoCo |  | Low delay |  | Low delay LoCo |  | Average |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate | Enc time | BD-rate | Enc time | BD-rate | Enc time | BD-rate | Enc time | BD-rate | Enc time |
| M22 | -0.9% | 102% | -1.1% | 102% | -1.0% | 102% | -1.6% | 101% | -1.15% | 102% |
| M23 | -0.8% | 102% | -1.1% | 102% | -1.2% | 102% | -1.9% | 102% | -1.25% | 102% |
| M25 | -0.8% | 102% | -1.0% | 102% | -1.2% | 102% | -1.9% | 102% | -1.23% | 102% |
| M26 | -1.3% | 103% | -1.6% | 103% | -1.5% | 104% | -2.0% | 105% | -1.6% | 104% |

Table 3 : Experimental results of M22

Table 4 : Experimental results of M23

Table 5 : Experimental results of M25

Table 6 : Experimental results of M26

- Conclusion

The results of CE9 experiment M22-M26 were confirmed.

- References

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-D600, Daegu, January 2011.

- Joel Jung et al., “Description of Core Experiment 9: MV Coding and Skip/Merge operations”, JCTVC-D609, Daegu, January 2011.