| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E232 |
| --- | --- |

| Title: | CE1: Cross-verification report on NCTU’s proposal (JCTVC-E154) by TUB |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | M. Tok, A. Krutz Technische Universität Berlin, Sekr EN1, Einsteinufer 17 10587 Berlin, Germany | Tel: Email: | tok@nue.tu-berlin.de krutz@nue.tu-berlin.de |
| Source: | Technische Universität Berlin |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck “Test 1” of JCTVC-E154 on DMVD-based Bi-prediction submitted by National Chiao Tung University (NCTU)/ITRI International. The verification task has been done successfully and the results match those reported in JCTVC-E154.

# Verifikation

NCTU has provided encoder and decoder source code for the original HM 2.0 anchor and their proposal concerning DMVD-based Bi-prediction and the accordant encoded video bit-stream files for following settings:

- Low Delay / Low Complexity with hierarchical B-Frames

- Low Delay / High Efficiency with hierarchical B-Frames

- Random Access / Low Complexity with hierarchical B-Frames (GOP 8)

- Random Access / High Efficiency with hierarchical B-Frames (GOP 8)

The source code has been checked and the fundamental functionality of the proposal has been understood. The provided bit-streams for “Test 1” were decoded and bit-rates, Y, U and V-PSNR values and BD-rates were compared.

As test platform a Phenom II X4 945 (3.0GHz) with 4GB RAM is used.

# Results

The BD-rates measured match those reported by NCTU. The measured decoding times for all test cases except “Random Access Low Complexity” vary slightly from the reported times. For “Random Access Low Complexity” a decoding time of 105% is reported while a decoding time of 109% is measured. The changes proposed in JCTVC-E154 match those in the code. Following table summarizes the crosscheck results.

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1,0 | -1,5 | -1,5 | -1,4 | -1,4 | -1,2 |
| Class B | -0,9 | -1,3 | -1,2 | -1,1 | -1,4 | -1,4 |
| Class C | -1,4 | -1,9 | -1,9 | -1,5 | -2,1 | -2,2 |
| Class D | -1,5 | -2,1 | -1,9 | -1,6 | -2,3 | -2,2 |
| Class E |  |  |  |  |  |  |
| All | -1,2 | -1,7 | -1,6 | -1,4 | -1,8 | -1,7 |
| Dec Time[%] | 103% |  |  | 109% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1,2 | -2,1 | -2,3 | -2,0 | -2,5 | -2,1 |
| Class C | -1,7 | -2,2 | -2,4 | -2,3 | -2,8 | -2,9 |
| Class D | -1,9 | -1,8 | -2,5 | -2,3 | -2,8 | -2,4 |
| Class E | -1,9 | -3,1 | -2,5 | -2,6 | -3,2 | -2,8 |
| All | -1,6 | -2,2 | -2,4 | -2,3 | -2,8 | -2,5 |
| Dec Time[%] | 102% |  |  | 107% |  |  |

All decoding results can also be found in JCTVC-E232.xls.

# Conclusion

The decoding results for “Test 1” presented by National Chiao Tung University are mostly confirmed.