| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E238 |
| --- | --- |

| Title: | Verification results of MediaTek's contribution on sub-LCU delta QP (JCTVC-E051) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-verification |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: | madhukar@ti.com |
| Source: | Texas Instruments |  |  |

_____________________________

# Abstract

This contribution presents results of cross-verification of MediaTek's proposal on sub-LCU delta QP in JCTVC-E051. The PSNR/bit-rate numbers obtained were compared to those provided by MediaTek for QpMinCuSize = 32x32 (deltaQP signaled at 32x32 CU and above) and QpMinCuSize = 16x16 (deltaQP signaled at 16x16 CU and above). The number of frames used was 1/10 of that in common conditions to speed up simulation. It is reported that MediaTek and TI BD-Rate numbers matched. Reported summary of results for QpMinCuSize = 32x32: BD-Rate varies from -0.4% to -0.9%, Encoder time is in 2.5X-3X range, Decoder time is comparable to anchor. Reported summary of results for QpMinCuSize = 16x16: BD-Rate varies from 0.5% to -0.8%, Encoder time is in 2.5X-3X range, Decoder time is comparable to anchor. It is also reported that there is YUV output match between encoder and decoder.

# Simulation results

Detailed spreadsheets are attached to the contribution. High level summary of results provided below.

QpMinCuSize = 32x32 (deltaQP signaled at 32x32 CU and above).

|  | MediaTek | TI cross-check | MediaTek | TI cross-check | MediaTek | TI cross-check |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-Rate | BD-Rate | Enc Time | Enc Time | Dec Time | Dec Time |
| Intra | -0.9 | -0.9 | 251% | 266% | 99% | 101% |
| Random access | -0.7 | -0.7 | 285% | 277% | 98% | 91% |
| Low delay | -0.8 | -0.8 | 285% | 290% | 96% | 97% |
| Intra Loco | -0.7 | -0.7 | 293% | 303% | 100% | 105% |
| Random access Loco | -0.7 | -0.7 | 299% | 292% | 97% | 93% |
| Low delay Loco | -0.4 | -0.4 | 297% | 293% | 97% | 95% |

QpMinCuSize = 16x16 (deltaQP signaled at 16x16 CU and above).

|  | MediaTek | TI cross-check | MediaTek | TI cross-check | MediaTek | TI cross-check |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-Rate | BD-Rate | Enc Time | Enc Time | Dec Time | Dec Time |
| Intra | -0.8 | -0.8 | 249% | 267% | 100% | 103% |
| Random access | -0.5 | -0.5 | 284% | 277% | 98% | 91% |
| Low delay | -0.5 | -0.5 | 286% | 291% | 98% | 96% |
| Intra Loco | -0.3 | -0.3 | 292% | 301% | 101% | 106% |
| Random access Loco | 0.0 | 0.0 | 298% | 292% | 97% | 95% |
| Low delay Loco | 0.5 | 0.5 | 299% | 295% | 97% | 98% |