| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E240 |
| --- | --- |

| Title: | Reducing size of the LCEC coefficient coding tables |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema Kemal Ugur | Email: | jani.lainema@nokia.com kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes to reduce size of the VLC tables used in coding the last active DCT coefficient position in the LCEC entropy coder for block sizes 8x8 and above. In HM 2.0 the total size of these tables is 1024 bytes while the proposed approach reduces the size to 190 bytes. The reduced total table size is attributed to two things: reducing the number of tables from 8 to 5 and grouping some of the positions together to reduce the size of the individual tables. The reported effect to coding efficiency is 
0.00 % / -0.04 % / 0.06 % for LC Intra / LC random access / LC low delay configurations, respectively.

# Introduction

HM 2.0 / LCEC codes position of the last active coefficient in a transform block utilizing adaptive VLC tables. There are 8 different tables applied for different block types. The size of each of these tables is 128 bytes (64 entries for each of the possible position of the last active coefficient “last_pos”, combined with indication of whether the amplitude of the last active coefficient is 1 or above doubling the table size).

# Proposal

The first aspect of this proposal is to reduce the number of tables used in (de)coding of the last_pos information. This is achieved by utilizing a single table for the chroma components instead of having a separate table for the two and also combining the tables for uni-predicted and bi-predicted inter slices. The table below summarizes the different table types in HM 2.0 and in the proposed approach.

| HM 2.0 |  | Proposed |  |
| --- | --- | --- | --- |
| HM 2.0 block type | Table index | Block type | Table index |
| U | 0 | Chroma | 0 |
| V | 1 | 8x8 Intra Y | 1 |
| 8x8 Intra Y | 2 | 8x8 Inter Y | 2 |
| 8x8 Inter Y Uni | 3 | 16x16+ Intra Y | 3 |
| 8x8 Inter Y Bi | 4 | 16x16+ Inter Y | 4 |
| 16x16+ Intra Y | 5 |  |  |
| 16x16+ Inter Y Uni | 6 |  |  |
| 16x16+ Inter Y Bi | 7 |  |  |

With this grouping the total number of tables is reduced from 8 to 5.

The second aspect of the proposal is to reduce the sizes of the individual tables. The three first coefficient positions in zigzag order (DC, first horizontal and first vertical AC coefficients) are allocated individual entries in the adaptive VLC tables. The remaining positions are grouped into groups of four and each group is assigned one entry in the VLC tables. The remaining highest AC coefficient is also allocated an individual entry. If the last active coefficient for a given block belongs to one of the grouped positions, the indication of the group is followed by a 2-bit fixed length code identifying the individual position of the coefficient. The grouping is demonstrated in the picture below.

Figure 1: Proposed grouping of the last_pos identifiers. Groups 3 through 17 are followed by a 2-bit fixed length code to identify which of the four is the position of the last active coefficient.

The information indicating whether the last active coefficient has an absolute value of one or larger than one is indicated as part of the same table identical to how it is done in HM 2.0 (doubling the table size now from 19 to 38). This way the size of the individual tables are reduced from 2*64 bytes to 2*19 bytes.

# Coding results

The table below summarizes coding efficiency results for the Low Complexity configurations. High Efficiency results are not affected as the proposal only concerns the HM 2.0 low complexity entropy coder. Source code of the implementation is attached to this contribution and modifications with respect to the HM 2.0 anchor can be found under the NOK_LAST_POS_8x8 macro.

|  | Intra LoCo |  |  |  | Random access LoCo |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.12 | 0.14 | 0.01 | 0.06 | -0.28 | -0.45 |  |  |  |
| Class B | 0.03 | 0.15 | 0.06 | 0.05 | 0.15 | -0.12 | 0.27 | 0.12 | 0.24 |
| Class C | -0.07 | 0.04 | -0.02 | -0.24 | -0.22 | -0.13 | -0.21 | -0.22 | -0.24 |
| Class D | -0.09 | -0.06 | -0.08 | -0.08 | 0.05 | 0.02 | 0.04 | 0.15 | 0.29 |
| Class E | 0.00 | 0.07 | -0.01 |  |  |  | 0.11 | 0.47 | 0.01 |
| All | 0.00 | 0.07 | -0.01 | -0.04 | -0.06 | -0.17 | 0.06 | 0.11 | 0.09 |
| Enc Time[%] | 101 % |  |  | 100 % |  |  | 100 % |  |  |
| Dec Time[%] | 101 % |  |  | 98 % |  |  | 98 % |  |  |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).