| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E243 |
| --- | --- |

| Title: | Transform design for HEVC with 16 bit intermediate data representation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Fuldseth, Cisco Systems Norway G. Bjøntegaard, Cisco Systems Norway M. Sadafale, Texas Instruments Inc., USA M. Budagavi, Texas Instruments Inc., USA | Tel: Email: Tel: Email Tel: Email: Tel: Email: | +47 92414082 arild.fuldseth@cisco.com gbjonteg@cisco.com mangesh@ti.com +1-214-480-1861 madhukar@ti.com |
| Source: | Cisco Systems, Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a set of transform matrices for HEVC, covering all transform sizes from 4x4 to 32x32. The proposed transforms are intended to replace the existing transforms specified in HM2.0. The proposed transform design has the following properties; 16 bit data representation before and after each transform stage (applies to both IBDI off and IBDI on), 16 bit multipliers for all internal multiplications, no need for correction of different norms of basis vectors during quantization/dequantization, all transform sizes above 4x4 can reuse arithmetic operations for smaller transform sizes, and implementations using either pure matrix multiplication or a combination of matrix multiplication and butterfly structures are possible. The proposed transforms are implemented without transform precision extension (TPE) and compared with the existing HM2.0 transforms with TPE=2 and/or IBDI=2. For normal QP range (22-37) and high QP range (36-51), the average BD-rate gains vary between 0.1% and 0.6% depending on the test condition (low complexity/high efficiency, intra/random access/low delay). For low QP range (1-13), the average BD-rate gains vary between 0.8% and 2.5%.

# Introduction

In [1] and [2] similar transform designs facilitating matrix multiplication and 16 bit intermediate data representations were proposed independently by Cisco and TI. This contribution combines both contributions into a joint proposal.

In HM2.0 [3], the transforms specified for TUs of size 4x4 and 8x8 are the same as in AVC. For TUs of size 16x16 and 32x32, an approximate implementation of the Chen algorithm for DCT is used. The intention of this contribution is to improve the existing transforms in several aspects.

- The current 16x16 and 32x32 transforms uses a bit width of 20 or more before each stage of the inverse transform. With TPE, this number is increased correspondingly for the intermediate stage. The proposed transform design does not rely on TPE and chooses intermediate scaling carefully to limit the bit width to 16 at all intermediate stages. The bit width is limited to 16 also when IBDI is enabled.

- Also, the current 16x16 and 32x32 transforms use multipliers that exceeds 16 bit for internal calculations. On most software platforms, this implies 32 bit multipliers. The proposed inverse transforms use 16 bit multipliers for all internal calculations. This is true even with IBDI/TPE enabled.

- The current transforms have different norms for different basis vectors. This complicates quantization/dequantization since different scaling needs to be used for different transform coefficients (i.e. quantization matrices). With the proposed transforms, the norms of the basis vectors are sufficiently close so that the same scaling can be used for all transform coefficients during quantization/dequantization. Also, the scaling is the same for all transform sizes, except for right shifts that depend on the transform size.

- Coefficients and basis vectors of the smaller transforms are a subset of coefficients and basis vectors of larger transforms. This allows for re-use of logic for multiply/adds in hardware design.

- As opposed to the current 16x16 and 32x32 transforms, the proposed transform design is specified in terms of an NxN matrix of transform coefficients and also the right shift to be used after each transform stage. This allows for more additional flexibility in implementations, such as direct matrix multiplications with SIMD instruction for the innermost loop, or multi-stage butterfly implementations utilizing symmetry properties.

In addition, the proposed transforms have some properties that are also considered desirable:

- 8 bit representation of transform coefficients.

- Bit width of accumulators for matrix multiplication (before right shift and storage) does not exceed 32 bit.

- Same symmetry/anti-symmetry properties as the DCT transform.

- Scaled transform coefficients are close to the DCT transform.

- Basis vectors are almost orthogonal.

- Same quantization/dequantization scheme for all transform sizes where the multipliers depend on the value of QP and the shifts depends only on log2(N).

# Description

2.1 Basis vectors

The basis vectors of the 4x4 and 8x8 transforms are shown below. The basis vectors of the 16x16 and 32x32 transforms are shown in Appendix A. We have also designed a 64x64 transform that can be made available in the software for experimental purposes.

4x4 transform:

{64, 64, 64, 64}

{83, 36,-36,-83}

{64,-64,-64, 64}

{36,-83, 83,-36}

8x8 transform:

{64, 64, 64, 64, 64, 64, 64, 64}

{89, 75, 50, 18,-18,-50,-75,-89}

{83, 36,-36,-83,-83,-36, 36, 83}

{75,-18,-89,-50, 50, 89, 18,-75}

{64,-64,-64, 64, 64,-64,-64, 64}

{50,-89, 18, 75,-75,-18, 89,-50}

{36,-83, 83,-36,-36, 83,-83, 36}

{18,-50, 75,-89, 89,-75, 50,-18}

2.2 Intermediate scaling and quantization/dequantization.

In this subsection, intermediate scaling and quantization/dequantization are described in more detail. In addition to specifying the scaling in the dequantization and the inverse transform, we also specify the forward transform and quantizer that we have used for our simulation results.

Definitions

B	= source bit width (8 or 10 bit in the experiments described below)

DB	= B-8 (internal bit-depth increase with 8-bit input)

N 	= transform size

M	= log2(N)

Q	= f(QP%6), 	where f(x)   =   {26214,23302,20560,18396,16384,14564}, x=0,…,5

IQ 	= g(QP%6), 	where g(x)  =   {40,45,51,57,64,72}, x=0,…,5

Forward transform

The following applies to the implementation of the forward transform.

- The forward transform is specified as a matrix multiplication in two stages, horizontal and vertical.

- After the first stage, the resulting transform coefficients are scaled as follows

y  =  (x + offset)>>(M-1+DB),		offset = (1<<(M-2+DB))

- After the second stage, the resulting transform coefficients, coeff, are scaled as follows

y  =  (x + offset)>>(M+6),		offset = (1<<(M+5))

This ensures a bit width of 16 after each transform stage.

Quantization

Quantization can be implemented as follows (RDOQ=off):

level  =  (coeff*Q + offset)>>(21+QP/6–M–DB)

For RDOQ, the scaling and cost function calculations are adjusted accordingly.

Dequantization

The dequantizer is specified as follows

coeffQ 	=  ((level*IQ << (QP/6)) + offset)>>(M-1+DB),  	offset = 1<<(M-2+DB)

coeffQ = min(32767,max(-32768,coeffQ))

The clipping ensures that the value of the quantized transform coefficients, coeffQ, can be represented with 16 bits. The scaling is chosen so that clipping only occur for extreme input values and/or unreasonable quantizer behaviour.

Inverse transform

The following applies to the implementation of the inverse transform.

- The inverse transform is specified as a matrix multiplication in two stages, horizontal and vertical.

- After the first stage, the resulting transform coefficients are scaled as follows

y  =  (x + offset)>>7			offset = 1<<(7-1)

This ensures a bit width of 16 after the first inverse transform stage assuming the quantization/dequantization scheme above.

- After the second stage, the resulting transform coefficients are scaled as follows

y  =  (x + offset)>>(12-DB)		offset = 1<<(11-DB)

Alternatively, one could use clipping between the first and the second transform stage. This would allow one bit of right shift to be moved from the first to the second transform stage as follows

- After the first stage:

y  =  (x + offset)>>6			offset = 1<<(6-1)

y = min(32767,max(-32768,y))

- After the second stage:

y  =  (x + offset)>>(13-DB)		offset = 1<<(12-DB)

Quantization step size and QP

The quantizer multipliers, f(x) and dequantizer multipliers, g(x) are chosen so that

f(x) * g(x)  (1<<20), 	x = 0,1,…,5

Furthermore, assume that we have an N*N DC block with pixel values equal to X.  If this block is input to an orthonormal transformation containing a DC basis vector (e.g. the Discrete Cosine Transform), the value of the DC coefficient is equal to X*N.  With the proposed transforms and quantization/ dequantiztion scheme, the same relationship between input pixel values and the value of level applies when QP = 4. In other words, if all pixel values in an NxN block are equal to X, and QP=4, this will result in a DC coefficient with level = X*N after quantization. The range of QP values maps to approximately the same range of quantization step sizes as in HM2.0.

Modifications compared to JCTVC-D224

The current proposal is identical to the proposal of JCTVC-D224 except for the following modifications:

- Reduced bit depth of Q and IQ matrices for more efficient hw implementation.

- Clipping after dequantizer to accommodate random quantizers.

- One extra shift after the first inverse transform stage to accommodate reasonable quantizers (i.e. quantizes rounding upwards to at most the closest representation level.

- Intermediate bit width is limited to 16 bit also when IBDI is enabled.

# Matrix multiplication and partial butterfly

The proposed transforms can be implemented in two ways; matrix multiplications, and partial butterfly.

3.1 Matrix multiplication

With straightforward matrix multiplication, the number of operations for each transform stage is N3 multiplications and N2(N-1) additions. Note however, that matrix multiplication is particularly well suited for SIMD instructions. Assuming a SIMD width of N, the matrix multiplication can be implemented using N2 SIMD instructions.

3.2 Partial butterfly

The basis vectors shown in section 2 and in Appendix A have symmetry properties that are inherited from the DCT transform:

- Even rows with index 0,2,4,… are symmetric with a symmetry point before coefficient no. N/2.

- Odd rows with index 1,3,5,7,… are anti-symmetric with a symmetry point before coefficient no. N/2.

- Even rows with index 0,4,8,… have additional symmetry points before coefficients no. N/4 and 3N/4.

- Even rows with index 2,6,10,… have additional anti-symmetry points before coefficients no. N/4 and 3N/4.

For larger transforms this pattern repeats itself with additional symmetry and anti-symmetry points before coefficients n*N/8, n*N/16 etc.

These symmetry properties allows for using parts of the “butterfly” structures used for fast FFT-like algorithms to reduce the number of multiply/adds (e.g. the Chen algorithm). Thus, we refer to this implementation as partial butterfly.

In our software both methods are supported, matrix multiplications for readability and partial butterfly for speed.

# Results

Table 1 shows BD-rate results for the new transforms using HM2.0 as the anchor for common test conditions [4]. As can be seen from the table, there is a small but consistent gain, ranging from 0.1% to 0.5%.

Table 2 shows BD-rate results for common test conditions, but with low QP values, (QP=1, 5, 9, 13). In some extreme cases, (e.g. low QP range, Intra, low complexity, BQTerrace & BasketballDrive), a BD-rate gain of 5.0% is observed. Similarly, for QP=1, Intra, low complexity, Cactus, a PSNR increase of about 6 dB is measured (see attached spreadsheets).

Table 3 shows BD-rate results for common test conditions, but with high QP values, (QP=36, 42, 47, 51).

For the sake of completeness, results for 18 bit intermediate storage and processing for high efficiency settings (IBDI=2) are enclosed. The results are shown in Table 4, Table 5, and Table 6 for normal QP, low QP, and high QP ranges respectively. As can be seen from the results, at low QP, the gain of using 18 bit intermediate storage is 0.2%, 0.1%, and 0.1% for intra, random access, and low delay respectively. For normal and high QP values, the gain of using 18 bit is negligible. Based on these results we propose to use 16 bit for intermediate storage whether IBDI is enabled or not.

Finally, results using clipping instead of one extra right shift after the first inverse transform stage are shown in Tables 7 through Table 9. The results show only negligible differences (<0.1%)  between doing clipping or not.

Table 1. BD-rate results for new transforms, normal QP range (QP=22, 27, 32, 37)

|  | Intra |  |  |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate |  | U BD-rate | V BD-rate |  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0,4 |  | -0,5 | -0,5 |  | -1,0 | -0,3 | -0,2 |
| Class B | -0,3 |  | -0,1 | -0,2 |  | -0,5 | -0,3 | -0,3 |
| Class C | -0,1 |  | 0,0 | 0,0 |  | -0,3 | -0,3 | -0,4 |
| Class D | -0,1 |  | 0,0 | 0,1 |  | -0,2 | -0,3 | -0,4 |
| Class E | -0,3 |  | 0,3 | 0,0 |  | -0,6 | 0,0 | -0,1 |
| All | -0,3 |  | -0,1 | -0,1 |  | -0,5 | -0,2 | -0,3 |
| Enc Time[%] | 106 % |  |  |  |  | 113 % |  |  |
| Dec Time[%] | 106 % |  |  |  |  | 111 % |  |  |
|  | Random access |  |  |  |  | Random access LoCo |  |  |
|  | Y BD-rate |  | U BD-rate | V BD-rate |  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0,2 |  | 0,1 | -0,2 |  | -0,6 | 0,2 | 0,2 |
| Class B | -0,2 |  | 0,2 | 0,2 |  | -0,3 | 0,2 | 0,3 |
| Class C | -0,1 |  | 0,1 | 0,0 |  | -0,1 | 0,1 | 0,1 |
| Class D | 0,0 |  | 0,2 | 0,3 |  | -0,1 | 0,3 | 0,2 |
| Class E |  |  |  |  |  |  |  |  |
| All | -0,1 |  | 0,2 | 0,1 |  | -0,3 | 0,2 | 0,2 |
| Enc Time[%] | 100 % |  |  |  |  | 101 % |  |  |
| Dec Time[%] | 98 % |  |  |  |  | 104 % |  |  |
|  | Low delay |  |  |  |  | Low delay LoCo |  |  |
|  | Y BD-rate |  | U BD-rate | V BD-rate |  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0,2 |  | 0,1 | -0,1 |  | -0,2 | -0,4 | 0,0 |
| Class C | -0,1 |  | 0,2 | 0,2 |  | -0,2 | -0,1 | -0,3 |
| Class D | -0,1 |  | 0,0 | 0,3 |  | 0,0 | 0,0 | 0,0 |
| Class E | -0,4 |  | -0,6 | 0,4 |  | -0,2 | 0,0 | -0,5 |
| All | -0,2 |  | -0,1 | 0,2 |  | -0,1 | -0,2 | -0,2 |
| Enc Time[%] | 98 % |  |  |  |  | 101 % |  |  |
| Dec Time[%] | 98 % |  |  |  |  | 103 % |  |  |

Table 2. BD-rate results for new transforms, low QP range (QP=1, 5, 9, 13)

|  | Intra |  |  | Intra LoCo |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | -1,4 | -1,5 | -1,5 | -2,4 |  | -1,5 | -1,5 |  |
| Class B | -1,5 | -2,0 | -2,1 | -3,4 |  | -1,8 | -1,7 |  |
| Class C | -0,3 | -0,5 | -0,5 | -2,0 |  | -1,0 | -0,9 |  |
| Class D | -0,4 | -0,5 | -0,5 | -1,9 |  | -1,2 | -1,1 |  |
| Class E | -2,2 | -2,4 | -2,4 | -2,4 |  | -1,7 | -1,9 |  |
| All | -1,1 | -1,3 | -1,4 | -2,5 |  | -1,4 | -1,4 |  |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |  |  |
|  | Random access |  |  | Random access LoCo |  |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | -0,9 | -1,0 | -0,8 | -1,6 |  | -0,9 | -0,9 |  |
| Class B | -1,1 | -1,1 | -1,3 | -2,0 |  | -0,8 | -0,8 |  |
| Class C | -0,6 | -0,3 | -0,4 | -1,4 |  | -0,7 | -0,9 |  |
| Class D | -0,5 | 0,0 | -0,1 | -1,4 |  | -0,8 | -0,9 |  |
| Class E |  |  |  |  |  |  |  |  |
| All | -0,8 | -0,6 | -0,7 | -1,6 |  | -0,8 | -0,8 |  |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -1,1 | -0,8 | -1,0 | -2,3 |  | -0,6 | -0,7 |  |
| Class C | -0,7 | -0,3 | -0,3 | -1,6 |  | -0,6 | -0,7 |  |
| Class D | -0,7 | 0,1 | 0,0 | -1,5 |  | -0,5 | -0,6 |  |
| Class E | -1,1 | -0,9 | -0,8 | -1,8 |  | -1,0 | -1,3 |  |
| All | -0,9 | -0,5 | -0,5 | -1,9 |  | -0,7 | -0,8 |  |
| Enc Time[%] | 103 % |  |  | 103 % |  |  |  |  |
| Dec Time[%] | 101 % |  |  | 101 % |  |  |  |  |

Table 3. BD-rate results for new transforms, high QP range (QP=36, 42, 47, 51)

|  | Intra |  |  |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate |  | U BD-rate | V BD-rate |  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0,1 |  | 0,1 | 0,4 |  | -0,2 | -1,4 | -1,1 |
| Class B | -0,1 |  | -0,2 | 0,5 |  | -0,1 | -1,0 | -1,0 |
| Class C | -0,1 |  | 0,1 | -0,1 |  | -0,1 | -1,4 | -0,9 |
| Class D | -0,1 |  | 0,3 | -0,2 |  | -0,1 | -1,7 | -0,7 |
| Class E | 0,0 |  | 0,5 | 0,0 |  | -0,2 | -0,5 | -0,7 |
| All | -0,1 |  | 0,1 | 0,1 |  | -0,1 | -1,2 | -0,9 |
| Enc Time[%] | #NUM! |  |  |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |  |  | #NUM! |  |  |
|  | Random access |  |  |  |  | Random access LoCo |  |  |
|  | Y BD-rate |  | U BD-rate | V BD-rate |  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0,2 |  | -0,3 | 0,2 |  | -0,1 | -0,1 | -0,3 |
| Class B | 0,0 |  | -1,3 | 1,3 |  | -0,1 | -0,7 | -0,9 |
| Class C | -0,4 |  | 0,5 | 0,5 |  | 0,0 | -0,6 | -1,1 |
| Class D | 0,0 |  | 0,7 | -3,0 |  | -0,2 | -1,8 | -2,3 |
| Class E |  |  |  |  |  |  |  |  |
| All | -0,2 |  | -0,2 | -0,2 |  | -0,1 | -0,9 | -1,2 |
| Enc Time[%] | #NUM! |  |  |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |  |  | #NUM! |  |  |
|  | Low delay |  |  |  |  | Low delay LoCo |  |  |
|  | Y BD-rate |  | U BD-rate | V BD-rate |  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0,2 |  | -0,7 | -1,8 |  | -0,1 | -2,2 | 0,3 |
| Class C | -0,1 |  | 1,0 | -1,1 |  | 0,0 | -2,8 | -3,0 |
| Class D | 0,1 |  | -1,8 | -0,2 |  | -0,4 | -2,7 | -1,3 |
| Class E | -0,5 |  | -2,2 | -1,9 |  | -0,2 | -1,6 | -0,6 |
| All | -0,2 |  | -0,8 | -1,2 |  | -0,2 | -2,4 | -1,1 |
| Enc Time[%] | 103 % |  |  |  |  | 102 % |  |  |
| Dec Time[%] | 101 % |  |  |  |  | 104 % |  |  |

Table 4. BD-rate results for new transforms, 18 bit, normal QP range (QP=22, 27, 32, 37)

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0,4 | -0,5 | -0,5 |
| Class B | -0,3 | -0,1 | -0,2 |
| Class C | -0,1 | 0,0 | 0,0 |
| Class D | -0,1 | 0,0 | 0,1 |
| Class E | -0,3 | 0,2 | -0,1 |
| All | -0,3 | -0,1 | -0,1 |
| Enc Time[%] | 105 % |  |  |
| Dec Time[%] | 105 % |  |  |
|  | Random access |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0,1 | 0,4 | -0,1 |
| Class B | -0,2 | 0,1 | 0,2 |
| Class C | -0,1 | 0,0 | 0,1 |
| Class D | 0,0 | 0,2 | 0,3 |
| Class E |  |  |  |
| All | -0,1 | 0,2 | 0,1 |
| Enc Time[%] | 99 % |  |  |
| Dec Time[%] | 97 % |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -0,2 | 0,2 | -0,1 |
| Class C | -0,2 | 0,1 | 0,1 |
| Class D | -0,1 | 0,3 | 0,4 |
| Class E | -0,4 | -0,4 | 0,6 |
| All | -0,2 | 0,1 | 0,2 |
| Enc Time[%] | 100 % |  |  |
| Dec Time[%] | 99 % |  |  |

Table 5. BD-rate results for new transforms, 18 bit, low QP range (QP=1, 5, 9, 13)

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1,6 | -1,8 | -1,8 |
| Class B | -1,9 | -2,3 | -2,3 |
| Class C | -0,4 | -0,6 | -0,6 |
| Class D | -0,5 | -0,5 | -0,6 |
| Class E | -2,5 | -2,8 | -2,8 |
| All | -1,3 | -1,5 | -1,6 |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Random access |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1,1 | -1,1 | -0,9 |
| Class B | -1,3 | -1,3 | -1,5 |
| Class C | -0,7 | -0,4 | -0,4 |
| Class D | -0,6 | -0,1 | -0,2 |
| Class E |  |  |  |
| All | -0,9 | -0,7 | -0,8 |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -1,2 | -0,9 | -1,2 |
| Class C | -0,8 | -0,3 | -0,3 |
| Class D | -0,8 | 0,1 | 0,0 |
| Class E | -1,3 | -1,0 | -1,0 |
| All | -1,0 | -0,5 | -0,7 |
| Enc Time[%] | 103 % |  |  |
| Dec Time[%] | 100 % |  |  |

Table 6. BD-rate results for new transforms, 18 bit, high QP range (QP=36, 42, 47, 51)

|  | Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y BD-rate |  | U BD-rate | V BD-rate |
| Class A | -0,1 |  | 0,2 | 0,7 |
| Class B | -0,1 |  | -0,1 | 0,5 |
| Class C | -0,1 |  | -0,2 | -0,3 |
| Class D | -0,1 |  | 0,5 | -0,4 |
| Class E | 0,0 |  | 0,4 | -0,2 |
| All | -0,1 |  | 0,1 | 0,1 |
| Enc Time[%] | #NUM! |  |  |  |
| Dec Time[%] | #NUM! |  |  |  |
|  | Random access |  |  |  |
|  | Y BD-rate | U BD-rate |  | V BD-rate |
| Class A | -0,2 | 0,3 |  | 0,7 |
| Class B | 0,0 | -0,8 |  | 0,3 |
| Class C | -0,2 | -0,5 |  | 0,1 |
| Class D | 0,0 | 0,4 |  | -1,6 |
| Class E |  |  |  |  |
| All | -0,1 | -0,3 |  | -0,2 |
| Enc Time[%] | #NUM! |  |  |  |
| Dec Time[%] | #NUM! |  |  |  |
|  | Low delay |  |  |  |
|  | Y BD-rate | U BD-rate |  | V BD-rate |
| Class A |  |  |  |  |
| Class B | -0,2 | -0,4 |  | 0,2 |
| Class C | 0,1 | 0,2 |  | -2,2 |
| Class D | 0,0 | -1,3 |  | -0,3 |
| Class E | -0,6 | -2,1 |  | -2,0 |
| All | -0,1 | -0,8 |  | -0,9 |
| Enc Time[%] | 103 % |  |  |  |
| Dec Time[%] | 99 % |  |  |  |

Table 7. BD-rate results for new transforms, clipping, normal QP range (QP=22, 27, 32, 37)

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0,4 | -0,5 | -0,5 | -1,0 | -0,3 | -0,2 |
| Class B | -0,3 | -0,1 | -0,2 | -0,5 | -0,3 | -0,3 |
| Class C | -0,1 | 0,0 | 0,0 | -0,3 | -0,3 | -0,5 |
| Class D | -0,1 | 0,0 | 0,1 | -0,2 | -0,3 | -0,3 |
| Class E | -0,3 | 0,2 | -0,1 | -0,6 | -0,1 | -0,1 |
| All | -0,3 | -0,1 | -0,1 | -0,5 | -0,3 | -0,3 |
| Enc Time[%] | 111 % |  |  | 110 % |  |  |
| Dec Time[%] | 113 % |  |  | 110 % |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0,1 | 0,1 | -0,1 | -0,6 | 0,0 | 0,2 |
| Class B | -0,2 | 0,2 | 0,4 | -0,3 | 0,2 | 0,3 |
| Class C | -0,1 | 0,1 | 0,0 | -0,2 | 0,0 | 0,2 |
| Class D | 0,1 | 0,3 | 0,1 | -0,1 | 0,2 | 0,2 |
| Class E |  |  |  |  |  |  |
| All | -0,1 | 0,2 | 0,1 | -0,3 | 0,1 | 0,2 |
| Enc Time[%] | 103 % |  |  | 101 % |  |  |
| Dec Time[%] | 108 % |  |  | 103 % |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0,2 | -0,2 | 0,2 | -0,2 | -0,4 | 0,0 |
| Class C | -0,1 | 0,3 | 0,2 | -0,1 | -0,3 | -0,3 |
| Class D | -0,1 | 0,4 | 0,1 | 0,0 | -0,2 | -0,2 |
| Class E | -0,4 | -0,2 | -0,1 | -0,1 | -0,1 | -0,2 |
| All | -0,2 | 0,1 | 0,1 | -0,1 | -0,3 | -0,2 |
| Enc Time[%] | 105 % |  |  | 105 % |  |  |
| Dec Time[%] | 114 % |  |  | 119 % |  |  |

Table 8. BD-rate results for new transforms, clipping, low QP range (QP=1, 5, 9, 13)

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1,4 | -1,5 | -1,5 | -2,4 | -1,6 | -1,5 |
| Class B | -1,5 | -2,0 | -2,1 | -3,5 | -1,9 | -1,7 |
| Class C | -0,3 | -0,5 | -0,5 | -2,1 | -1,0 | -0,9 |
| Class D | -0,4 | -0,5 | -0,5 | -2,0 | -1,2 | -1,1 |
| Class E | -2,2 | -2,4 | -2,5 | -2,4 | -1,7 | -2,0 |
| All | -1,1 | -1,4 | -1,4 | -2,5 | -1,5 | -1,4 |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1,0 | -1,0 | -0,8 | -1,6 | -0,9 | -0,9 |
| Class B | -1,1 | -1,1 | -1,3 | -2,0 | -0,8 | -0,8 |
| Class C | -0,6 | -0,3 | -0,4 | -1,4 | -0,7 | -0,9 |
| Class D | -0,5 | 0,0 | -0,1 | -1,4 | -0,8 | -0,9 |
| Class E |  |  |  |  |  |  |
| All | -0,8 | -0,6 | -0,7 | -1,6 | -0,8 | -0,9 |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1,1 | -0,8 | -1,1 | -2,4 | -0,6 | -0,7 |
| Class C | -0,7 | -0,3 | -0,3 | -1,6 | -0,6 | -0,7 |
| Class D | -0,7 | 0,1 | 0,0 | -1,6 | -0,6 | -0,7 |
| Class E | -1,2 | -1,0 | -0,8 | -1,8 | -1,1 | -1,1 |
| All | -0,9 | -0,5 | -0,6 | -1,9 | -0,7 | -0,8 |
| Enc Time[%] | 103 % |  |  | 101 % |  |  |
| Dec Time[%] | 101 % |  |  | 100 % |  |  |

Table 9. BD-rate results for new transforms, clipping, high QP range (QP=36, 42, 47, 51)

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0,2 | 0,2 | 0,6 | -0,2 | -1,6 | -1,0 |
| Class B | -0,1 | -0,2 | 0,3 | -0,1 | -1,2 | -0,7 |
| Class C | 0,0 | -0,1 | -0,3 | 0,0 | -1,4 | -0,8 |
| Class D | 0,0 | 0,4 | -0,4 | -0,1 | -1,7 | -0,8 |
| Class E | 0,0 | 0,5 | 0,1 | -0,2 | -0,4 | -0,7 |
| All | -0,1 | 0,1 | 0,0 | -0,1 | -1,2 | -0,8 |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0,3 | 0,1 | 0,8 | -0,3 | -0,5 | -0,7 |
| Class B | -0,1 | -0,9 | 0,7 | -0,1 | -0,8 | -1,2 |
| Class C | -0,2 | 0,5 | 0,4 | 0,0 | -0,6 | -1,0 |
| Class D | -0,1 | 0,3 | -2,5 | 0,0 | -2,1 | -2,0 |
| Class E |  |  |  |  |  |  |
| All | -0,1 | -0,1 | -0,2 | 0,0 | -1,0 | -1,3 |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0,3 | -0,7 | 0,4 | -0,2 | -3,0 | 0,3 |
| Class C | 0,1 | 1,7 | 0,0 | 0,1 | -3,1 | -2,1 |
| Class D | -0,1 | -0,5 | -0,1 | -0,4 | -2,2 | 0,0 |
| Class E | -0,5 | -1,5 | -3,0 | -0,2 | -1,1 | -1,0 |
| All | -0,2 | -0,2 | -0,5 | -0,2 | -2,5 | -0,6 |
| Enc Time[%] | 103 % |  |  | 101 % |  |  |
| Dec Time[%] | 101 % |  |  | 104 % |  |  |

# Complexity analysis

This section provides a detailed complexity analysis. It is structured to comply with section B.2 the core experiment description on core transform design [5]. Some of this complexity analysis uses a standalone test program.

5.1 Arithmetic operations

The numbers of arithmetic operations for the 2D inverse transform are shown in Table 10. It should be noted that all multiplications can be achieved using 16 bit multipliers.

Table 10. Number of arithmetic operations for the 2D inverse transform

|  | Matrix multiplication |  |  | Partial butterfly |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Transform size | Mult | Add | Shift | Mult | Add | Shift |
| 4x4 | 128 | 128 | 32 | 48 | 64 | 32 |
| 8x8 | 1024 | 1024 | 64 | 352 | 448 | 64 |
| 16x16 | 8192 | 8192 | 256 | 2752 | 3200 | 256 |
| 32x32 | 65536 | 65536 | 2048 | 21888 | 23808 | 2048 |

5.2 Software execution speed

Although the core experiment document [6] suggests measuring execution speed of “non-optimized” software using C or C++, it was a common understanding among CE participants that it was more meaningful to report execution speed of optimized software based on SIMD.  This is reported in section 5.4.

5.3 Dynamic range analysis

The proposed transforms and the associated shifts/clipping are designed to ensure a bit width equal to 16 at all intermediate stages:

Forward transform, first stage
The scaling is adjusted to stay within 16 bit for worst case input signal.

Forward transform, second stage
The scaling is adjusted to stay within 16 bit for worst case input signal.

Dequantizer	
The scaling is chosen to stay within 16 bit for worst case input signal without quantization. Additional clipping is adjusted to stay within 16 bit for randomized quantizers.

Inverse transform, first stage
The scaling is chosen to stay within 16 bit for worst case input signal and reasonable quantizers. Alternatively, one can do clipping between the first and the second stage and adjust the shifts by one bit as described in section 2.2.

This design is confirmed by analysis by the standalone program using random test vectors. Also, the BD-rate measurements are performed using 16 bit variables (short). This means that any overflow would have had a severe impact on BD-rate results.

5.4 Parallelizability with SIMD

The standalone test program used in the core experiment was modified to accommodate SIMD generation by the Intel compiler. This was achieved mostly by writing the C source code to help the Intel compiler generate SIMD instructions. In one case (transpose implementation), intrinsics were used.The resulting execution speeds are shown in Table 11. The number of clock cycles per 2D (inverse) transform are listed in Table 12. It is our opinion that further significant speedup can be achieved by more elaborate use of intrinsics.

Details for the CPU/compiler are:

- CPU:		Intel Core i5 670, 3.6 GHz

- Compiler:	Intel C++ Compiler Professional Edition 12.0.

Table 11. Software execution speed with SIMD optimization (seconds)

|  |  | Matrix Multiplication |  | Partial Butterfly |  |
| --- | --- | --- | --- | --- | --- |
| Transform size | No. of iterations | Forward | Inverse | Forward | Inverse |
| 8x8 | 2^24 | 3.8 | 3.7 | 1.6 | 1.4 |
| 16x16 | 2^22 | 4.1 | 4.1 | 2.3 | 1.8 |
| 32x32 | 2^20 | 5.7 | 5.5 | 4.0 | 3.1 |

Table 13. Clock ticks per 2D transforms with SIMD optimization

|  | Matrix Multiplication |  | Partial Butterfly |  |
| --- | --- | --- | --- | --- |
| Transform size | Forward | Inverse | Forward | Inverse |
| 8x8 | 816 | 801 | 351 | 297 |
| 16x16 | 3521 | 3547 | 1969 | 1606 |
| 32x32 | 19576 | 19144 | 13803 | 10623 |

5.5 Number of sequential operations

For matrix multiplication implementation, the number of cascaded operators (multipliers/adders) in the longest execution path is 1 multiplication followed by 1 addition for all sizes of inverse transform.

For partial butterfly implementation, the number of cascaded operators (multipliers/adders) in the longest execution path is 1 multiplication followed by log2(N) additions, where N is the size of inverse transform.

5.6 HW analysis

TBD

# Conclusions

Integer transforms for HEVC have been proposed. The proposed transforms have several implementation benefits compared to the existing transforms, including

- 16 bit data representation before and after each transform stage independent of IBDI settings.

- Only 16 bit multipliers are needed for internal calculations.

- No need for correction of different norms of basis vectors during quantization/dequantization.

- Reusability of small transforms in larger transforms.

- Allows both matrix multiplication and partial butterfly structure.

BD-rate gains between 0.1% and 2.5% compared to the anchor are observed.

# References

- A. Fuldseth and G. Bjøntegaard, “Unified transform design for HEVC with 16 internal data representation,” JCTVC-D224, Daegu, January, 2011.

- M. Sadafale, M. Budagavi, “Matrix multiplication specification for HEVC transforms,” JCTVC-D036, Daegu, January, 2011.

- T. Wiegand, W.-J. Han, J.-R. Ohm, G. J. Sullivan, “High Efficiency Video Coding (HEVC) text specification Working Draft 2,” JCTVC-D503, Daegu, January, 2011.

- F. Bossen, “Common test conditions and software reference configurations,” JCTVC-D600, Daegu, January, 2011.

- P. Topiwala, M Budagavi, A. Fuldseth, R. Joshi, I.-K. Kim, “CE10 Core transform design,” JCTVC-D610, Daegu, January, 2011.

# Patent rights declaration

Cisco and Texas Instruments may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix A

16x16 transform

{64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64}

{90 87 80 70 57 43 25  9 -9-25-43-57-70-80-87-90}

{89 75 50 18-18-50-75-89-89-75-50-18 18 50 75 89}

{87 57  9-43-80-90-70-25 25 70 90 80 43 -9-57-87}

{83 36-36-83-83-36 36 83 83 36-36-83-83-36 36 83}

{80  9-70-87-25 57 90 43-43-90-57 25 87 70 -9-80}

{75-18-89-50 50 89 18-75-75 18 89 50-50-89-18 75}

{70-43-87  9 90 25-80-57 57 80-25-90 -9 87 43-70}

{64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64}

{57-80-25 90 -9-87 43 70-70-43 87  9-90 25 80-57}

{50-89 18 75-75-18 89-50-50 89-18-75 75 18-89 50}

{43-90 57 25-87 70  9-80 80 -9-70 87-25-57 90-43}

{36-83 83-36-36 83-83 36 36-83 83-36-36 83-83 36}

{25-70 90-80 43  9-57 87-87 57 -9-43 80-90 70-25}

{18-50 75-89 89-75 50-18-18 50-75 89-89 75-50 18}

{ 9-25 43-57 70-80 87-90 90-87 80-70 57-43 25 -9}

32x32 transform

{64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64}

{90 90 88 85 82 78 73 67 61 54 46 38 31 22 13  4 -4-13-22-31-38-46-54-61-67-73-78-82-85-88-90-90}

{90 87 80 70 57 43 25  9 -9-25-43-57-70-80-87-90-90-87-80-70-57-43-25 -9  9 25 43 57 70 80 87 90}

{90 82 67 46 22 -4-31-54-73-85-90-88-78-61-38-13 13 38 61 78 88 90 85 73 54 31  4-22-46-67-82-90}

{89 75 50 18-18-50-75-89-89-75-50-18 18 50 75 89 89 75 50 18-18-50-75-89-89-75-50-18 18 50 75 89}

{88 67 31-13-54-82-90-78-46 -4 38 73 90 85 61 22-22-61-85-90-73-38  4 46 78 90 82 54 13-31-67-88}

{87 57  9-43-80-90-70-25 25 70 90 80 43 -9-57-87-87-57 -9 43 80 90 70 25-25-70-90-80-43  9 57 87}

{85 46-13-67-90-73-22 38 82 88 54 -4-61-90-78-31 31 78 90 61  4-54-88-82-38 22 73 90 67 13-46-85}

{83 36-36-83-83-36 36 83 83 36-36-83-83-36 36 83 83 36-36-83-83-36 36 83 83 36-36-83-83-36 36 83}

{82 22-54-90-61 13 78 85 31-46-90-67  4 73 88 38-38-88-73 -4 67 90 46-31-85-78-13 61 90 54-22-82}

{80  9-70-87-25 57 90 43-43-90-57 25 87 70 -9-80-80 -9 70 87 25-57-90-43 43 90 57-25-87-70  9 80}

{78 -4-82-73 13 85 67-22-88-61 31 90 54-38-90-46 46 90 38-54-90-31 61 88 22-67-85-13 73 82  4-78}

{75-18-89-50 50 89 18-75-75 18 89 50-50-89-18 75 75-18-89-50 50 89 18-75-75 18 89 50-50-89-18 75}

{73-31-90-22 78 67-38-90-13 82 61-46-88 -4 85 54-54-85  4 88 46-61-82 13 90 38-67-78 22 90 31-73}

{70-43-87  9 90 25-80-57 57 80-25-90 -9 87 43-70-70 43 87 -9-90-25 80 57-57-80 25 90  9-87-43 70}

{67-54-78 38 85-22-90  4 90 13-88-31 82 46-73-61 61 73-46-82 31 88-13-90 -4 90 22-85-38 78 54-67}

{64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64}

{61-73-46 82 31-88-13 90 -4-90 22 85-38-78 54 67-67-54 78 38-85-22 90  4-90 13 88-31-82 46 73-61}

{57-80-25 90 -9-87 43 70-70-43 87  9-90 25 80-57-57 80 25-90  9 87-43-70 70 43-87 -9 90-25-80 57}

{54-85 -4 88-46-61 82 13-90 38 67-78-22 90-31-73 73 31-90 22 78-67-38 90-13-82 61 46-88  4 85-54}

{50-89 18 75-75-18 89-50-50 89-18-75 75 18-89 50 50-89 18 75-75-18 89-50-50 89-18-75 75 18-89 50}

{46-90 38 54-90 31 61-88 22 67-85 13 73-82  4 78-78 -4 82-73-13 85-67-22 88-61-31 90-54-38 90-46}

{43-90 57 25-87 70  9-80 80 -9-70 87-25-57 90-43-43 90-57-25 87-70 -9 80-80  9 70-87 25 57-90 43}

{38-88 73 -4-67 90-46-31 85-78 13 61-90 54 22-82 82-22-54 90-61-13 78-85 31 46-90 67  4-73 88-38}

{36-83 83-36-36 83-83 36 36-83 83-36-36 83-83 36 36-83 83-36-36 83-83 36 36-83 83-36-36 83-83 36}

{31-78 90-61  4 54-88 82-38-22 73-90 67-13-46 85-85 46 13-67 90-73 22 38-82 88-54 -4 61-90 78-31}

{25-70 90-80 43  9-57 87-87 57 -9-43 80-90 70-25-25 70-90 80-43 -9 57-87 87-57  9 43-80 90-70 25}

{22-61 85-90 73-38 -4 46-78 90-82 54-13-31 67-88 88-67 31 13-54 82-90 78-46  4 38-73 90-85 61-22}

{18-50 75-89 89-75 50-18-18 50-75 89-89 75-50 18 18-50 75-89 89-75 50-18-18 50-75 89-89 75-50 18}

{13-38 61-78 88-90 85-73 54-31  4 22-46 67-82 90-90 82-67 46-22 -4 31-54 73-85 90-88 78-61 38-13}

{ 9-25 43-57 70-80 87-90 90-87 80-70 57-43 25 -9 -9 25-43 57-70 80-87 90-90 87-80 70-57 43-25  9}
{ 4-13 22-31 38-46 54-61 67-73 78-82 85-88 90-90 90-90 88-85 82-78 73-67 61-54 46-38 31-22 13 -4}

Appendix B

This annex provides proposed text changes to the working draft. Only paragraphs that need to be changed are included in this appendix.

### 8.5.3 Scaling process for transform coefficients

The scaled transform coefficient array dij is derived as follows.

shift = log2(nS) – 1 + trafoPrecisionExt

dij = Clip3(-32768,32767,(( cij * LevelScale[ qP%6 ] << ( qP/6 )) + (1<<(shift-1))>>shift), with i, j = 0..nS-1

### 8.5.4 Transformation process for scaled transform coefficients

The constructed residual samples are derived as specified in the following ordered steps.

- Each (horizontal) row of scaled transform coefficients dij (i, j=0..nS-1) is transformed to eij (i, j=0..nS-1) by invoking the one-dimensional transformation process as specified in subclause  according to the size of the transform unit nS, with the (nS)x(nS) array d as the input and the output is the (nS)x(nS) array e.

- The intermediate sample values gij (i,j=0,..nS-1) are derived by

gij = (eij + 64)>>7

- Each (vertical) column of the resulting matrix eij (i, j=0..nS-1) is transformed to fij (i, j=0..nS-1) by invoking the one-dimensional transformation process as specified in subclause  according to the size of the transform unit nS, with the (nS)x(nS) array e as the input and the output is the (nS)x(nS) array f.

- The residual sample value rij is derived by

rij = ( fij + 211-trafoPrecisionExt ) >> ( 12 - trafoPrecisionExt ), with i, j=0.. (nS)x(nS)-1

#### 8.5.4.1 Transformation process

Inputs of this process are:

- a variable nS specifying the size of the current transform unit,

- an array of nS samples of the scaled transform coefficients x with elements xi, with i = 0..nS-1.

Output of this process is an array of nS samples of the residual samples y with elements yi, with i = 0..nS.

The nS-point one-dimensional inverse transformation process is defined as following:

yi 	=	ci0*x0 + ci1*x1 + … + cinS-1*xnS-1 with i = 0,..,nS-1