| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, 16-23 March, 2011 | Document: JCTVC-E246 WG11 Number: m19770 |
| --- | --- |

| Title: | CE9: Cross-check of experiment B (HHI JCTVC-E204), M27 and M28 (MediaTek JCTVC-E048) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | J. Jung, J.-M. Thiesse Orange Labs 38-40 rue du Général Leclerc 9279 ISSY LES MOULINEAUX | Tel: Email: | +33 1 45 29 53 66 joelb.jung@orange-ftgroup.com jeanmarc.thiesse@orange-ftgroup.com |
| Source: | Orange Labs |  |  |

_____________________________

# Abstract

The document reports the cross-check results of HHI’s proposal JCTVC-E204 (experiment B) and MediaTek’s proposals JCTVC-E048 (experiment M27 and M28). The results provided by HHI for the experiment B of CE9 and by MediaTek for the experiments M27 and M28 of CE9 are accurate and perfectly match the cross-checked results. The source codes were also checked to verify the accuracy of HHI and MediaTek proposals.

# Introduction

This contribution is the verification of the Test B of the contribution JCTVC-E204 by HHI and the verification of the Tests M27 and M28 of the contribution JCTVC-E048 by Mediatek. The softwares have been verified and the experimental results are confirmed.

# Experiment B

The experiment B of the CE9 proposed by HHI in the contribution JCTVC-E204 consists in applying the following modification: Merge mode is used for the SKIP motion vector candidate list, ref_idx and inter_pred_idc are derived from neighbors.

The software has been verified and does not raise any issue.

The cross check of the results have been done for all classes and configurations sequences. Results obtained by HHI and Orange Labs are reported in the following table. A perfect match is achieved.

|  | Random Access Y BD-rate |  | Random Access LoCo Y BD-rate |  | Low Delay Y BD-rate |  | Low Delay LoCo Y BD-rate |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | HHI | Orange Labs | HHI | Orange Labs | HHI | Orange Labs | HHI | Orange Labs |
| ClassA | 0,16 | 0,16 | -0,12 | -0,12 | - | - | - | - |
| ClassB | -0,10 | -0,10 | -0,67 | -0,67 | -0,30 | -0,30 | 1,40 | 1,40 |
| ClassC | -0,09 | -0,09 | -0,30 | -0,30 | -0,12 | -0,12 | 0,70 | 0,70 |
| ClassD | -0,12 | -0,12 | -0,31 | -0,31 | -0,34 | -0,34 | 0,36 | 0,36 |
| ClassE | - | - | - | - | -1,59 | -1,59 | 0,49 | 0,49 |
| AVG | -0,04 | -0,04 | -0,37 | -0,37 | -0,51 | -0,51 | 0,80 | 0,80 |

# Experiment M27

The experiment M27 of the CE9 proposed by MediaTek in the contribution JCTVC-E048 consists in applying the following modifications:

- Inter: modification of the motion vector candidates list, JCTVC-D125 with one single temporal MV predictor H (or Tctr when H not available); predefined priority and MV scaling for deriving each spatial MVP candidate and predefined priority and MV scaling for deriving each temporal MVP candidate.

- Merge: predefined priority and MV scaling for deriving each temporal MVP candidate.

- Skip: modification of the motion vector candidates list, JCTVC-D125 with one single temporal MV predictor H (or Tctr when H not available); predefined priority and MV scaling for deriving each spatial MVP candidate and predefined priority and MV scaling for deriving each temporal MVP candidate.

The software has been verified and does not raise any issue.

The cross check of the results have been done for all classes and configurations sequences. Results obtained by MediaTek and Orange Labs are reported in the following table. A perfect match is achieved.

|  | Random Access Y BD-rate |  | Random Access LoCo Y BD-rate |  | Low Delay Y BD-rate |  | Low Delay LoCo Y BD-rate |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mediatek | Orange Labs | Mediatek | Orange Labs | Mediatek | Orange Labs | Mediatek | Orange Labs |
| ClassA | -1.5 | -1.5 | -2.2 | -2.2 | - | - | - | - |
| ClassB | -0.8 | -0.8 | -1.3 | -1.3 | -1.0 | -1.0 | -2.4 | -2.4 |
| ClassC | -1.4 | -1.4 | -1.7 | -1.7 | -1.4 | -1.4 | -2.2 | -2.2 |
| ClassD | -1.5 | -1.5 | -1.7 | -1.7 | -1.3 | -1.3 | -1.7 | -1.7 |
| ClassE | - | - | - | - | -0.9 | -0.9 | -1.6 | -1.6 |
| AVG | -1.3 | -1.3 | -1.7 | -1.7 | -1.2 | -1.2 | -2.0 | -2.0 |

# Experiment M28

The experiment M28 of the CE9 proposed by MediaTek in the contribution JCTVC-E048 consists in applying the following modifications:

- Inter: modification of the motion vector candidates list, JCTVC-D125 with four temporal MV predictors in competition (H, F, G, Tctr); predefined priority and MV scaling for deriving each spatial MVP candidate and predefined priority and MV scaling for deriving each temporal MVP candidate.

- Merge: predefined priority and MV scaling for deriving each temporal MVP candidate.

- Skip: modification of the motion vector candidates list, JCTVC-D125 with four temporal MV predictors in competition (H, F, G, Tctr); predefined priority and MV scaling for deriving each spatial MVP candidate and predefined priority and MV scaling for deriving each temporal MVP candidate.

The software has been verified and does not raise any issue.

The cross check of the results have been done for all classes and configurations sequences. Results obtained by MediaTek and Orange Labs are reported in the following table. A perfect match is achieved.

|  | Random Access Y BD-rate |  | Random Access LoCo Y BD-rate |  | Low Delay Y BD-rate |  | Low Delay LoCo Y BD-rate |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mediatek | Orange Labs | Mediatek | Orange Labs | Mediatek | Orange Labs | Mediatek | Orange Labs |
| ClassA | -0.8 | -0.8 | -1.2 | -1.2 | - | - | - | - |
| ClassB | -0.8 | -0.8 | -1.1 | -1.1 | -0.9 | -0.9 | -1.9 | -1.9 |
| ClassC | -1.3 | -1.3 | -1.5 | -1.5 | -1.3 | -1.3 | -1.9 | -1.9 |
| ClassD | -1.4 | -1.4 | -1.4 | -1.4 | -1.0 | -1.0 | -1.3 | -1.3 |
| ClassE | - | - | - | - | -0.6 | -0.6 | -1.2 | -1.2 |
| AVG | -1.1 | -1.1 | -1.3 | -1.3 | -1.0 | -1.0 | -1.6 | -1.6 |

# Conclusion

This verification contribution confirms the results obtained by HHI for the experiment B of the CE9 and the results obtained by MediaTek for experiments M27 and M28 of the CE9.