| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E251 WG11 Number: m19777 |
| --- | --- |

| Title: | Decisions for deblocking |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Matthias Narroschke Thomas Wedi Panasonic R&D Center Germany Monzastr. 4c, 63225 Langen, Germany | Tel: Email: | +49 6103 766 204 Matthias.Narroschke@eu.panasonic.com +49 6103 766 1194 Thomas.Wedi@eu.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

In the HM2.0, one single decision for enabling the deblocking is performed for each edge segment of eight columns/lines using two calculated decision values. In contrast to the HM2.0, H.264/MPEG-4 AVC uses eight individual decisions based on eight individually calculated decision values for each edge segment. The change of the decisions to ones similar as in H.264/MPEG-4 AVC can reduce the BD-bit rate by 0.2% in average over all test cases. However, the calculation of additional decision values is associated with additional computational expense. In order to achieve this BD-bit rate reduction at a lower additional computational expense, it is proposed to modify the decisions according to a compromise between the current HM2.0 and H.264/MPEG-4 AVC. The proposed compromise performs eight individual decisions but needs to calculate only four decision values for each edge segment. The same average BD-bit rate reduction of 0.2% is achieved compared to HM2.0 (I-HE: 0.1%, I-LC: 0.1%, RA-HE: 0.2%, RA-LC: 0.2%, LD-HE: 0.3%, LD-LC: 0.3%) with approximately no encoder/decoder run time increases. For the low delay high efficiency configuration, an average BD-bit rate reduction of 0.7% in achieved for the Class E sequences. An increased subjective quality is noticeable at the same bit rate. It is proposed to adopt this technique into the next version of the HM.

# Introduction

The current HM 2.0 [1][2] applies hybrid coding. In Figure 1, the generalized block diagram of the hybrid coder is shown. In a first step, the input signal to be coded is predicted block-wise by either motion compensated prediction or Intra prediction. The resulting prediction error is block-wise transform coded by applying an approximation of the discrete cosine transform (Integer DCT) followed by a quantization of the coefficients. Due to the block wise motion compensated prediction and a block wise prediction error coding, so called blocking artifacts often become visible in the decoded images. These blocking artifacts tend to be annoying for human observers. In order to reduce these annoying blocking artifacts, an adaptive deblocking filter is applied. The deblocked signal is further filtered by the use of an adaptive loop filter before being output and stored for further predictions. Figure 2 illustrates the signal before and after the deblocking filter for a region of the example test sequence Kimono.

Figure 1: Generalized block diagram of the hybrid video encoder according to the HM 2.0 [1][2].

Figure 2: Illustration of the signal before and after the deblocking filter for a region of the example test sequence Kimono.

The deblocking of images is performed based on coding units (CU), which may have various sizes, e.g. 8x8 samples, 16x16 samples. Vertical and horizontal edges of prediction and transform blocks are deblocked. Each edge consists of one or several segments, whereas a segment consists of 8 consecutive lines or columns. The segments vi of the vertical edges are deblocked before the segments hi of the horizontal edges. Figure 3 shows an example coding unit of the size 16x16 samples and the positions of the corresponding 4 segments v1,…,v4 and four segments h1,…,h4. The order of deblocking the vertical edges is from top to bottom and from left to right. The order of deblocking the horizontal edges is from left to right and from top to bottom.

Figure 3: Illustration of the vertical edges and the horizontal edges of an example coding unit (CU) of the size 16x16 samples.

In the following, the samples on the respective sides of the segments of the edges are denoted as  A and B, see Figure 4. The segment A corresponds to the left neighboring partition to B for vertical edges and to the above neighboring partition to B for horizontal edges.

Figure 4: Notation of a part of a vertical edge for deblocking (from [3]).

For each segment of 8 lines/colums, the decisions and filtering operations are performed as explained in the following section

# Decisions according to the HM2.0

In a first step, the two vales d2 and d5 are calculated by the use of the samples of two lines/columns as illustrated in Figure 5:

d2=p22−2⋅p12+p02+q22−2⋅q12+q02

d5=p25−2⋅p15+p05+q25−2⋅q15+q05.

By the use of the two values d2 and d5, it is decided by the threshold operation

d2+d5<β

if all 8 lines/columns of the corresponding segment are filtered or not.

Figure 5: Illustration of the samples used to decide whether to filter or not according to the HM2.0.

In order to perform the decisions, 20 operations are required for each segment of 8 lines/columns.

# Decisions similar as in H.264/MPEG-4 AVC

In contrast to the HM2.0, H.264/MPEG-4 AVC applies individual decisions for each line/column. In order to investigate decisions similar as in H.264/MPEG-4 AVC, an individual value di is calculated for each of the 8 lines/columns as illustrated in Figure 6:

di=p2i−2⋅p1i+p0i+q2i−2⋅q1i+q0i    with i=0,…,7.

By the use of the individual values di, it is decided for each line/column by the threshold operation

2⋅di<β

if a line/column of the corresponding segment is filtered or not.

Figure 6: Illustration of the samples used to decide whether to filter or not similar as in H.264/MPEG-4 AVC.

In order to perform the decisions, 88 operations are required for each segment of 8 lines/columns.

# Decisions compromising the HM2.0 and H.264/MPEG-4 AVC

In order to perform the decisions for a segment of 8 lines/columns, HM2.0 requires 20 operations. If the decisions are performed similar as in H.264/MPEG-4 AVC, 88 operations are required.

In this contribution, decisions are proposed which compromise the ones of HM2.0 and H.264/MPEG-4 AVC with respect to computational expense, measured by number of required operations.

Four values d0, d2, d5, and d7 are calculated for each segment of 8 lines/columns as illustrated in Figure 7:

di=p2i−2⋅p1i+p0i+q2i−2⋅q1i+q0i    with i=0,2,5,7.

By the use of these values, it is decided for each individual line/column by the threshold operations

2⋅di<β        for i=0,2,5,7

d0+d2<β    for i=1

d5+d7<β    for i=6

4⋅d2+2⋅d5<3⋅β    for i=3

4⋅d5+2⋅d2<3⋅β    for i=4

if a line/column of the corresponding segment is filtered or not.

Figure 7: Illustration of the samples used to decide whether to filter or not according to the proposal.

In order to perform the decisions, only 58 operations are required for each segment of 8 lines/columns.

# Experiments and results

The decisions similar as in H.264/MPEG-4 AVC, see section 3, as well as the decisions compromising HM2.0 and H.264/MPEG-4 AVC, see section 4, are both integrated into the reference software of HM2.0 [1].

## BD-bit rate and run time ratios

Following the common conditions [5] the performance of all six test cases is evaluated, which is Intra, Random access, and Low delay, each in high efficiency and low complexity operation mode. For all run time measurements, computers of the same configuration are used.

The BD-rate results as well as the encoder-/decoder run time ratios compared to the reference HM2.0 [1] are shown in Figure 8 for the decisions similar as in H.264/MPEG-4 AVC and in Figure 9 for the decisions compromising HM2.0 and H.264/MPEG-4 AVC. Negative BD-rate numbers show a gain compared to the reference. Run-time ratios less than 100% show reflect that the run time is lower than the one of the reference.

The following results can be observed for both cases:

- BD-bit rate reduction

  - 0.2% in average of over all test sequences and configurations

  - 0.7% in average for LD-LC, Class E

- Approximately no encoder-/decoder run time increase

Figure 8: BD-bit rates and run time ratios of the decisions similar as in H.264/MPEG-4 AVC compared to the reference HM2.0 [1].

Figure 9: BD-bit rates and run time ratios of the decisions compromising HM2.0 and H.264/MPEG-4 AVC compared to the reference HM2.0 [1].

For the decoder, additional run time measurements have been performed. For each of the six coding configurations, the whole test set has been decoded 80 times. Each time, the decoding time for the whole test set has been measured. Using the 80 measured decoding times, an average decoding time T and the corresponding standard deviation σT is calculated. The following table shows the mean decoding times and the standard deviations for the proposal and for the reference. It can be seen that the decoding time is approximately not changed compared to the reference.

| Coding configuration | Reference |  | Proposal |  | Decoder run time increase [%] |
| --- | --- | --- | --- | --- | --- |
|  | T[s] | σT[s] | T[s] | σT[s] |  |
| Intra, High efficiency | 4735.35 | 40.43 | 4692.42 | 40.42 | -0.91 |
| Intra, Low complexity | 2157.63 | 23,30 | 2178.78 | 18.45 | 0.98 |
| Random access, High efficiency | 2474.28 | 34.87 | 2444.41 | 32.50 | -1.21 |
| Random access, Low complexity | 1380.30 | 18.32 | 1370.48 | 13.80 | -0.71 |
| Low delay, High efficiency | 1758.84 | 24.60 | 1743.31 | 17.14 | -0.88 |
| Low delay, Low complexity | 1000.76 | 14.43 | 996.81 | 11.91 | -0.39 |

## Subjective evaluation

In CE12, various test sequences have been selected for subjective evaluations. For these test sequences, the subjective quality of the proposal compared to the reference has been performed with the results shown in the following table.

| Test sequence | Coding configuration | QP | Subjective quality |
| --- | --- | --- | --- |
| Kimono | Low delay, low complexity | 32 | No difference noticeable |
| Kimono | Random access, high efficiency | 37 | No difference noticeable |
| BQ_Mall | Low delay, low complexity | 32 | No difference noticeable |
| BQ_Mall | Random access, high efficiency | 37 | No difference noticeable |
| Vidyo3 | Low delay, low complexity | 32 | No difference noticeable |
| Vidyo3 | Low delay, high efficiency | 37 | Proposal sharper than reference without increase of blocking. Proposal has less color artifacts than reference. |

For five out of the six test sequences, no difference in subjective quality is noticeable. For one out of the six test sequences, the proposal is clearly sharper than the reference without increased blocking. In addition, the proposal shows less color artifacts.

The increase of the sharpness is illustrated in Figure 10 and Figure 11. In Figure 10, a cropped part of a deblocked frame of the test sequence Vidyo3 is shown for the case of the reference HM2.0 [1], low delay, high efficiency, QP37. Figure 11 shows the same cropped part for the case of the proposed deblocking.

The reduction of color artifacts is illustrated in Figure 12 and Figure 13. In Figure 12, a cropped part of a deblocked frame of the test sequence Vidyo3 is shown for the case of the reference HM2.0 [1], low delay, high efficiency, QP37. Figure 13 shows the same cropped part for the case of the proposed deblocking.

Figure 10: Cropped part of a deblocked frame of the test sequence Vidyo3 in the case of the reference [1]. Test case: Low delay, High Efficiency, QP37.

Figure 11: Cropped part of a deblocked frame of the test sequence Vidyo3 in the case of the proposal. Test case: Low delay, High Efficiency, QP37.

Figure 12: Cropped part of a deblocked frame of the test sequence Vidyo3 in the case of the reference [1]. Test case: Low delay, High Efficiency, QP37.

Figure 13: Cropped part of a deblocked frame of the test sequence Vidyo3 in the case of the proposal. Test case: Low delay, High Efficiency, QP37.

## Coding efficiency versus complexity

In Figure 14, the achieved BD-bit rate reduction averaged over all test cases and test sequences is shown versus the additional number of required operations per edge segment of 8 lines/colums, both compared to the reference HM2.0. It can be observed that the decisions compromising H.264/MPEG-4 AVC achieve the same average bit rate reduction of 0.2% compared to the reference but with 44% less operations than decisions similar as in H.264/MPEG-4 AVC.

Figure 14: BD-bit rate reduction averaged over all test cases and test sequences versus additional number of required operations per edge segment compared to the reference HM2.0 [1].

# Conclusion

It is proposed to modify the decisions for enabling the deblocking according to a compromise between the current HM2.0 and H.264/MPEG-4 AVC. By this modification, an average BD-bit rate reduction of 0.2% (I-HE: 0.1%, I-LC: 0.1%, RA-HE: 0.2%, RA-LC: 0.2%, LD-HE: 0.3%, LD-LC: 0.3%) is achieved compared to HM2.0 with approximately no encoder/decoder run time increases. For the low delay high efficiency configuration, an average BD-bit rate reduction of 0.7% in achieved for the Class E sequences. An increased subjective quality is noticeable at the same bit rate. Therefore, it is proposed to adopt this technique into the next version of the HM.

# References

- HM2.0 software: http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-2.0.

- T. Wiegand, W.-J. Han, J.-R. Ohm, G. J. Sullivan, High Efficiency Video Coding (HEVC) text specification Working Draft 1, JCTVC-C403, Guangzou, China, October 2010.

- JCT-VC, Test Model under Consideration, JCTVC-B205_draft007, Geneva, Switzerland, 21-28 July 2010.

- A. Norkin, B. Jeon, M. Narroschke, Description of Core Experiment 12 (CE12): Deblocking filtering, JCTVC-D612, Daegu, Korea, January, 2011.

- F. Bossen, Common test conditions and software reference configurations, JCTVC-D500, Daegu, Korea, January, 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).