| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E256 |
| --- | --- |

| Title: | CE12 subset 1: Cross verification of MediaTek’s deblocking by Ericsson |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross verification |  |  |
| Author(s) or Contact(s): | Kenneth Andersson, Andrey Norkin, Rickard Sjöberg Färögatan 6 164 80 Stockholm Sweden | Tel: Email: | +46 10 7141538 kenneth r andersson @ericsson com |
| Source: | Ericsson |  |  |

# Abstract

This is a cross verification of Mediatek’s deblocking filter in JCTVC-E079. The software consists of several modifications compared to HM2.0. One proposal for both luma and chroma deblocking and one proposal only changing the chroma have been tested. The proposals have been verified successfully for the common conditions for BDR, PSNR, bit rate and decoding time.  Visual inspection by the cross-checkers showed that the proposal with both luma and chroma changes looked slightly better than the anchor in a few more cases than it looked worse than the anchor. The chroma only modifications were reported to have a subjective quality similar to the anchor on average.

# Software

The software has been compiled successfully with gcc 4.3.2 on Linux and Visual Studio for Windows.

Modifications compared to JCTVC-D163 are:

- no directional filtering,

- use of strong filter also for inter/inter blocks

- a block based decision for chroma components. It can be noted that different metrics are used for the case of both luma and chroma modifications and chroma only modifications. The latter only measure gradient on both sides and the first also measure gradient over block boundary. The chroma only modification uses the HM tc and beta tables.

- rewriting of delta filter computation for chroma but the filter taps are the same.

- using H.264 alpha and beta tables with some modifications:

  - reducing beta for line based check for filtering of 2:nd pixel on respective side of block boundary by 4 QPs

  - scaling alpha and beta thresholds dependent on if one block is intra so that alpha and beta thresholds are higher if one block is intra

  - beta thresholds are always higher than H.264 beta thresholds

- reduced clipping thresholds compared to H.264

- modification of decision for filtering when both blocks are inter and at least one of the block has residual to also require that it should be a TU block edge. It can be noted that this only is used for the case of both luma and chroma modifications.

In comparison with the anchor HM2.0 the differences are:

- different block based filter decision for luma.

- block based filter decision also for chroma deblocking.

- different weak filter for luma.

- different filter for chroma .

- strength of local structure estimated by gradient.

- larger thresholds for strength of local structure for filter decision

- additional metric that measures gradient across block boundary for block based filter decision

- one additional table for comparing measured gradient across block boundary with a threshold

- additional line based decision for weak filtering of 2:nd pixel from block boundary. It measures gradient across the 2:nd pixel and compare it with threshold from beta table.

- additional delta filter for filtering of 2:nd pixel from block boundary

- different values in clipping table allows for more filtering for higher QP than HM2.0

- additional requirement for filtering a block edge when both neighbouring blocks are inter coded and at least one of the blocks has residual to also require that it is a TU block edge.

There are defines for luma and chroma modifications (MTK_COMBINELUMACHROMA 1, MTK_DF_DECISION 1, MTK_DF_TU 1), chroma only modifications using HM thresholds (MTK_COMBINELUMACHROMA 1, MTK_DF_DECISION 0, MTK_DF_TU 0), and using transform edge modifications for luma and chroma modifications (MTK_DF_TU 1), The last have not been tested.

# Results

## BDR performance

The software has been used for encoding and decoding according to common conditions [3] on a Linux cluster and the average BDR [4] luma compared to HM2.0 for both luma and chroma modifications is -1.1% and 0.0% for chroma only modifications.

Table I BDR performance for both luma and chroma modifications

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1,9 | -1,0 | -1,3 | -2,4 | -2,1 | -2,0 |
| Class B | -1,2 | -1,2 | -1,8 | -1,4 | -2,8 | -3,0 |
| Class C | -1,0 | -2,1 | -2,1 | -1,2 | -3,6 | -4,1 |
| Class D | -0,9 | -2,0 | -2,2 | -1,1 | -4,0 | -4,4 |
| Class E | -1,3 | -0,3 | -0,5 | -1,5 | -2,5 | -3,3 |
| All | -1,3 | -1,4 | -1,6 | -1,5 | -3,0 | -3,4 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 106% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1,2 | -0,9 | -1,2 | -1,6 | -1,4 | -1,4 |
| Class B | -0,9 | -1,6 | -1,7 | -1,2 | -1,7 | -1,6 |
| Class C | -0,8 | -2,5 | -2,4 | -0,8 | -2,3 | -2,5 |
| Class D | -0,7 | -2,2 | -2,4 | -0,7 | -2,0 | -2,2 |
| Class E |  |  |  |  |  |  |
| All | -0,9 | -1,8 | -1,9 | -1,1 | -1,8 | -1,9 |
| Enc Time[%] | 108% |  |  | 101% |  |  |
| Dec Time[%] | 104% |  |  | 105% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0,8 | -0,7 | -0,9 | -1,2 | -0,6 | -0,3 |
| Class C | -0,9 | -1,0 | -1,1 | -1,1 | -1,0 | -1,2 |
| Class D | -1,1 | -0,7 | -1,2 | -1,0 | -0,7 | -0,6 |
| Class E | -0,9 | -0,3 | -0,2 | -1,4 | -1,9 | -1,7 |
| All | -0,9 | -0,7 | -0,9 | -1,2 | -1,0 | -0,9 |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 104% |  |  | 107% |  |  |

Table II BDR performance for chroma only modifications

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0,0 | -1,1 | -1,3 | 0,0 | -2,2 | -2,1 |
| Class B | 0,0 | -1,4 | -2,0 | 0,0 | -2,7 | -2,8 |
| Class C | 0,0 | -2,3 | -2,3 | 0,0 | -3,4 | -3,9 |
| Class D | 0,0 | -2,1 | -2,4 | 0,0 | -3,6 | -4,0 |
| Class E | 0,0 | -0,3 | -0,5 | 0,0 | -2,5 | -3,3 |
| All | 0,0 | -1,5 | -1,8 | 0,0 | -2,9 | -3,2 |
| Enc Time[%] | 102% |  |  | 127% |  |  |
| Dec Time[%] | 104% |  |  | 123% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0,0 | -0,9 | -1,3 | -0,1 | -2,2 | -2,1 |
| Class B | 0,0 | -1,6 | -1,8 | 0,0 | -1,8 | -1,7 |
| Class C | -0,1 | -2,7 | -2,4 | -0,1 | -2,7 | -3,0 |
| Class D | 0,0 | -2,0 | -2,4 | 0,0 | -2,1 | -2,3 |
| Class E |  |  |  |  |  |  |
| All | 0,0 | -1,8 | -2,0 | 0,0 | -2,2 | -2,2 |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 104% |  |  | 107% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0,0 | -0,4 | -0,7 | 0,0 | -0,8 | -0,5 |
| Class C | 0,0 | -1,0 | -1,1 | 0,0 | -1,2 | -1,4 |
| Class D | -0,1 | -0,6 | -1,0 | 0,0 | -0,9 | -0,7 |
| Class E | -0,1 | -0,3 | -0,4 | 0,0 | -1,6 | -1,7 |
| All | -0,1 | -0,6 | -0,8 | 0,0 | -1,1 | -1,0 |
| Enc Time[%] | 105% |  |  | 104% |  |  |
| Dec Time[%] | 107% |  |  | 105% |  |  |

## Subjective inspection

Some selected sequences and coding conditions have been viewed by blind viewing of anchor and proposal several times by some expert viewers. After decision (similar or one is better) the identities were revealed.

### Both luma and chroma modifications

The proposal with both luma and chroma modifications looked slightly better than the anchor in a few more cases than it is similar or worse than the anchor. In some cases there were some improvements in colors but in other cases some more color artifacts. In some cases details were removed and some cases suffered from blocking artifacts.

### Chroma only modifications

The proposal with chroma only modifications looked similar to the anchor in slightly more cases than it is better or worse than the anchor.  Less chroma artifacts can be noted in some cases and in other cases there are more chroma artifacts.

## Decoding time

Decoding time was measured on Windows on a single machine as an average of several runs. No YUV files were written. The proposal shows similar decoding time as the HM-2.0 anchor.

Table III Decoding time for both luma and chroma modifications vs HM-2.0

|  | MT_LumaChroma _deblock (s) | HM-2.0 (s) | Dec. speed (%) |
| --- | --- | --- | --- |
| ra | 3573,23 | 3577,89 | 99,9 |
| ra_loco | 1691,55 | 1682,46 | 100,5 |
| ld | 2491,26 | 2507,11 | 99,4 |
| ld_loco | 1220,91 | 1209,93 | 100,9 |
| i | 7174,59 | 7209,22 | 99,5 |
| i_loco | 2813,24 | 2765,52 | 101,7 |
| Average |  |  | 100,3 |

Table IV Decoding time for chroma only modifications vs HM-2.0

|  | MT_Chroma _deblock (s) | HM-2.0 (s) | Dec. speed (%) |
| --- | --- | --- | --- |
| ra | 3570,46 | 3577,89 | 99,8 |
| ra_loco | 1678,02 | 1682,46 | 99,7 |
| ld | 2499,73 | 2507,11 | 99,7 |
| ld_loco | 1208,85 | 1209,93 | 99,9 |
| i | 7175,19 | 7209,22 | 99,5 |
| i_loco | 2764,09 | 2765,52 | 99,9 |
| Average |  |  | 99,8 |

# References

- Y.-W. Huang, C.-M. Fu, C.-Y. Chen, C.-Y. Tsai, Y. Gao, J. An, K. Zhang, S. Lei, “In-loop adaptive restoration”, JCTVC-D163, Daegu, Korea, January 2011

- Y.-W. Huang, C.-M. Fu, C.-Y. Chen, C.-Y. Tsai, Y. Gao, J. An, K. Zhang, S. Lei, “CE12: Improved Deblocking Filter”, JCTVC-E079, Geneva, Switzerland, March 2011

- F. Bossen, “Common test conditions and software reference configurations” JCTVC-D600, Daegu, Korea, January 2010.

- G. Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.