| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E258 |
| --- | --- |

| Title: | Improvement of inter mode coding and split flags coding for LCEC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Virginie Drugeon Thomas Wedi Matthias Narroschke Panasonic R&D Center Germany Monzastr. 4c, 63225 Langen, Germany | Tel: Email: | +49 6103 766 240 Virginie.Drugeon@eu.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution proposes modifications for inter mode and split flags coding in Low Complexity Entropy Coding (LCEC). In particular, the adaptation scheme of the tables used for inter mode coding in LCEC is modified as well as the number of tables. The statistics of the modes and the partitioning information of neighboring CUs are exploited. These modifications are tested in HM2.0: 0.2% and 0.4% gain for the random access and the low delay configurations are reported. Furthermore, a joint coding of the split flags for I frames with LCEC is proposed. This modification is reported to bring 0.2% gain for the intra configuration.

# Introduction

This contribution contains several small improvements to the current LCEC coding of partitioning and high-level prediction information.

In the current HM2.0, the split flag, skip flag, merge flag and prediction mode and size of one CU are grouped and coded together if low complexity entropy coding (LCEC) is used (see document JCTVC-D370 [1]). Seven modes and seven fixed code words are defined for that purpose. A table T contains the mapping between the modes and the code words. This table T is adapted at each CU in the following way:

- cx is the codeword index of the current mode x

- cy = cx – 1 and y is the mode corresponding to cy

- T[cy] = x and T[cx] = y (the code words of the two modes x and y are swapped)

Currently one table is defined for each depth.

However, this adaptation scheme is not efficient. In the following example (Table 1), a table for depth 2 (CU of size 16x16) is shown with the frequency of occurrences of use of the modes for depth 2 in the image:

| Mode number | Table (codeword index) | Frequency of occurrences |
| --- | --- | --- |
| 0 | 0 | 173 |
| 1 | 4 | 79 |
| 2 | 1 | 60 |
| 3 | 3 | 44 |
| 4 | 5 | 21 |
| 5 | 6 | 22 |
| 6 | 2 | 167 |

Table 1: Example of table for inter mode coding with LCEC

The code word length corresponds to the codeword index in bits plus one bit. It can be noticed that mode 6 is more likely to appear than mode 2. However, mode 6 uses the codeword number 2 that requires 3 bits and mode 2 the codeword number 1 that requires 2 bits. Therefore, 1 bit more is needed to code mode 6 than to code mode 2, although mode 6 is more likely to appear than mode 2.

Furthermore, it is assumed that the statistics are similar for all the CUs at one given depth. However, the statistics of the modes may be different depending on the content of the CUs. For example, in homogeneously moving background areas, bigger CUs are more likely to be used that in areas in which foreground objects have complex motion (see example in Figure 1 for BasketballDrill).

Figure 1: CUs for a B frame of the sequence BasketballDrill encoded with QP 22 in the random access configuration

For intra, a split flag is encoded for each CU. If the CU is further partitioned in four subCUs, the bit 1 is signaled; otherwise the bit 0 is signaled. However high quality I frames contain mainly 8x8 and 16x16 CUs. Therefore, the bit 1 indicating further partitioning is often signaled.

For example, to signal that a whole LCU (64x64) is partitioned in CUs of size 16x16, it is necessary to send once the bit 1 to signal that the LCU is further partitioned in four 32x32 subCUs and to send four bits 1 to signal that each one of the four 32x32 subCUs are further partitioned. Instead of signaling the bit sequence 1111, it can be more efficient to signal only 1 if it is likely that four CUs in Zscan order are further partitioned. Therefore, depending on the partitioning flags of the CUs above and left, the codeword to send the sequence of bit flags is modified.

# Adaptation of the tables

The first modification is related to the adaptation scheme of the tables. Instead of simply swapping two modes for each CU, the frequency of occurrences of each mode is considered. For each table, a table of counters C is kept in memory. Each time a mode x is signaled, the counter C[x] corresponding to that mode x is incremented by one. The table T is then adapted as follows:

- cx is the codeword index of the current mode x and C[x] is the counter of the mode x

- cy = cx – 1, y is the mode corresponding to cy and C[y] is the counter of the mode y

- If C[x] > C[y]: T[cy] = x, T[cx] = y (the code words of the two modes x and y are swapped) and go back to step 2

With this small change, the modes that are the most likely to appear are assigned to the shortest code words.

# Additional tables

As mentioned above, the frequency of occurrences of the different modes depend on the content of the CU. As can be seen in Figure 1, CUs around object edges are often smaller than in homogeneously moving areas. Therefore, several tables for each depth are introduced. The appropriate code word table for a CU is selected depending on the partitioning and prediction information of the CUs above and left.

For each depth d, the contexts for the split flags as used in CABAC are exploited. There are three contexts for the split flag. If both CUs above and left have a depth smaller than or equal to d, the table corresponding to context 0 is used. If one of the CUs above or left has a higher depth than d and the other CU has a depth equal to d or smaller, the table corresponding to context 1 is used. If both CUs above and left have a depth higher than d, the table corresponding to context 2 is used.

For each depth, three tables are defined and adapted using the algorithm described in section 2.

A second experiment was performed using the three contexts for the split flags and the three contexts for the skip flags as defined in CABAC. For each depth, nine tables are introduced and adapted using the algorithm described in section 2.

# Joint coding of split flags for I frames

In I frames, the whole CU tree is signaled using one split flag at each depth. An example of a CU tree and corresponding split flags can be seen on Figure 2.

Figure 2: example of CU tree

One LCU contains at least one split flag and at most 21 split flags.

Furthermore, in high quality I frames, most CUs are of size 8x8 or 16x16. Therefore, the split flag sequence 1111, used to signal that four CUs in Zscan order at one given depth are further partitioned, is used often in the bitstream. On the contrary for low quality I frames, the split flag sequence 0000 happens more often.

Therefore, the split flag sequences 1111 or 0000 are signaled with one bit 1 depending on the context of the CU. For that purpose, the contexts for the split flags as used in CABAC and as explained in section 3 are exploited.

The depth of the current CU is d. If both CUs above and left have a depth smaller than or equal to d (context 0), 0000 is coded with 1. If one of the CUs above or left has a higher depth than d and the other CU has a depth equal to d or smaller (context 1), the current CU tree coding is used. If both CUs above and left have a depth higher than d (context 2), 1111 is coded with 1.

For a CU from context 2, the following is applied to code a sequence of four split flags corresponding to four CUs in Zscan order:

- The first bit is 1 if all four CUs are further partitioned

- Otherwise, the first bit is 0 and the split flags of the four CUs are sent

An example can be found in Figure 3.

Figure 3: Split flag coding for I frames in context 2

# Results for inter mode coding

The modifications described in sections 2 and 3 using three contexts per depth are implemented in the version HM2.0 of the software. The tests are performed using the common conditions as described in JCTVC-D600 [2] for random access and low delay configurations (see results in Table 2). The executables are compiled using gcc 4.5.0 and executed on Linux 64 bits. The encoding runtimes for the low delay configuration are not reliable because the reference was run on another computer with a different configuration. The decoding runtimes were measured on the same computer and are therefore reliable.

|  | Random access LoCo |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0,3 | -0,2 | -0,3 |  |  |  |
| Class B | -0,1 | -0,1 | 0,0 | -0,4 | -0,1 | 0,4 |
| Class C | -0,3 | -0,2 | -0,2 | -0,5 | -0,4 | -0,4 |
| Class D | -0,2 | -0,2 | -0,2 | -0,2 | -0,1 | 0,2 |
| Class E |  |  |  | -0,4 | 0,0 | 0,1 |
| All | -0,2 | -0,2 | -0,2 | -0,4 | -0,2 | 0,1 |
| Enc Time[%] | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 2: inter mode coding using new table adaptation scheme and three tables per depth

The modifications described in sections 2 and 3 using nine contexts per depth are implemented in the version HM2.0 of the software. The tests are performed using the common conditions as described in JCTVC-D600 [2] for random access and low delay configurations (see results in ). The executables are compiled using gcc 4.5.0 and executed on Linux 64 bits. The encoding and decoding runtimes are reliable.

|  |  | Random access LoCo |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0,3 | -0,2 | -0,3 |  |  |  |
| Class B | -0,2 | -0,1 | -0,1 | -0,5 | -0,3 | 0,0 |
| Class C | -0,3 | -0,2 | -0,2 | -0,6 | -0,4 | -0,6 |
| Class D | -0,2 | -0,3 | -0,2 | -0,3 | 0,2 | 0,1 |
| Class E |  |  |  | -0,4 | 0,1 | 0,4 |
| All | -0,3 | -0,2 | -0,2 | -0,5 | -0,1 | -0,1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

Table 3: inter mode coding using new table adaptation scheme and nine tables per depth

# Results for joint coding of split flags for I frames

The modifications described in section 4 are implemented in the version HM2.0 of the software. The tests are performed using the common conditions as described in JCTVC-D600 [2] for intra configuration (see results in Table 4). The executables are compiled using gcc 4.5.0 and executed on Linux 64 bits. The encoding runtimes are not reliable as the reference was run on another computer with a different configuration, but the decoding runtimes are reliable.

|  | Intra LoCo |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0,2 | -0,1 | -0,1 |
| Class B | -0,2 | -0,2 | -0,2 |
| Class C | -0,1 | -0,1 | -0,2 |
| Class D | -0,1 | -0,1 | -0,1 |
| Class E | -0,1 | 0,0 | 0,0 |
| All | -0,2 | -0,1 | -0,1 |
| Dec Time[%] | 100% |  |  |

Table 4: joint coding of split flags for I frames

# Conclusions

The presented small modifications to the coding of inter mode and split flags with LCEC bring small gains without impacting the decoder runtime of the software. It is proposed to study further these ideas in a Core Experiment. In particular, the optimal number of tables per depth should be investigated.

# References

- JCTVC-D370 CE5: Improved coding of inter prediction mode with LCEC, Wei-Jung Chen, Xianglin Wang, Marta Karczewicz, Deagu, Korea, 20-28 January 2011

# JCTVC-D600 Common test conditions and software reference configurations, Frank Bossen, Daegu, Korea, 20-28 January 2011

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).